3GPP TSG RAN WG2#116-e	R2-2111221

e-Meeting, 1st - 12th November, 2021

3GPP TSG-RAN WG1 Meeting #106bis-e	R1-2110663

e-Meeting, October 11th – 19th, 2021

Title:	LS on UE TA reporting

Release:	Release 17

Work Item:	NR_NTN_solutions

Source:	RAN1

To:	RAN2

Cc:

Contact Person:	Xingqin Lin

xingqin.lin@ericsson.com

Attachments:	None

1.	Overall Description:

RAN1 definition of UE’s TA is given by the following agreement:

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

TTA=NTA+NTA, UE-specific+NTA,common+NTA,offset×Tc

Where:

- NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- NTA, UE-specific  is UE self-estimated TA to pre-compensate for the service link delay.

- NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network.

- NTA,common with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- NTA,offset is a fixed offset used to calculate the timing advance.

In addition, RAN1 has agreed the following for UE TA reporting:

Agreement:

The granularity of the reported TA is slot.

- FFS how to round TA value to slot level granularity

It is up to RAN2 to decide which component or what combination of the components in the UE’s TA formula to use in TA reporting.

2.	Actions:

To RAN2 group:

ACTION: 	RAN1 respectfully requests RAN2 to take the above into account.

3.	Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107-e	11 – 19 November 2021	Electronic Meeting

TSG-RAN WG1 Meeting #107-bis-e	17 – 25 January 2022	Electronic Meeting