3GPP TSG RAN WG2#116-e	R2-2111236

e-Meeting, 1st - 12th November, 2021

3GPP TSG SA WG2 Meeting #147E 		S2-2107972

18 – 22 October, 2021, Elbonia

Title:	Reply LS on discovery and relay (re)selection

Response to:	LS on discovery and relay (re)selection (S2-2106991/R2-2104649)

Release:	Rel-17

Work Item:	5G_ProSe, NR_SL_Relay-Core

Source:	SA2

To:	RAN2

Cc:	-

Contact person:	Qiang Deng

dengqiang1@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.304 CR 0020rev1

# 1	Overall description

SA2 thanks RAN2 for the LS on discovery and relay (re)selection (S2-2106991/R2-2104649).

SA2 has discussed and agreed to include NCGI in the discovery message for 5G ProSe Layer-2 UE-to-Network Relay per the approved CR 0020rev1 as attached.

SA2 also would like to ask RAN2 to provide feedback on the following issues:

1) SA2 has assumed 5G MOCN architecture is supported for 5G ProSe Layer-2 UE-to-Network Relay as described in clause 4.2.7.2 of TS 23.304, and would like to ask RAN2 to confirm this assumption. SA2 has also realized PLMN IDs are required (before Layer-2 link has been established) for the Layer-2 Remote UE to perform PLMN selection as well as Relay selection under 5G MOCN architecture, and would like to know whether PLMN IDs are forwarded by Layer-2 UE-to-Network Relay to Layer-2 Remote UE via the AS layer message.

2) SA2 has realized TAI is needed for 5G ProSe Layer-2 Remote UE to determine the type of initial access message (Mobility Registration Update or Service Request), and would like to ask whether TAI is forwarded by 5G ProSe Layer-2 UE-to-Network Relay to the 5G ProSe Layer-2 Remote UE via the AS layer message.

3) SA2 has discussed the Editor’s Note in TS 23.304 clause 6.4.3.6:

Editor’s Note 	Whether the Layer-2 link modification procedure is also applicable to ProSe Communication via 5G ProSe Layer-2 UE-to-Network Relay requires cooperation with RAN2.

SA2 understands that during the Layer-2 link establishment procedure the 5G ProSe Layer-2 UE-to-Network Relay and 5G ProSe Layer-2 Remote UE do not interact with the QoS Info (the information about PC5 QoS Flows), meaning there is no PC5 QoS Flow established in the PC5 unicast link between Layer-2 UE-to-Network Relay and Layer-2 Remote UE and the QoS handing is therefore setup by RAN. SA2 would like to ask as the Layer-2 link modification procedure is used to add/modify/remove PC5 QoS Flow(s) in the PC5 unicast link, whether this procedure is applicable or not to the Layer-2 UE-to-Network Relay?

4) Per TS 23.304 clause 6.6.2, NG-RAN is provided with 5G ProSe authorised information indicating whether a UE is authorized to use 5G ProSe Direct Discovery, 5G ProSe Direct Communication, to act as a 5G ProSe Layer-2 UE-to-Network Relay, a 5G ProSe Layer-3 UE-to-Network Relay and a 5G ProSe Layer-2 Remote UE. NG-RAN is not provided with authorisation information for whether a UE is authorised to act as a 5G ProSe Layer-3 Remote UE.

Is the authorisation information for whether a UE can act as a 5G ProSe Layer-3 Remote UE needed by NG-RAN to enable configuring the UE with correct discovery configuration information via dedicated signalling?

# 2	Actions

To RAN2

ACTION: 	SA2 kindly asks RAN2 to take above into account, and provides feedback on the raised issues.

# 3	Dates of next TSG SA WG 2 meetings

TSG-SA2 Meeting #148E	15 – 19 November 2021	Elbonia

TSG-SA2 Meeting #149E	14 – 25 February 2022	Elbonia