3GPP TSG-RAN WG2 Meeting #116-e	R2-2110141

Electronic, November 01-12, 2021

Agenda item:	8.11.5

Source: 		Swift Navigation, Mitsubishi Electric Corporation, Intel Corporation, Ericsson

Title: 	Discussion on GNSS Integrity Assistance Data

Document for:	Discussion, Agreement

# 1. 	Introduction

The WI objectives for positioning integrity in Release 17 are as follows [1]:

- Specify the signalling, and procedures to support GNSS positioning integrity determination, including [RAN2, RAN3]: - The assistance information that will be used to support integrity determination - The information that will be used to provide the positioning integrity KPIs and integrity results - Support of integrity for UE-based and UE-assisted A-GNSS positioning. Note: This objective is applicable to NR and E-UTRA.

This discussion paper describes key concepts for interpreting the integrity messages from the accompanying TP [2] and email discussion [3]. We then propose additional text for describing these messages in Stage 2 (TS 38.305).

# 2. 	Discussion

The principle concepts and mathematical relationships between the integrity parameters proposed in [2][3] were first introduced in [4] and further examined in [5][6]. In this submission we propose additional text to describe how these messages are interpreted.

## 2.1	GNSS integrity assistance data parameters

The same categories from the email discussion in [3] have been adopted below to aid interpretation.

### 2.1.1 	Validity / Applicability

The purpose of the validity and applicability parameters are to ensure that the assistance data is still valid at the time that the integrity solution is computed, as well as that all the different assistance data (positioning as well as integrity) that are used together are mutually compatible. For integrity, it is essential that compatible and applicable assistance data is used together in a controlled way, so sufficient message fields must be present to ensure this.

For the integrity assistance data to be valid / applicable, the following conditions must be satisfied. We have also provided justifications for why each message field and condition is necessary.

- The epoch time of the integrity solution is in the range [epochTime : epochTime + validityPeriod] (inclusive) for all of the integrity assistance data that will be used in the integrity solution.

  - This is intended to allow the network to bound the time period of validity of the assistance data such that the AD can be set to expire after a certain time. It is necessary to bound the time of validity as the bounds are often computed accounting for the variability of the parameter over this time period.

- The iod-ssr of the integrity assistance data matches the iod-ssr of the positioning assistance data.

  - The integrity assistance data is computed with reference to a particular set of positioning assistance data. By matching the iod-ssr field, it can be assured that the correct integrity AD is being used with the corresponding positioning AD.

- The svID of the integrity assistance data (if present) matches the svID of the positioning assistance data.

- The correctionPointSetID of the integrity assistance data (if present) matches the correctionPointSetID of the positioning assistance data.

- The gridList (if present) of the integrity assistance data overlaps the gridList of the positioning assistance data.

  - These three conditions are simply ensuring that the correct element of the integrity AD is matched with the corresponding element of the positioning AD, i.e. satellite, signal, grid point etc. are aligned.

Proposal 1: Agree that epochTime, validityPeriod, iod-ssr and (where applicable) svID, correctionPointSetID and gridList fields are provided in the integrity assistance data to provide the possibility to check applicability with respect to the positioning assistance data.

### 2.1.2 	Alerts

Several Do Not Use (DNU) flags are specified in the proposal [2]. If a DNU flag is issued then the UE shall not utilize the corresponding assistance data for the purposes of computing an integrity solution (while the assistance data still can be utilized for positioning).

The rationale for providing these DNU flags in a separate Alert message, as opposed to for example signalling the validity of the bounds in the bounds IEs themselves, is that the UE must fail safe even in the event of a loss of connectivity. This means that the UE must receive an affirmative signal that the bounds are still valid within the TTA, otherwise it must assume that a feared event may have occurred and the notification was not delivered.

This would result in a need to send the bounds IEs very frequently which would be wasteful of bandwidth. This is why we propose to use DNU flags in a separate and very compact Alert IE that can be updated very frequently.

Proposal 2: Agree to provide “DNU” flags in separate Alert IE(s)

The alerts in proposal [2] are further subdivided into a master DNU flag for the entire integrity service (serviceDoNotUse), as well as more granular DNU flags corresponding to the atmospheric components of the positioning assistance data (ionosphereDoNotUse, troposphereDoNotUse) and the signal-in-space (i.e. orbit and clock) components (GNSS-Integrity-ConstellationAlert). This is to allow for the service to potentially degrade from e.g. PPP-RTK to PPP mode if ionosphere or troposphere are unavailable but orbit and clock are still available.

Observation 1: The DNU flags can be further sub-divided into service, ionosphere, troposphere, constellation and satellite flags.

<--------------------------------------------------Start of Text Proposal-------------------------------------------------->

#### 8.1.2.1.29	Integrity Service Alert

Integrity Service Alerts provide information on whether the service can be used for integrity. A Do Not Use (DNU) flag indicates that the corresponding assistance data is not suitable for the purpose of computing integrity. If no DNU flag is issued, then the corresponding assistance data may be used for the purpose of computing integrity.

<--------------------------------------------------End of Text Proposal--------------------------------------------------->

Proposal 3: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Service Alert’.

### 2.1.3	Integrity Bounds & Residual Risks

In the case that no DNU flag is issued, then the corresponding assistance data may be used for the purposes of computing an integrity solution. In this case the Network is making a positive assertion that the probability of an unflagged incidence of an error exceeding the corresponding bound is less than a specified threshold.

We can express this mathematically as follows:

P(Integrity Event)=P(Error>Bound | NOT DNU) <= Threshold

We can decompose this into Fault and Fault-free cases:

P(Integrity Event)=P(Error>Bound | NOT DNU, Fault)+P(Error>Bound | NOT DNU, NOT Fault)

Each of these cases can be assigned a probability upper bound:

P(Error>Bound | NOT DNU, Fault)<=Residual Risk

P(Error>Bound | NOT DNU, NOT Fault)<=IRallocation

Therefore, putting all of this together we arrive at a specification of the assurance that the network provides to the UE on which the UE’s integrity calculation is based:

| P(Error>Bound ¦ NOT DNU)<=Residual Risk + IRallocation | (1) |
| --- | --- |

for all values of IRallocation in the range irMinimum <= IRallocation <= irMaximum

The bound is computed by the UE according to the formula:

Bound = mean + K * stdDev K = normInv(IRallocation / 2) irMinimum <= IRallocation <= irMaximum

Where:

mean: mean value for this specific error

stdDev: standard deviation for this specific error

irMinimum, irMaximum: From GNSS-Integrity-ServiceParameters

Residual Risk: Residual Risk value provided by the Network for this specific error (e.g. pTroposphereFault)

Note, the UE is at liberty to choose any value of IRallocation (and hence K) so long as it is within the prescribed range. The Residual Risk values are provided by the Network as an additional fault probability that is in addition to the probability implied by the bound and choice of IRallocation. This is typically interpreted to be a decomposition into fault and fault free cases however so long as (1) is maintained, any allocation of risk between the Residual Risk and the IRallocation is acceptable. Note that (1) must hold for all possible choices of IRallocation that the user may make within the valid range. These concepts are well established in the field of GNSS integrity [7][8] and have been introduced in the previous comparisons (e.g. [5]) with RTCM and ARAIM etc (e.g. Psat and Pconst).

<--------------------------------------------------Start of Text Proposal-------------------------------------------------->

#### 8.1.2.1.b	Integrity Principle of Operation

For integrity operation, the network will ensure that:

P(Error>Bound | NOT DNU)<=Residual Risk + IRallocation

for all values of IRallocation in the range irMinimum <= IRallocation <= irMaximum

for all the errors in Table 8.1.2.1b-1, which have corresponding integrity assistance data available and where the corresponding DNU flag is set to false.

Where:

Error: Error is the difference between the true value of a GNSS error, and its value as estimated and provided in the corresponding assistance data as per Table 8.1.2.1b-1

Bound: Bound is computed as per the formula in Section 8.1.2.1.30

DNU: The DNU flag corresponding to this error as per Table 8.1.2.1b-1

Residual Risk: The residual risk corresponding to this error as per Table 8.1.2.1b-1

irMinimum, irMaximum: Service parameters provided in GNSS-Integrity-ServiceParameters

Table 8.1.2.1b-1: Mapping of Integrity Parameters

| Error | GNSS Assistance Data (Existing LPP IEs) | DNUs (Proposed IEs) | Bound Mean (Proposed IEs) | Bound StdDev (Proposed IEs) | Residual Risk (Proposed IEs) | Time Correlation (Proposed IEs) |
| --- | --- | --- | --- | --- | --- | --- |
| Orbit | GNSS-SSR-OrbitCorrections | serviceDoNotUse constellationDoNotUse svDoNotUse | orbitClockErrorMeanShapeVector orbitClockRateErrorMeanShapeVector orbitClockErrorMeanScaleFactor orbitClockRateErrorMeanScaleFactor | orbitClockErrorCovarianceShapeMatrix orbitClockRateErrorCovarianceShapeMatrix orbitClockErrorCovarianceScaleFactor orbitClockRateErrorCovScaleFactor | pConstellation pSatellite | tCorrelationRangeOrbit tCorrelationRangeRateOrbit |
| Clock | GNSS-SSR-ClockCorrections |  |  |  |  | tCorrelationRangeClock tCorrelationRangeRateClock |
| Code Bias | GNSS-SSR-CodeBias |  | meanCodeBias meanCodeBiasRate | stdDevCodeBias stdDevCodeBiasRate |  |  |
| Phase Bias | GNSS-SSR-PhaseBias |  | meanPhaseBias meanPhaseBiasRate | stdDevPhaseBias stdDevPhaseBiasRate |  |  |
| Ionosphere | GNSS-SSR-STEC-Correction GNSS-SSR-GriddedCorrection | serviceDoNotUse ionosphereDoNotUse | meanIonosphere meanIonosphereRate | stdDevIonosphere stdDevIonosphereRate | pIonosphereFault | tCorrelationIonosphere |
| Troposphere Vertical HydroStatic Delay | GNSS-SSR-GriddedCorrection | serviceDoNotUse troposphereDoNotUse | meanTroposphereVerticalHydroStaticDelay meanTroposphereVerticalHydroStaticDelayRate | stdDevTroposphereVerticalHydroStaticDelay stdDevTroposphereVerticalHydroStaticDelayRate | pTroposphereFault | tCorrelationTroposphere |
| TroposphereVerticalWetDelay |  |  | meanTroposphereVerticalWetStaticDelay meanTroposphereVerticalWetDelayRate | stdDevTroposphereVerticalWetStaticDelay stdDevTroposphereVerticalWetDelayRate |  |  |

<--------------------------------------------------End of Text Proposal--------------------------------------------------->

Proposal 4: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Principle of Operation’.

<--------------------------------------------------Start of Text Proposal-------------------------------------------------->

#### 8.1.2.1.30	Integrity Bounds

Integrity Bounds provide the statistical distribution of the residual errors associated with the GNSS positioning corrections (e.g. RTK, SSR etc). Integrity bounds are used to statistically bound the residual errors after the positioning corrections have been applied. The bound is computed according to the following formula:

Bound = mean + K * stdDev

K = normInv(IRallocation / 2)

irMinimum <= IRallocation <= irMaximum

Where:

mean: mean value for this specific error

stdDev: standard deviation for this specific error

<--------------------------------------------------End of Text Proposal--------------------------------------------------->

Proposal 5: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Bounds’.

<--------------------------------------------------Start of Text Proposal-------------------------------------------------->

#### 8.1.2.1.31	Integrity Service Parameters

Integrity Service Parameters provide the range of integrity risk allocations for which the Integrity Bounds are valid (see formula in Section 8.1.2.1.30).

<--------------------------------------------------End of Text Proposal--------------------------------------------------->

Proposal 6: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Service Parameters’.

### 2.1.4 	Correlation Times

For users who wish to take advantage of time-based estimation techniques such as Kalman Filtering they must also be provided with information about the time correlation of the errors.

Proposal 7: Agree to include parameters to allow the use of time-based estimation techniques (e.g. Kalman Filtering) in addition to snapshot based techniques.

<--------------------------------------------------Start of Text Proposal-------------------------------------------------->

8.1.2.1.32	Integrity Correlation Times

Integrity Correlation Times provide the minimum time interval beyond which two measurements of the same parameter can be considered to be independent from one another.

<--------------------------------------------------End of Text Proposal--------------------------------------------------->

Proposal 8: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Correlation Times’.

# 3. 	Conclusions

Proposal 1: Agree that epochTime, validityPeriod, iod-ssr and (where applicable) svID, correctionPointSetID and gridList fields are provided in the integrity assistance data to provide the possibility to check applicability with respect to the positioning assistance data.

Proposal 2: Agree to provide “DNU” flags in separate Alert IE(s).

Proposal 3: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Service Alert’.

Proposal 4: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Principle of Operation’.

Proposal 5: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Bounds’.

Proposal 6: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Service Parameters’.

Proposal 7: Agree to include parameters to allow the use of time-based estimation techniques (e.g. Kalman Filtering) in addition to snapshot based techniques.

Proposal 8: Agree to adopt in TS 38.305 the proposed text on ‘Integrity Correlation Times’.

# 4. 	References

[1]	RP-210903, “WID: NR Positioning Enhancements”, RAN#91-e, March 2021.

[2]	R2-2108475, “TP on GNSS Integrity Assistance Data, Swift Navigation, RAN2#115-e, November 2021.

[3]	R2-2110181, “[Post115-e][607][POS] Integrity assistance data (Huawei)”, RAN2#116-e, November 2021.

[4]	R2-2103954, “Considerations on Positioning Integrity Determination”, Swift Navigation, Intel Corporation,

Ericsson, RAN2#113-Bis-e, April 2021.

[5]	R2-2106105, “Proposals on GNSS integrity assistance information”, Swift Navigation, RAN2#114-e, May

2021.

[6]	R2-2108474, “Discussion on GNSS Integrity Assistance Data”, Swift Navigation, Ericsson, Mitsubishi

Electric Corporation, RAN2#114-e, May 2021.

[7]	Working Group C (WG-C), “EU-U.S. Cooperation on Satellite Navigation”, ARAIM Technical Subgroup,

Milestone 2 Report Final Version, February 2015.

[8]	Federal Aviation Administration (FAA) Satellite Navigation Branch, “Advanced Receiver Autonomous

Integrity Monitoring Performance Analysis Report”, November 2020.