3GPP TSG-RAN WG2 #116-e	 R2-2111472

eMeeting, 1st – 12th November, 2021

Title:	Reply LS on R17 NR MG enhancements – Concurrent MG

Response to:	R4-2115343/ R2-2109361

Release:	Rel-17

Work Item:	NR_MG_enh-Core

Source:	RAN2

To:	RAN4

Cc:                              RAN1

Contact Person:

#### Name:	Chun-Fan (Felix), Tsai

E-mail Address:	Chun-Fan.Tsai (at) mediaTek.com

1. Overall Description

RAN2 would like to thank RAN4 for the LS on R17 NR MG enhancements – Concurrent MG [1]. RAN2 has discussed the operation and limitation for concurrent gap and reached the following agreements.

RAN2 confirms the following understanding for concurrent gap operation:

- 1. Concurrent gaps are multiple measurement gaps and each gap pattern could be associated with one or multiple frequency layers.

- 2. Each frequency layer can be associated with only one of the concurrent gaps.

- 3. Without considering pre-configured MG, concurrent gaps are always activated if it is setup by the network.

- 4. No new gap pattern is introduced for concurrent gap, the existing R15/R16 gap pattern could be configured for the concurrent gaps.

RAN2 to clarify “frequency layer” and limitations as below:

- PRS measurement can be associated with one gap pattern, no matter how many frequencies are measured for PRS.

- Each measured SSB or LTE frequency is considered as one frequency layer.

- Measured CSI-RS resources with the same center frequency is considered as one frequency layer. It is possible to have Multiple MOs including CSI-RS resources with same center frequency.

- SSB and CSI-RS measurement in one MO are considered as different frequency layers.

Firstly, RAN2 would like to confirm with RAN4 that the above understanding is correct.

In addition, RAN2 would like to ask the following questions.

RAN2 signaling could ensure to always provide an association between Rel-17 concurrent MGs and the frequency layers to be measured (at least in NR SA). However, since legacy MG do not provide this association: 
Q1 – Can Rel-17 concurrent gaps be configured together with legacy gap? If ‘yes’, what would be the UE behavior?

Q2 – How many concurrent gaps could be configured simultaneously?

Q3 – Could concurrent gaps be configured with different gap types (i.e., some gaps are per-UE while some gaps are per-FR)? If so, what is the maximum number of gaps that could be configured simultaneously for each gap type (per-UE /per-FR1/per-FR2)?

Q4 – Is the legacy gap sharing configuration (configured in MeasGapSharingConfig) applicable to Rel-17 concurrent gaps? If ‘yes’, could RAN4 clarify how this would work?

Q5 – Could RAN4 help to clarify whether UTRAN-FDD measurement (configured in MeasObjectUTRA-FDD) is also applicable in concurrent gap operation?

2. Actions:

ACTION: 	RAN2 respectfully asks RAN4 to answer the above questions.

3. References:

[1] R2-2109361/R4-2115343, “LS on R17 NR MG enhancements – Concurrent MG”, RAN4

4. Date of Next RAN2 Meetings:

RAN2#116bis-e, eMeeting, 17-25 Jan. 2022

RAN2#117-e, eMeeting, 21 Feb. to 03 Mar. 2022