3GPP TSG-RAN WG2 #116-e	R2-2111473

eMeeting, 1st – 12th November, 2021

Title:	Reply LS on association between serving cell and measurement object

Response to:	R5-215762 / R2-2109370

Release:	Rel-15

Work Item:	5GS_NR_LTE-UEConTest

Source:	RAN2

To:	RAN5

Cc:

Contact Person:

#### Name:	Chun-Fan (Felix), Tsai

E-mail Address:	Chun-Fan.Tsai (at) mediaTek.com

1. Overall Description

RAN2 would like to thank RAN5 for the LS on association between serving cell and measurement object [1]. RAN2 has discussed this issue and would like to provide the following feedback to RAN5’s questions.

Question 1: Is the indication by servingCellMO the way to determine the association between serving cell and measurement object mentioned in TS 38.331?

Answer: Yes. The servingCellMO indication is used to determine the association between serving cell and measurement object in TS 38.331.

Question 2: For event A3/A5 triggering reporting configured on SCC, is it compulsory to configure servingCellMO for SCell in order to enable UE considering SCell to be a neighbouring cell as well?

Answer: Yes. For event A3/A5 triggering reporting configured on SCC, it is compulsory to configure servingCellMO for SCell in order to enable UE considering SCell to be a neighbouring cell.

2. Actions:

ACTION: 	RAN2 respectfully asks RAN5 to take above RAN2 conclusions into consideration.

3. References:

[1] R2-2109370/ R5-215762, “Association between serving cell and measurement object”, RAN5

4. Date of Next RAN2 Meetings:

RAN2#116bis-e, eMeeting, 17-25 Jan. 2022

RAN2#117-e, eMeeting, 21 Feb. to 03 Mar. 2022