3GPP TSG-RAN WG2 Meeting #116-e	R2-2109447

Online, 1-12 November 2021

Title:	Reply to RAN3 on the coordination between gNBs on the supporting of RedCap UEs

Response to:	LS on the coordination between gNBs on the supporting of RedCap UEs (R3-214422)

Release:	Rel-17

Work Item:	NR_redcap_Core

Source:	RAN2

To:	RAN3

Cc:

Contact Person:

#### Name:	Linhai He

E-mail Address:	linhaihe@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN2 thanks RAN3 for their questions regarding the coordination between gNBs for support of RedCap UEs. RAN2’s reply to the questions is the following:

- RAN3’s question: Can RAN2 confirm that RedCap UEs should not attempt to camp/access in legacy cells or be handed over to such cells; if so, can RAN2 please explain how access control will work for legacy gNBs. This is related to one option considered in RAN3, where it is assumed that the broadcast in supporting cells would be designed to indicate support (or access allowed), and the presence (or contents) of such broadcast would be indicated at Xn level by the possible introduction of new information elements, rather than a barring indication as mentioned in the LS.

RAN2’s answer: RAN2 confirm that RedCap UEs should not attempt to camp/access in legacy cells. A RedCap-capable gNB always includes RedCap-specific cell barring indicators in its system information (i.e. even if the cell is not RedCap barred). RedCap UEs hence can determine whether a gNB is RedCap capable based on the presence of those RedCap-specific indicators in its system information.

- RAN3’s question: Can RAN2 confirm whether a legacy gNB can detect via the (RedCap) UE Radio Capabilities (e.g. at Handover preparation) that it cannot configure or serve the RedCap UE? This is related to another option considered by RAN3 in which a Rel-17 gNB can perceive, e.g., the support or barring by a neighbour gNB cell of RedCap UE via the handover preparation failure with signalling a proper cause value at XnAP level.

RAN2’s answer:  RAN2 confirm that a legacy gNB cannot identify a RedCap UE as such via UE Radio Capabilities and therefore cannot indicate that rejection is due to non-RedCap support. A RedCap UE may have capabilities whose values are outside the range of legacy UEs or new capabilities signalled in non-critical extension. In either case, RAN2 cannot guarantee that a legacy gNB is able to handle those capabilities properly and use them to identify RedCap UEs. And even if they do, it may not be possible for the source node to understand that rejection is due to the UE being a RedCap UE.

2. Actions:

To RAN3

ACTION: 	RAN2 kindly asks RAN3 to take the above reply into consideration in their discussions on the coordination between gNBs for the support of RedCap UEs.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG2 Meeting #117-e                       January 2021	E-Meeting

TSG-RAN WG2 Meeting #117bis-e                  February 2022 	E-Meeting