3GPP TSG-RAN WG2 Meeting #116 electronic	R2-2110271

Online, November 1 - 12, 2021

Agenda Item:	8.7.3.1

Source:	MediaTek Inc.

Title:	Remaining issues of Relay Discovery

Document for:     Discussion and Decision

# Introduction

In RAN2#115-e meeting, we have discussion and agreement for relay discovery as follows.

Agreements:

FFS if the network can configure shared and dedicated pool simultaneously.

For mode 1, if agreed that both shared and dedicated resource pools can be configured, it is up to gNB which one the UE should use to transmit discovery message.

For mode 2, if agreed that both shared and dedicated resource pools can be configured, downselect from the following options:

- Left to UE implementation

- Dedicated pool should be prioritised

- Shared pool should be prioritised

Agreement:

Discovery for IC relay UE, for IC remote UE which has not been connected to network via a relay UE, and for OOC remote UE which has not been connected to network via a relay UE, relay discovery reuses the Rel-16 V2X resource allocation principles.

Agreement:

Proposal 3: [13/19] For UE (including IC remote UE and OOC remote UE) which has been connected to network via a relay UE, only resource allocation mode 2 can be used.

In this paper, we address the remaining issues.

# Discussion

## When both shared and dedicated pools are configured

## Whether shared pools can be used to transmit discovery message

As addressed in [1], when both shared and dedidcated pools are configured, it is controvertial whether the shared resource pool can be used for transmission of discovery message or not. In our view, the pros and cons can be summarized below:

|  | Share pools cannot be used to transmit discovery message | Share pools can be used to transmit discovery message |
| --- | --- | --- |
| Pros | - Power saving gain - Easier to measure SD-RSRP | - Reduce latency for a UE to transmit discovery message - Better resource utilization |
| Cons | - Longer lantency if RP for discovery is sparse | - No power saving gain for a UE to monitor discovery message |

In our view, we think power saving gain is a cririal metric to considered, and thus we suggest that discovery message cannot be transitted on the shared RPs, if both dedicated and shared RP is configured. Besides, we think if network has concern on resource utilization, then network shall not configure dedicated RPs at all.

Proposal 1-a: Shared RPs are not used for discovery message transmission, if both shared and dedicated RPs are configured.

A compromised solution is that whether UE can transmit discovery message on a shared RP is configurable per RP.

    - If network think the load of discovery traffic is low, the network could configure all shared RP not applicable for discovery message transmission

    - If network detects that the load of discovery traffic in dedicated RPs are high, the network could configure several shared RPs allowed for discovery message transmission

Proposal 1-b: Whether a shared RP can be used for discovery message transmission is a per RP configuration, if both shared and dedicated RPs are configured.

## Resource pool selection when both shared and dedicated RPs are configured

For mode 1 scheduling, it is straightforward that it is up to NW to schedule resource for transmission of discovery message.

For mode 2 scheduling, we think we could leave the resource selection for UE implementation, i.e., let a mode 2 UE to determine which resource pool is used for discovery message transmission by itself. This may reduce spec impact and implementation complexity.

Proposal 2: For mode-2 UE, it is left to UE implementation how to select a RP for discovery message transmission, if both shared and dedicated RPs are configured.

## LCP restriction for discovery message transmission

Both data and relay discovery message may be transmitted on a shared resource pool. A question is whether data and relay discovery message can be multiplexed together into the same MAC PDU. Since they both are for different purposes, we tend to think multiplexing should not be allowed, even of they both have the same cast type (e.g. both are broadcast).

Proposal 3: RAN2 confirms that discovery messag and data (regardless of the same or different cast types) cannot be multiplexed together into the same MAC PDU.

To distinguish traffic for discovery message from traffic for data, a logical channel could be specific to discovery message (e.g. introducing an indicator bit in the logical channel configuration) or a logical channel could be associated with a L2 destination ID specific for discovery message. Whether there is a L2 ID specific for discovery message is up to SA2 decision.

Proposal 4: A SL logical channel is associated with discovery messages in one of the following ways:

- A SL logical channel is associated with discovery messasge by an explicit indicator in RRC IE

- A SL logical channel is associated with a discovery message specific L2 desitnation ID

For a SL grant in a dedicated RP (e.g. scheduled by NW in mode-1 scheduling, or selected by UE in mode-2 scheduling), the UE should multiplex only discovery messages into the SL grant.

Proposal 5: UE selects only those SL logical chnanels associated with discovery messages when the SL grant is in dedicated RPs.

In contrast, for a SL grant in a shared RP, there are several cases:

- If dedicated RP is not configured, or if discovery message can be transmitted in a shared RP:

    - UE follows legacy R16 SL LCP procedure to determine which of data or discovery message to transmit by comparing the SL logical channel priority

- If dedicated RP is configured and the discovery message cannot be transmitted in a shared RP:

    - UE consider only those SL logical channels not for discovery messages for LCP operation

Proposal 6: UE selects only those SL logical chnanels associated with discovery messages when the SL grant is in a shared RP wherein transmission of discovery message is not allowed.

Proposal 7: UE follow legacy R16 procedure to determine which of discovery messages or data is multiplexed into the SL grant, if the SL grant is in a shared RP wherein transmission of discovery message is allowed.

A minor question is, how to perform prioritization if the highest-priority data has the same priority as the discovery message (i.e. SL logical priority 1). In our view, it could be up to UE implementation to save spec effort and to be aligned with legacy R16 behaviour.

Proposal 7-1: If the highest-priority data has the same priority as the discovery message, it is up to UE implementation which one of data and discovery messages is multiplexed into the SL grant.

## BSR for discovery message

For mode-1 scheduling, if dedicated resource pools are configured, a question is how gNB knows the size of discovery message. In current SL BSR format, buffer size is reported per sideink logical channel group per desitnation, and thus gNB cannot know exactly the size of discovery message. Without the discovery message size, gNB cannot schedule SL grant on the dedicated resource pools sutiablely.

Observation: To schedule SL grant for discovery messages on dedicated resource pools, gNB need the size information of discovery message, which is not revealed in current SL BSR report.

It’s preferred for UE to report size of discovery message without changing the exising SL BSR format. Thus, two solutions could be considered:

- Option 1: Use existing destination index field in SL BSR

    - E.g. a destination index is specific for discovery message

    - In our view, we don't think option 1 must be linked to relay-specific L2 desitnation ID. For example, in the spec we can just reserve a desitnation index value dedicated for discovery message.

- Option 2: Use existing LCG ID field in SL BSR

    - E.g. a LCG ID value is specific for discovery messsage

In our view, we are fine with either options.

Proposal 8: RAN2 discuss how mode-1 UE report discovery message size for transmission in dedicated RPs.

## On non-relay discovery

In RAN#93, a new objective is added into the revised WI [2]  for non-relay discovery.

Secondly, the objective of this work item also covers the non-relay discovery (i.e. 5G ProSe Direct Discovery). - Specify mechanisms for 5G ProSe Direct Discovery [RAN2, RAN3, RAN4]; … NOTE 5: RAN2 prioritizes completion of relay discovery work, and for 5G ProSe Direct Discovery (Objective 7) takes the agreement for relay-based discovery achieved as baseline while enhancements and optimizations beyond basic functionality for the 5G ProSe Direct Discovery scenario are not considered.

Regarding whether a UE is allowed to transmit discovery message, the threshold-based design in relay-based discovery is not applicable to non-relay based discovery. This is because for 5G ProSe Direct Discovery, the intention for discovery message transmission is not to serve as a remote or a relay, and thus Uu RSRP shall not be a criteria.

Regarding how a UE acquire configuration for discovery message, we can follow L2/L3 common procedure. Note that some L2 specific design for relay-based discovery (an example as follows) is not applicable for non-relay-specifc discovery, because in non-relay scheme, it is not expected that a UE would forward discovery configuration to other UE.

RAN2#114 agreement

Proposal 3a (modified): RAN2 agree that for L2 remote UE which is out-of-coverage, but connected to network via a relay UE (i.e., either in RRC CONNECTED or RRC IDLE/INACTIVE), it should follow network configuration, i.e., SIB or dedicated signalling, if available.

Proposal 9: The thereshold-based method used to determine whether a UE is allowed to transmit discovery message is not applicable to non-relay discovery.

Proposal 10:  5G ProSe Direct Discovery can takes the agreement for L2/L3 common design for relay discovery as the baseline, except for threshold-based design.

One more question is, whether NR SL relay and 5G ProSe Direct Discovery share the same or apply different dedicated resource pool for discovery, if configured. In our view, it could be up to gNB implementation. For example, if the dedicated resource pool has low utilization, the gNB may configure dedicated pools shared by NR SL relay and 5G ProSe Direct Discovery. In contrast, if the dedicated resource pools have high loading, or if there is specific QoS requirement for SL relay, gNB can configure separate dedicated pools for SL relay and for 5G ProSe Direct Discovery.

Proposal 11: It’s up to gNB configuration whether a dedicated resource pool for discovery message transmission is used by NR SL relay, 5G ProSe Direct Discovery, or both.

# Conclusion

Proposal 1-a: Shared RPs are not used for discovery message transmission, if both shared and dedicated RPs are configured.

Proposal 1-b: Whether a shared RP can be used for discovery message transmission is a per RP configuration, if both shared and dedicated RPs are configured.

Proposal 2: For mode-2 UE, it is left to UE implementation how to select a RP for discovery message transmission, if both shared and dedicated RPs are configured.

Proposal 3: RAN2 confirms that discovery messag and data (regardless of the same or different cast types) cannot be multiplexed together into the same MAC PDU.

Proposal 4: A SL logical channel is associated with discovery messages in one of the following ways:

- A SL logical channel is associated with discovery messasge by an explicit indicator in RRC IE

- A SL logical channel is associated with a discovery message specific L2 desitnation ID

Proposal 5: UE selects only those SL logical chnanels associated with discovery messages when the SL grant is in dedicated RPs.

Proposal 6: UE selects only those SL logical chnanels associated with discovery messages when the SL grant is in a shared RP wherein transmission of discovery message is not allowed.

Proposal 7: UE follow legacy R16 procedure to determine which of discovery messages or data is multiplexed into the SL grant, if the SL grant is in a shared RP wherein transmission of discovery message is allowed.

Proposal 7-1: If the highest-priority data has the same priority as the discovery message, it is up to UE implementation which one of data and discovery messages is multiplexed into the SL grant.

Proposal 8: RAN2 discuss how mode-1 UE report discovery message size for transmission in dedicated RPs.

Proposal 9: The thereshold-based method used to determine whether a UE is allowed to transmit discovery message is not applicable to non-relay discovery.

Proposal 10:  5G ProSe Direct Discovery can takes the agreement for L2/L3 common design for relay discovery as the baseline, except for threshold-based design.

Proposal 11: It’s up to gNB configuration whether a dedicated resource pool for discovery message transmission is used by NR SL relay, 5G ProSe Direct Discovery, or both.

# Reference

- R2-210XXXX, [Post115-e][611][Relay] Discovery shared and dedicated pool issue (Qualcomm)

- RP-212601, New WID on NR Sidelink Relay