3GPP TSG-RAN WG2 Meeting #116-e                                                     R2-2110610

Electronic, 1st - 12th Nov 2021

Source:              CATT, APPLE, vivo, Huawei, HiSilicon, OPPO

Title:	             PDCP/RLC Entity Maintenance for SL-SRBs

Agenda Item:	6.2.3

Document for:	Discussion and Decision

Introduction

According to the current description in RAN2 specs, the PDCP/RLC entity maintenance is incomplete. In this contribution, we descript this issue according to the spec content and raise further discussion.

Discussion

According to the TS33.536, for sidelink unicast, the high-level flow of connection establishment is as below:

Figure-1 High-level flow of connection establishment

In the following, the cast type of each PC5-S message is summarized in the following table:

Table-1 Cast type of each PC5-S signalling during PC5-S connection establishment procedure

| PC5-S signaling | Cast type | Reference |
| --- | --- | --- |
| DIRECT LINK ESTABLISHMENT REQUEST | Unicast or broadcast | TS23.287 UE-1 sends the Direct Communication Request message via PC5 broadcast or unicast using the source Layer-2 ID and the destination Layer-2 ID. |
| DIRECT LINK AUTHENTICATION REQUEST | Unicast | TS24.587 After the DIRECT LINK AUTHENTICATION REQUEST message is generated, the initiating UE shall pass this message to the lower layers for transmission along with the initiating UE's layer-2 ID for unicast communication and the target UE's layer-2 ID for unicast communication. |
| DIRECT LINK AUTHENTICATION RESPONSE | Unicast | TS24.587 After the DIRECT LINK AUTHENTICATION RESPONSE message is generated, the target UE shall pass this message to the lower layers for transmission along with the target UE's layer-2 ID for unicast communication and the initiating UE's layer-2 ID for unicast communication. |
| DIRECT LINK SECURITY MODE COMMAND | Unicast | TS24.587 After the DIRECT LINK SECURITY MODE COMMAND message is generated, the initiating UE shall pass this message to the lower layers for transmission along with the initiating UE's layer-2 ID for unicast communication and the target UE's layer-2 ID for unicast communication, …… |
| DIRECT LINK SECURITY MODE COMPLETE | Unicast | TS24.587 After the DIRECT LINK SECURITY MODE COMPLETE message is generated, the target UE shall pass this message to the lower layers for transmission along with the target UE's layer-2 ID for unicast communication and the initiating UE's layer-2 ID for unicast communication, NRPIK, NRPEK if applicable, KNRP-sess ID,…… |
| DIRECT LINK ESTABLISHMENT ACCEPT | Unicast | TS24.587 After the DIRECT LINK ESTABLISHMENT ACCEPT message is generated, the target UE shall pass this message to the lower layers for transmission along with the initiating UE's layer-2 ID for unicast communication and the target UE's layer-2 ID for unicast communication, …... |

Observation 1: For DIRECT LINK ESTABLISHMENT REQUEST (SL-SRB0), it can be transmitted by broadcast or unicast manner.

Observation 2:  For the other PC5-S signaling (SL-SRB1, SL-SRB2) during PC5-S connection establishment procedure, only unicast is used.

Regarding to the PDCP/RLC entities maintenance, the corresponding description is in TS38.331, TS 38.323 and TS38.322 are as below:

- TS38.331 - 5.8.9.1a.4 Sidelink SRB addition The UE shall: 1> if transmission of PC5-S message for a specific destination is requested by upper layers for sidelink SRB: 2> establish PDCP entity, RLC entity and the logical channel of a sidelink SRB for PC5-S message, as specified in sub-clause 9.1.1.4; 1> if a PC5-RRC connection establishment for a specific destination is indicated by upper layers: 2> establish PDCP entity, RLC entity and the logical channel of a sidelink SRB for PC5-RRC message of the specific destination, as specified in sub-clause 9.1.1.4; 2> consider the PC5-RRC connection is established for the destination.

- TS38.323 - 5.1.1 PDCP entity establishment When upper layers request a PDCP entity establishment for a radio bearer for Uu or PC5 interface; or for NR sidelink communication for groupcast and broadcast, when receiving the first PDCP PDU, and there is not yet a corresponding PDCP entity, the UE shall: - establish a PDCP entity for the radio bearer; - set the state variables of the PDCP entity to initial values; - follow the procedures in clause 5.2.

- TS38.322 - 5.1.1 RLC entity establishment When upper layers request an RLC entity establishment, the UE shall: - establish a RLC entity; - set the state variables of the RLC entity to initial values; - follow the procedures in clause 5.2. For NR sidelink groupcast and broadcast, when receiving the first UMD PDU from a Source Layer 2 ID and Destination Layer 2 ID pair for an LCID, and there is not yet a corresponding receiving RLC entity for a radio bearer, the UE shall: - establish a receiving RLC entity; - set the state variables of the RLC entity to initial values; - follow the procedures in clause 5.2.

- TS38.321 5.22.2.2.2 Sidelink process For each PSSCH duration where a transmission takes place for the Sidelink process, one TB and the associated HARQ information is received from the Sidelink HARQ Entity. For each received TB and associated Sidelink transmission information, the Sidelink process shall: 1> if this is a new transmission: 2> attempt to decode the received data. 1> else if this is a retransmission: 2> if the data for this TB has not yet been successfully decoded: 3> instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data. 1> if the data which the MAC entity attempted to decode was successfully decoded for this TB; or 1> if the data for this TB was successfully decoded before: 2> if this is the first successful decoding of the data for this TB: 3> if this TB is associated to unicast, the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, and the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI; or 3> if this TB is associated to groupcast or broadcast and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI: 4> deliver the decoded MAC PDU to the disassembly and demultiplexing entity; 2> consider the Sidelink process as unoccupied. … …

From TS38.323 and TS38.322, it can be found that:

- For groupcast and broadcast, the PDCP and RLC entity may be established with the below conditions:

- Requested by upper layer

- Receiving a new PDU which is not yet corresponding to the existing RLC/PDCP entity

- For unicast there is only one condition to establish a RLC/PDCP entity

- Requested by upper layer

Observation 3: For unicast, the PDCP/RLC entity establishment can only be triggered by upper layer request.

From TS 38.321, it can be found that:

- For groupcast and broadcast, before deliver the decoded MAC PDU to the disassembly and demultiplexing entity, UE should check if

- The Layer-2 destination ID capture from the decoded MAC PDU subheader is equal to any of the destination ID of the UE

- But for unicast, before deliver the decoded MAC PDU to the disassembly and demultiplexing entity, UE should check if both

- The Layer-2 source ID capture from the decoded MAC PDU subheader is equal to any of the destination ID of the UE, and

- The Layer-2 destination ID capture from the decoded MAC PDU subheader is equal to any of the source ID of the UE

Since the upper layer of Rx UE doesn’t know that unicast messages from special Layer-2 ID are coming before the first PC5-S messages are transmitted. So, high layer cannot request to establish the associate Rx unicast entity to handle the receiving PDU before it coming.

As to MAC entity, it is assumed that the MAC of Rx UE doesn’t know that unicast messages from special Layer-2 ID are coming before the first PC5-S messages are transmitted, that is to say unicast MAC PDU from a new layer-2 source ID cannot be submitted to higher layers in MAC entity.

Observation 4: For unicast, when SRC field of the decoded MAC PDU is not any of the existing destination ID of the UE, MAC entity will not submit the PDU to higher layer.

Based on above analysis, we further summary whether the PDCP/RLC transmitting/receiving entities establishment procedures is complete in the following table.

Table 2 Analysis on transmitting/receiving PDCP/RLC entities establishment for SL-SRB0/SL-SRB1/SL-SRB2

| PC-S signaling | Cast type | Transmitting entity |  | Receiving entity |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PDCP | RLC | PDCP | RLC |
| DIRECT LINK ESTABLISHMENT REQUEST(SL-SRB0) | Broadcast | √ | √ | √ | √ |
|  | Unicast | √ | √ | × | × |
| DIRECT LINK AUTHENTICATION REQUEST(SL-SRB0) | Unicast | √ | √ | × | × |
| DIRECT LINK AUTHENTICATION RESPONSE(SL-SRB0) | Unicast | √ | √ | × | × |
| DIRECT LINK SECURITY MODE COMMAND(SL-SRB1) | Unicast | √ | √ | × | × |
| DIRECT LINK SECURITY MODE COMPLETE(SL-SRB1) | Unicast | √ | √ | × | × |
| DIRECT LINK ESTABLISHMENT ACCEPT (SL-SRB2) | Unicast | √ | √ | × | × |

Observation 5: For SL-SRB0, SL-SRB1 and SL-SRB2, the transmitting PDCP/RLC entities establishment procedures have been captured in spec.

Observation 6: For SL-SRB0, if it is transmitted via broadcast, the receiving PDCP/RLC entities establishment procedures have been captured in spec.

Observation 7: For SL-SRB0 or SL-SRB1, if it is transmitted via unicast, the receiving PDCP/RLC entities establishment procedures have not been captured in spec. That is to say, the high layer signaling carried by unicast SL-SRB(s) cannot be received by peer UE.

Since the Rx UE can establish PDCP/RLC entity if the first unicast PC5-S message is received successfully.  The issue is focus on the first unicast PC5-S message reception.

Based on the above observations, it is proposed that RAN2 to confirm the issue that UE has no entry to establish Rx entity to receive the first unicast PC5-S message.

Proposal 1: RAN2 to confirm the issue that UE has no entry to establish Rx entity to receive the first unicast PC5-S message (SL-SRB0/1).

There could be some potential solutions to address this issue. Final option to go with can be decided after checking companies’ view during the meeting, once the issue itself is confirmed.

Conclusion

According to the analysis in section 2, we have the following observations:

Observation 1: For DIRECT LINK ESTABLISHMENT REQUEST (SL-SRB0), it can be transmitted by broadcast or unicast manner.

Observation 2:  For the other PC5-S signaling (SL-SRB1, SL-SRB2) during PC5-S connection establishment procedure, only unicast is used.

Observation 3: For unicast, the PDCP/RLC entity establishment can only be triggered by upper layer request.

Observation 4: For unicast, when SRC field of the decoded MAC PDU is not any of the existing destination ID of the UE, MAC entity will not submit the PDU to higher layer.

Observation 5: For SL-SRB0, SL-SRB1 and SL-SRB2, the transmitting PDCP/RLC entities establishment procedures have been captured in spec.

Observation 6: For SL-SRB0, if it is transmitted via broadcast, the receiving PDCP/RLC entities establishment procedures have been captured in spec.

Observation 7: For SL-SRB0 or SL-SRB1, if it is transmitted via unicast, the receiving PDCP/RLC entities establishment procedures have not been captured in spec. That is to say, the high layer signaling carried by unicast SL-SRB(s) cannot be received by peer UE.

And we propose:

Proposal 1: RAN2 to confirm the issue that UE has no entry to establish Rx entity to receive the first unicast PC5-S message (SL-SRB0/1).

Reference

- TS 23.287 V16.5.0 Vehicle-to-Everything (V2X) services(Release 16)

- TS 24.587 V16.5.0 Vehicle-to-Everything (V2X) services in 5G System (5GS)

- TS 33.536 V16.4.0 Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services

- TS 38.321 V16.5.0 Medium Access Control (MAC) protocol specification

- TS 38.322 V16.2.0 Radio Link Control (RLC) protocol specification

- TS 38.323 V16.4.0 Packet Data Convergence Protocol (PDCP) specification