3GPP TSG-RAN WG2 Meeting #116 Electronic	R2-2110629

Online, 01 – 12 November 2021

Agenda item:	6.1.4.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Clarification regarding CodebookVariantsList-r16

WID/SID:	NR_newRAT-Core, TEI16

Document for:	Discussion and Decision

# 1	Introduction

The supportedCSI-RS-ResourceListAlt-r16 is per BC capability. Description of each parameter is aligned with the new R16 design which was targeted mainly to prevent inadvertent under-reporting of CSI-RS capabilities. However the capability description is misleading because it says “across all bands/CCs within a band combination”. However, the names of the parameters are still referring to the Rel-15 ones, i.e. they are per band, i.e. maxNumberResourcesPerBand and totalNumberTxPortsPerBand.

This TDOC is written just to drive home the point that the proposed change is fully backward compatible.

# 2	Background

## 2.1	Issue described from TS 38.306

| supportedCSI-RS-ResourceListAlt-r16 Indicates the list of supported CSI-RS resources across all bands in a band combination by referring to codebookVariantsList. The following parameters are included in codebookVariantsList for each code book type: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource across all bands within a band combination; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs within a band combination, simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs within a band combination, simultaneously. For each band in a band combination, supported values for these three parameters are determined in conjunction with supportedCSI-RS-ResourceListAlt reported in MIMO-ParametersPerBand. | BC | No | N/A | N/A |
| --- | --- | --- | --- | --- |

Observation 1: The capability supportedCSI-RS-ResourceListAlt-r16 is per BC but the capability parameters have a “per band” name which is confusing to the reader.

| codebookParameters Indicates the codebooks and the corresponding parameters supported by the UE. Parameters for type I single panel codebook (type1 singlePanel) supported by the UE, which are mandatory to report: - supportedCSI-RS-ResourceList; - a UE shall support a maxNumberTxPortsPerResource minimum value of 4 for codebook type I single panel in FR1 in the case of a single active CSI-resource across all bands in a band combination, regardless of what it reports in supportedCSI-RS-ResourceList with maxNumberTxPortsPerResource; - a UE shall support a maxNumberTxPortsPerResource minimum value of 8 when configured with wideband CSI report for codebook type I single panel in FR1 in the case of a single active CSI-resource across all bands in a band combination, regardless of what it reports in supportedCSI-RS-ResourceList with maxNumberTxPortsPerResource; - a UE shall support a maxNumberTxPortsPerResource minimum value of 2 for codebook type I single panel in FR2 in the case of a single active CSI-resource across all bands in a band combination, regardless of what it reports in supportedCSI-RS-ResourceList with maxNumberTxPortsPerResource. - modes indicates supported codebook modes (mode 1, both mode 1 and mode 2); - maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set. Parameters for type I multi-panel codebook (type1 multiPanel) supported by the UE, which are optional: - supportedCSI-RS-ResourceList; - modes indicates supported codebook modes (mode 1, mode 2, or both mode 1 and mode 2); - maxNumberCSI-RS-PerResourceSet indicates the maximum number of CSI-RS resource in a resource set; - nrofPanels indicates supported number of panels. Parameters for type II codebook (type2) supported by the UE, which are optional: - supportedCSI-RS-ResourceList; - parameterLx indicates the parameter "Lx" in codebook generation where x is an index of Tx ports indicated by maxNumberTxPortsPerResource; - amplitudeScalingType indicates the amplitude scaling type supported by the UE (wideband or both wideband and sub-band); - amplitudeSubsetRestriction indicates whether amplitude subset restriction is supported for the UE. Parameters for type II codebook with port selection (type2-PortSelection) supported by the UE, which are optional: - supportedCSI-RS-ResourceList; - parameterLx indicates the parameter "Lx" in codebook generation where x is an index of Tx ports indicated by maxNumberTxPortsPerResource; - amplitudeScalingType indicates the amplitude scaling type supported by the UE (wideband or both wideband and sub-band). supportedCSI-RS-ResourceList includes list of the following parameters: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs within a band simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs within a band simultaneously. For each codebook type, the UE may report another list of supported CSI-RS resources via supportedCSI-RS-ResourceListAlt in codebookParametersPerBand. For type I single panel codebook (type1 singlePanel) supportedCSI-RS-ResourceListAlt, - a UE shall report at least one triplet in supportedCSI-RS-ResourceListAlt with maxNumberTxPortsPerResource greater than or equal to 8 for FR1; - a UE shall report at least one triplet in supportedCSI-RS-ResourceListAlt with maxNumberTxPortsPerResource greater than or equal to 2 for FR2. | Band | FD | N/A | N/A |
| --- | --- | --- | --- | --- |

Observation 2: The capability reported per band does not have any problem in understanding as the capability parameter also is reported per band.

It is clear that the supportedCSI-RS-ResourceListAlt-r16 capability is confusing when one tries to understand it in the context of the per band combination. As each of these parameters define a value “across all bands within a band combination” the misleading names could be corrected.

## 2.2	Backward compatible solution

CA-ParametersNR-v1610 (per BC) includes codebook parameters as follows:

codebookParametersPerBC-r16                       CodebookParameters-v1610      OPTIONAL,

MIMO-ParametersPerBand (per band) includes codebook parameters as follows:

codebookParametersPerBand-r16       CodebookParameters-v1610                                                   OPTIONAL,

CodebookParameters-v1610 ::=        SEQUENCE {

supportedCSI-RS-ResourceListAlt-r16  SEQUENCE {

type1-SinglePanel-r16                SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type1-MultiPanel-r16                 SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type2-r16                            SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL,

type2-PortSelection-r16              SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF INTEGER (0..maxNrofCSI-RS-ResourcesAlt-1-r16)  OPTIONAL

}                                                                                                                                       OPTIONAL

}

CodebookParameters field descriptions

supportedCSI-RS-ResourceListAlt This field indicates the alternative list of SupportedCSI-RS-Resource supported for each codebook type. The supported CSI-RS resource is indicated by an integer value which pinpoints SupportedCSI-RS-Resource defined in CodebookVariantsList. The value 0 corresponds to the first entry of CodebookVariantsList. The value 1 corresponds to the second entry of CodebookVariantsList, and so on. For each codebook type, the field shall be included in both codebookParametersPerBC and codebookParametersPerBand.

Interestingly the CodebookVariantsList was defined for Rel-16 as follows:

CodebookVariantsList-r16 ::= SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesAlt-r16)) OF SupportedCSI-RS-Resource

SupportedCSI-RS-Resource ::=     SEQUENCE {

maxNumberTxPortsPerResource      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand        INTEGER (1..64),

totalNumberTxPortsPerBand        INTEGER (2..256)

}

Observation 3: IE CodebookVariantsList-r16 reuses the Rel-15 IE SupportedCSI-RS-Resource which has the per band capabilities.

Observation 4: The misunderstanding can be fixed if the IE CodebookVariantsList-r16 used Rel-16 IE with exactly the same fields as SupportedCSI-RS-Resource but removing the “PerBand” suffixes.

CodebookVariantsList-r16 ::= SEQUENCE (SIZE (1..maxNrofCSI-RS-ResourcesAlt-r16)) OF SupportedCSI-RS-Resource-r16

SupportedCSI-RS-Resource-r16 ::=     SEQUENCE {

maxNumberTxPortsPerResource      ENUMERATED {p2, p4, p8, p12, p16, p24, p32},

maxNumberResourcesPerBand        INTEGER (1..64),

totalNumberTxPortsPerBand        INTEGER (2..256)

}

This seems to fix the misunderstanding in a clearly backward compatible fashion as the ASN.1 encoding between SupportedCSI-RS-Resource and the Rel-16 proposed IE SupportedCSI-RS-Resource-r16 is exactly the same.

Proposal 1: Define an IE SupportedCSI-RS-Resource-r16 with exactly the same fields as SupportedCSI-RS-Resource but parameter names maxNumberResourcesPerBand and totalNumberTxPortsPerBand changed to maxNumberResources and totalNumberTxPorts respectively.

Proposal 2: RAN2 to discuss the backward compatible change in CRs in TS 38.331 and TS 38.306 for resolving the misunderstanding in resolving the issue in description of the supportedCSI-RS-ResourceListAlt-r16 capability

# 4	Conclusion

This document has made the following observations regarding the issue described above:

Observation 1: The capability supportedCSI-RS-ResourceListAlt-r16 is per BC but the capability parameters have a “per band” name which is confusing to the reader.

Observation 2: The capability reported per band does not have any problem in understanding as the capability parameter also is reported per band.

Observation 3: IE CodebookVariantsList-r16 reuses the Rel-15 IE SupportedCSI-RS-Resource which has the per band capabilities.

Observation 4: The misunderstanding can be fixed if the IE CodebookVariantsList-r16 used Rel-16 IE with exactly the same fields as SupportedCSI-RS-Resource but removing the “PerBand” suffixes.

And proposed the following proposals which fix the misunderstanding in a clearly backward compatible fashion as the ASN.1 encoding between SupportedCSI-RS-Resource and the Rel-16 proposed IE SupportedCSI-RS-Resource-r16 is exactly the same.

Proposal 1: Define an IE SupportedCSI-RS-Resource-r16 with exactly the same fields as SupportedCSI-RS-Resource but parameter names maxNumberResourcesPerBand and totalNumberTxPortsPerBand changed to maxNumberResources and totalNumberTxPorts respectively.

Proposal 2: RAN2 to discuss the backward compatible change in CRs in R2-2110627/R2-2110628 for resolving the misunderstanding in resolving the issue in description of the supportedCSI-RS-ResourceListAlt-r16 capability