3GPP TSG-RAN WG2 Meeting #116-e	R2-2111438

# Electronic meeting, November 01 – 12, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.300 | CR | 1349 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR to 36300 for Multi-USIM devices support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_MUSIM-Core |  |  |  |  |  | Date: |  |  | 2021-10-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Feature addition for Multi-USIM devices support |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add new abbreviation for MUSIM - Add new section for “Support for Multi-USIM devices” - Add new clause for “General” - Add new clause for “Paging Collision Avoidance” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Multi-USIM devices operation is not clarified in stage-2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2 Abbreviations X Support for Multi-USIM devices X.1 General X.2 Paging Collision Avoidance X.3 UE notification on Network Switching |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The Running CR is based on TS36.300_v16.6.0 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | It was revised to account for the agreements in RAN2#116-e |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

1xCSFB	Circuit Switched Fallback to 1xRTT

5GC	5G Core Network

ABS	Almost Blank Subframe

AC	Access Category

ACK	Acknowledgement

ACLR	Adjacent Channel Leakage Ratio

AM	Acknowledged Mode

AMBR	Aggregate Maximum Bit Rate

ANDSF	Access Network Discovery and Selection Function

ANR	Automatic Neighbour Relation

ARP	Allocation and Retention Priority

ARQ	Automatic Repeat Request

AS	Access Stratum

AUL	Autonomous Uplink

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BL	Bandwidth reduced Low complexity

BR-BCCH	Bandwidth Reduced Broadcast Control Channel

BSR	Buffer Status Report

C/I	Carrier-to-Interference Power Ratio

CA	Carrier Aggregation

CAZAC	Constant Amplitude Zero Auto-Correlation

CBC	Cell Broadcast Center

CC	Component Carrier

CG	Cell Group

CHO	Conditional Handover

CIF	Carrier Indicator Field

CIoT	Cellular Internet of Things

CMAS	Commercial Mobile Alert Service

CMC	Connection Mobility Control

C-plane	Control Plane

C-RNTI	Cell RNTI

CoMP	Coordinated Multi Point

CP	Cyclic Prefix

CQI	Channel Quality Indicator

CRC	Cyclic Redundancy Check

CRE	Cell Range Extension

CRS	Cell-specific Reference Signal

CSA	Common Subframe Allocation

CSG	Closed Subscriber Group

CSI	Channel State Information

CSI-IM	CSI interference measurement

CSI-RS	CSI reference signal

DAPS	Dual Active Protocol Stack

DC	Dual Connectivity

DCCH	Dedicated Control Channel

DCN	Dedicated Core Network

DeNB	Donor eNB

DFTS	DFT Spread OFDM

DL	Downlink

DMTC	Discovery Signal Measurement Timing Configuration

DRB	Data Radio Bearer

DRS	Discovery Reference Signal

DRX	Discontinuous Reception

DTCH	Dedicated Traffic Channel

DTX	Discontinuous Transmission

DwPTS	Downlink Pilot Time Slot

E-CID	Enhanced Cell-ID (positioning method)

E-RAB	E-UTRAN Radio Access Bearer

E-UTRA	Evolved UTRA

E-UTRAN	Evolved UTRAN

EAB	Extended Access Barring

ECGI	E-UTRAN Cell Global Identifier

ECM	EPS Connection Management

EDT	Early Data Transmission

EHC	Ethernet Header Compression

eHRPD	enhanced High Rate Packet Data

eIMTA	Enhanced Interference Management and Traffic Adaptation

EMM	EPS Mobility Management

eNB	E-UTRAN NodeB

EPC	Evolved Packet Core

EPDCCH	Enhanced Physical Downlink Control Channel

EPS	Evolved Packet System

ETWS	Earthquake and Tsunami Warning System

FDD	Frequency Division Duplex

FDM	Frequency Division Multiplexing

G-RNTI	Group RNTI

GBR	Guaranteed Bit Rate

GERAN	GSM EDGE Radio Access Network

GNSS	Global Navigation Satellite System

GP	Guard Period

GRE	Generic Routing Encapsulation

GSM	Global System for Mobile communication

GUMMEI	Globally Unique MME Identifier

GUTI	Globally Unique Temporary Identifier

GWCN	GateWay Core Network

GWUS	Group Wake Up Signal

H-SFN	Hyper System Frame Number

HARQ	Hybrid ARQ

(H)eNB	eNB or HeNB

HO	Handover

HPLMN	Home Public Land Mobile Network

HRPD	High Rate Packet Data

HSDPA	High Speed Downlink Packet Access

ICIC	Inter-Cell Interference Coordination

IDC	In-Device Coexistence

IP	Internet Protocol

ISM	Industrial, Scientific and Medical

KPAS	Korean Public Alert System

L-GW	Local Gateway

LAA	Licensed-Assisted Access

LB	Load Balancing

LBT	Listen Before Talk

LCG	Logical Channel Group

LCR	Low Chip Rate

LCS	LoCation Service

LHN	Local Home Network

LHN ID	Local Home Network ID

LIPA	Local IP Access

LMU	Location Measurement Unit

LPPa	LTE Positioning Protocol Annex

LTE	Long Term Evolution

LWA	LTE-WLAN Aggregation

LWAAP	LTE-WLAN Aggregation Adaptation Protocol

LWIP	LTE WLAN Radio Level Integration with IPsec Tunnel

LWIP-SeGW	LWIP Security Gateway

MAC	Medium Access Control

MBMS	Multimedia Broadcast Multicast Service

MBR	Maximum Bit Rate

MBSFN	Multimedia Broadcast multicast service Single Frequency Network

MCCH	Multicast Control Channel

MCE	Multi-cell/multicast Coordination Entity

MCG	Master Cell Group

MCH	Multicast Channel

MCS	Modulation and Coding Scheme

MDT	Minimization of Drive Tests

MeNB	Master eNB

MGW	Media Gateway

MIB	Master Information Block

MIMO	Multiple Input Multiple Output

MME	Mobility Management Entity

MMTEL	Multimedia telephony

MO-EDT	Mobile Originated Early Data Transmission

MPDCCH	MTC Physical Downlink Control Channel

MSA	MCH Subframe Allocation

MSI	MCH Scheduling Information

MSP	MCH Scheduling Period

MT-EDT	Mobile Terminated Early Data Transmission

MTC	Machine-Type Communications

MTCH	Multicast Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MUSIM	Multi-Universal Subscriber Identity Module

N2	Reference point between the NG-RAN and the AMF

NACK	Negative Acknowledgement

NAS	Non-Access Stratum

NB-IoT	Narrow Band Internet of Things

NCC	Next Hop Chaining Counter

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NG-RAN	NG Radio Access Network

NH	Next Hop key

NNSF	NAS Node Selection Function

NPBCH	Narrowband Physical Broadcast channel

NPDCCH	Narrowband Physical Downlink Control channel

NPDSCH	Narrowband Physical Downlink Shared channel

NPRACH	Narrowband Physical Random Access channel

NPUSCH	Narrowband Physical Uplink Shared channel

NPRS	Narrowband Positioning Reference Signal

NPSS	Narrowband Primary Synchronization Signal

NR	NR Radio Access

NRT	Neighbour Relation Table

NSSS	Narrowband Secondary Synchronization Signal

OFDM	Orthogonal Frequency Division Multiplexing

OFDMA	Orthogonal Frequency Division Multiple Access

OPI	Offload Preference Indicator

OTDOA	Observed Time Difference Of Arrival (positioning method)

P-GW	PDN Gateway

P-RNTI	Paging RNTI

PA	Power Amplifier

PAPR	Peak-to-Average Power Ratio

PBCH	Physical Broadcast CHannel

PBR	Prioritised Bit Rate

PCC	Primary Component Carrier

END OF FIRST CHANGE

START OF SECOND CHANGE

# X	Support for Multi-USIM devices

Editor’s note: Whether a separate section is used for MUSIM can be further discussed

## X.1	General

E-UTRAN may support one or more of the following enhancements for MUSIM UE operation:

-	Paging Collision Avoidance, as described in clause X.2;

FFS if paging cause impacts stage 2.

## X.2	Paging Collision Avoidance

The purpose of paging collision avoidance is to address the overlap of paging occasions on both USIMs when a MUSIM (e.g. dual USIM device) is in RRC_IDLE/RRC_INACTIVE state in both the networks (e.g. Network A and Network B) associated with respective USIMs.

A MUSIM device UE may determine potential paging collision on two networks and may trigger actions to prevent potential paging collision on E-UTRA connected to EPC as specified in TS 23.401 [17] or E-UTRA connected to 5GC as specified in TS 23.501 [82].

Editor’s note: It is left to UE implementation as to how it selects one of the two RATs/networks for paging collision avoidance.

END OF SECOND CHANGE

Annex: RAN2 Agreements (LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-202895)

## RAN2#116-e

4: RAN2 confirms that E-UTRA connected to 5GC scenario is also in the WID scope for paging collision avoidance. The solution agreed for NR is the baseline solution for this scenario.

1: RAN2 prefers that for EPS, the alternative IMSI or offset should be calculated in AS, i.e., RRC. Send an LS to SA2 and CT1 to indicate RAN2’s preference and request to specify the necessary details. LS will be discussed in offline [230].

Option 1 already agreed earlier, no need to optimize

5: For LTE and NR, RAN2 leaves it up to UE implementation how UE AS indicates to UE NAS that paging collision issue is identified.

7: For LTE and NR, RAN2 leaves other detailed UE behavior up to UE implementation, including how to make predictable UE behavior for RAT/Network selection to avoid paging collision, rules for declaring paging collision issue, and RAT/Network selection for reporting paging collision issue.

=> RAN2 already agreed not to have assistance information

After the session, it was noted that the agreement on no assistance information only applied for 5GS, so the notes were amended by adding the "for 5GS" as per below:

RAN2 already agreed not to have assistance information for 5GS

RAN2 will not specify MN-SN coordination of MUSIM gaps with MR-DC in Rel-17

RAN2 will not create MAC CE activation of gaps in MUSIM, but if the common gap discussion allows this anyway, RAN2 will not prevent that, either.

1: RAN2 will not work in Rel-17 for the case that Dual-RX/Single-TX UE or Single-RX/Single-TX UE stays in RRC_CONNECTED mode in NW A while performing reception and transmission in NW B (in RRC_ CONNECTED or during RRC setup/resume period).

2: MR-DC is not supported in Rel-17.

No need for LS to SA2 on this (no specification efforts needed to prevent or allow dual RRC_CONNECTED with MUSIM in Rel-17).

Wait for RAN4 feedback on gap pattern support (can use FFS in RRC for maximum value)

4: RAN2 understands that the intent of aperiodic gap is as follows (no need to specify):

- -	If until the end of the aperiodic gap the UE still has not completed activity in NW B, e.g. due to the random access for on-demand SI request, the UE should stop the activity in NW B and switch to NW A. If needed, the UE can request another aperiodic gap in NW A.

RAN2 does not intend to specify any new signalling in Rel-17 for early return. If legacy signalling allows it, RAN2 does not intend to preclude it.

5: Do not introduce gap purpose for gap related MUSIM assistance information.

6: FFS how UE indicates release of gap pattern.

7: FFS if UE is allowed to update UAI message after the UE performs cell reselection in NW B or after the UE performs handover in NW A.

8: Autonomous release of MUSIM gap by UE after N repetitions is not supported.

Bulk agreements

1: Introduce paging cause by using the ”nonCriticalExtension” in the Paging record.

2: No need to study solution proposals based on extending legacy Paging record.

4: The solution proposal to introduce paging cause in NR will be used for LTE.

5: No need to send an LS to SA2 asking to consider a NAS solution to introduce paging cause in LTE.

6: For paging reception in RRC_IDLE, UE forwards the paging cause to NAS.  It’s up to NAS whether to accept or reject the paging.

8: The AS-NAS interaction principles for NR are applied to LTE.

No LS to SA3 needed on the LTE solution. Companies can raise this up directly in SA3 if needed.

3: Adopt B.1 (parallel list with 1 optional paging cause value “voice”).

9: Introduction of paging cause impacts 38.331 and 36.331 specs; FFS if it impacts stage 2 specs (38.300 and 36.300)

7: The AS-NAS interaction for paging reception in RRC_INACTIVE is left up to UE implementation.

1	AS capability for paging collision avoidance is not needed (for any cases).

2	There is no need for AS capability for Busy indication.

6	There is no need for AS capability for Paging cause value.

Can discuss UE capabilities for periodic/aperiodic gap request and RRC processing delay requirements for MUSIM in Rel-17 further in the next RAN2 meeting.

The below is used as baseline for MUSIM capabilities (can still discuss exact details in the next meeting). FFS whether we need separate bits for periodic and aperiodic gaps. FFS if we need capability bit for leaving RRC_CONNECTED.

| Features | Index | Feature group | Components | Prerequisite feature groups | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x. Rel-17 MUSIM | X-0 | Gap support for MUSIM UE | Indicates UE support periodic gap for MUSIM | MUSIM support over NAS | UE | No | No |  |  | Optional with capability signalling |
|  |  |  | Indicates UE support aperiodic gap for MUSIM | MUSIM support over NAS | UE | No | No |  |  | Optional with capability signalling |

## RAN2#115-e

- Agreement

- 1	RAN2 retains the agreement on NAS-based busy indication for RRC_INACTIVE, and Reply SA2.

- Agreements

- Scenarios and supported gap types

- 1	RAN2 aims to support at least the below scenarios 1/2/3 in Rel-17 for cases when the UE is allowed to switch to network B without leaving connected state at network A.

- -	Scenarios 1: Periodic switching, including SSB detection/paging reception, serving cell measurement, neighboring cell measurement including intra-frequency,inter-frequency and inter-RAT measurement;

- -	Scenarios 2:  SI receiving at network B;

- -	Scenarios 3: Aperiodic (one-shot) switching with both transmission and reception at network B but will not enter RRC-connected state in NW B (e.g. no RRC connection Resume/Setup) at network B, including On-demand SI request;

- 2	For switching without leaving connected state at network A, support gap types 2a (Normal periodic gap) and 2b (Normal aperiodic gap) in Rel-17.

- 3	Only per UE level scheduling gap is supported in Rel-17 for non-DC. FFS if we support MR-DC.

- The scenarios will only be used for deriving RRC parameters. No need to capture them in e.g. Stage-2.

- Agreements

- Gap configuration and activation

- 5	The network is allowed to configure at most 3 gap patterns (for any MUSIM purpose).

- 6	Only a single aperiodic gap (for MUSIM) is supported in Rel-17. At most two periodic “gaps” (for MUSIM) and a single aperiodic gap (for MUSIM) is supported in Rel-17. FFS if signalling supports more.

- 7  	The SFN and subframe of the PCell of the network A is used in the gap configuration to calculate the gap.

- Agreements

- Periodic/Aperiodic/autonomous Gap configuration and activation

- 8:  The switching gap configuration will explicitly provide the gap starting position (e.g. offset value or start SFN and subframe explicitly), gap length and gap repetition period.

- 10:  Switching Gaps (of any type) are configured or released by RRC signalling (e.g. RRCReconfiguration message) in Rel-17. FFS if gap can be released autonomously by UE after N repetitions.

- Gap configuration assistance information

- 16 	UE is allowed to include assistance information for setup or release of gaps for both 1) periodic gaps and 2) aperiodic gap in one UEAssistanceInformation Msg.

- 18  To report the assistance information, the UE maps the timing info of the Gap on the network B  to the network A and reports the mapped timing info to the network A.

- 20  For the gap assistance information, the Gap start time, Duration of the gap and gap repetition period (for periodic) may be included. FFS is other information is included (e.g. gap purpose).

Do not support autonomous gaps for MUSIM in Rel-17.

1	UE can indicate it wants to leave RRC_CONNECTED in assistance information for MUSIM (FFS for signalling details, e.g. UAI).

3: UEAssistanceInformation message is extended for switching notification in both network switching procedures for leaving RRC_CONNECTED state and without leaving RRC_CONNECTED state.

6: UE is configured to provide assistance info for switching notification via otherConfig of RRCReconfiguration message

8: Introduce a new RRC timer for the “configured time”, used for the UE to leave RRC_CONNECTED without a response.

FFS if it's possible to configure UE to always wait for the network response (e.g. "infinite" waiting time)

7: UE is not allowed to enter RRC_INACTIVE state if no NW response message is received within a certain configured time period after the network switching notification message is sent.

9: As baseline, how to handle the case, that UE performs switching without the response from network for a configured time during switching procedure without leaving RRC_CONNECTED state, is not specified. Can re-discuss if there are serious issues found.

## RAN2#114-e

1: Send an LS to SA2 to inform that RAN2 majority would support, but there is no consensus to support NAS assistant information (similar to UE ID offset for LTE), so RAN2 thinks this issue should be discussed and decided by SA2.

2: RAN2 does not introduce RRC assistant information for paging collision issue for IDLE and INACTIVE. (Can revisit if serious problems are found.)

1: RRC signaling for network switching without leaving RRC_Connected state should allow multiple configurations of periodic “gaps” with different parameters (e.g. periodicities and durations). FFS is multiple can be active at the same time. FFS if multiple aperiodic gaps are supported.

4: UE provides assistance information to the gNB of NW A in Connected state based on the configuration of USIM of NW B for the gNB to determine the necessary switching parameters. Up to network what is the action based on UE assistance information. FFS what assistance information is needed.

1: AS -based solution for network switching includes two steps: 1-) If configured, UE can send an RRC message to leave RRC_CONNECTED for MUSIM purpose 2-) gNB may release the UE to Idle/Inactive.

2: Include the following RAN2#113bis-e agreement in the LS:

- During switching procedure for leaving RRC_CONNECTED state, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period. FFS for RRC_INACTIVE state

3: The “configured time” for AS-based solution for the UE to leave RRC_CONNECTED without a response is configured by the gNB. Indicate RAN2 is still discussing this for AS-based solution in the LS.

4: Indicate that RAN2 has not discussed the interaction between AS-based solution and any SA2 agreement on NAS messages or NAS-based solution for network switching.

## RAN2#113-bis-e

- Agreements

- 1: For the EPS PO/PF calculation, include the UE_offset to the UE_ID calculation formula.

- 2: No additional modification for the EPS eDRX case.

- Agreements

- 1	Only support NAS-based busy indication (for IDLE and INACTIVE)

- Agreements

- 1	RRC signalling is used for switching procedure without leaving RRC_CONNECTED state in network A for UE temporarily switching to network B as a baseline. FFS on additional need of MAC signalling.

- 2	During switching procedure for leaving RRC_CONNECTED state, UE is allowed to enter RRC_IDLE state if it does not receive response message from network within a certain configured time period. FFS for RRC_INACTIVE state.

1: RAN2 works to support the MUSIM paging cause feature that SA2 is working on and also addresses the paging cause issue raised by SA2 LS.

2: RAN2 attempts to reply LS to SA2 once we progress on solution and agree on CR(s) that support/address the above feature/issue.

5: If RAN2 agrees to add a paging cause value (or any other information that could lead to a specific paging cause) in Uu paging message, RAN2 specifies the relevant UE behavior (i.e. inform or passing to the upper layer) upon its reception in both LTE and NR specifications.

RAN2 does not intend to introduce alternative paging IDs for MUSIM paging (unless requested by SA2).

## RAN2#113-e

There is support for solution 1 (for 5GS) with something else, either solution 3 or 2b.

- Agreement

- Option 2b is the preferred solution to address paging collision for “LTE + LTE”.

- Agreements

- 1	MUSIM UE determines potential paging collision on two networks and triggers actions on potential paging collision avoidance.

- 2	It is left to UE implementation as to how it selects one of the two RATs/networks for paging collision avoidance.

FFS if we can make the UE behaviour predictable for paging collision avoidance

- Agreements

- 1	Switching procedure can be used to notify network A that the UE has a preference to leave RRC_CONNECTED state in network A.

- 2	The switching procedure can be used to notify network A that the UE has a preference to be kept in RRC_CONNECTED state in network A while temporarily switching to network B.

- Agreement

- 1	NAS signalling is baseline for UE reporting paging collision in 5GS side (to be confirmed by SA2).

- 2	It is FFS whether assistant information is needed for paging collision in 5GS side.

## RAN2#112-e

Use: "Extending paging signalling is possible but RAN2 haven’t decided on overall feasibility of paging cause, including how it should be supported."

With this change, the LS is approved in R2-2011241 (unseen)

From RAN2 point of view, Option 1 , 2a, 2b, and 3 are feasible to solve the paging collision issue in 5GS. Each have different effectiveness (as per analysis during the email discussion). When indicating reply to SA2, indicate both feasibility as well as effectiveness.

Indicate to SA2 that RAN2 continues to further evaluate the pros and cons of options 1, 2a, 2b, 3.

Option 4 is still allowed (but RAN2 will not specify UE implementation).

Clarifying "No E-UTRA impact" can be done in RANP.

Option 2c can be evaluated later as it doesn't work alone.

Enhancement for 5GS should be prioritized since it can handle paging collision issue in both NR+NR and NR+LTE scenarios.

Indicate to SA2 that the table 1 is a baseline on the discussion the expected time (in ms) required for UE to send a (NAS) busy indication to Network B.

From RAN2 point of view, it is feasible that the busy indication is sent as an RRC message with security for RRC_INACTIVE. FFS how this works.

RAN2 will continue to discuss RRC-based switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED. There may be different mechanisms (short/long, leaving/returning, etc.).

Provide SA2 with information on paging cause costs based on the email discussion + contributions. Indicate that this may change if assumptions change.

From RAN2 perspective, we haven't decided on paging cause feasibility yet.

RAN2 will evaluate short/long time switching in this WI

Agreements

1a: The sub-Case 3-1 is supported in WI, i.e., the switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED  includes the case where Dual-RX/Single-TX UE is in RRC_CONNECTED state in NW A while performing only reception in NW B (i.e., in RRC_idle State and RRC inactive state).

1b: For Sub-Case 3-1, whether the Rx capability coordination between UE and NW is needed can be decided after the RRC-based switching/leaving and returning procedure is defined.

2: The Sub-Case 3-2, i.e. Dual-RX/Single-TX UE stays in RRC_CONNECTED mode in NW A while performing reception and transmission in NW B(in RRC_ CONNECTED or during RRC setup/resume period ), is not considered in the WI from RAN2 viewpoint. Scheduling gap is not excluded.

4: 	FFS: The Sub-Case 4-1, i.e. Dual-RX/Dual-TX UE stays in RRC_CONNECTED mode in NW A while performing both reception and transmission in NW B without changing into RRC_CONNECTED state in NW B, is not considered in the WI from RAN2 viewpoint.

5: 	FFS: The Sub-Case 4-2, i.e. Dual-RX/Dual-TX UE stays in RRC_CONNECTED state in NW A while performing both reception and transmission in RRC_ CONNECTED in NW B, is not considered in the WI from RAN2 viewpoint.

=>	FFS if/how to ensure UE doesn't disconnect from RRC_CONNECTED during busy indication

=>	Capability change is not precluded by proposals.