3GPP TSG-WG2 Meeting #116bis electronic    	     R2-2200855

Online, 17th-26th, JAN, 2022

Source: 			CMCC

Title:  Control plane procedure

Agenda item:	8.7.2.1

Document for: 	Discussion

Introduction

In RAN2, control plan procedure for RRC establishment, RRC reestablishment, RRC remote had been discussed, the overall flow chart has been confirmed. But there are still some leftover detail issues, e.g., SRB of remote UE delivery on uu, which may block progress of other group and other topics.

In this contribution, we will discuss those issues on control plane procedure for L2 U2N relay.

Discussion

The uu RLC channel configuration of SRB0 and SRB1 have not been decided yet. As discussed in offline email discussion, relay UE is work in RRC CONNECTED state, dedicated configuration is feasible, sufficient and more flexible. Furthermore, the dedicated RLC configuration for remote UE SRB0 transmission should be configured after network identifies remote UE. In past RAN2 meeting, it was agreed that SUI message is reported to gNB and before forwarding the first SRB0 UL message of the remote UE. The SUI message could assist gNB to identify remote UE, and gNB configures the dedicated RLC configuration for SRB0 transmission.

Proposal 1: Dedicated uu RLC configuration for delivering remote UE’s SRB0.

Proposal 2: RRCSutupRequest, RRCReestablismentRequest and RRCResumeRequest message can be after RRCReconfig for relay UE.

RLC configuration for remote UE’s SRB1 is included in RRCSetup, RRCReestablisment and RRCResume message, and dedicated uu RLC configuration is sufficient for delivering remote UE’s SRB1.

Proposal 3: Dedicated uu RLC configuration for delivering remote UE’s SRB1.

Conclusions

According the above discussion we have following proposals:

Proposal 1: Dedicated uu RLC configuration for delivering remote UE’s SRB0.

Proposal 2: RRCSutupRequest, RRCReestablismentRequest and RRCResumeRequest message can be after RRCReconfig for relay UE.

Proposal 3: Dedicated uu RLC configuration for delivering remote UE’s SRB1.

References

[1] R2-2104405, Summary report of [AT113bis-e][603][Relay] Proposals from summary of agenda item 8.7.4.1 (ZTE)