3GPP TSG-RAN WG2 Meeting #116-bis-e 						R2-2201300

Electronic, 17th - 25th JAN, 2022

Source: 	Huawei, HiSilicon

Title: 	UE capability issues for eIAB

Agenda Item:	8.4.3

Document for:	Discussion

Introduction

The RAN2#116 e-meetings have agreed some enhancements on R17-IAB [1], which include the following aspects:

- MAC - LCG extension and BSR;

- RLF indications;

- F1 over NR access link (or CP-UP separation);

- inter topology routing and BAP operation, and so on;

Apparently, these issues will affect the corresponding UE (i.e., IAB-MT) capability, so the enhancement on R17-UE capability should be considered, which is discussed in this paper.

Discussion

Based on the newly agreed conclusions on R17-IAB, the corresponding enhancements on R17-UE (i.e., IAB) capability should consider the following parameters/features:

Proposal 1: For RAN2 specific eIAB capability, introduce below optional parameters/features:

- General parameters (included in UE-NR-Capability):

- Type2 indication;

- Type3 indication;

- BAP parameters (included in BAP-Parameters-r16):

- Header rewriting based inter-topology routing (including Inter-CU IAB-MT migration (e.g. BAP address related));

- Header rewriting based local re-routing;

- MAC parameters (included in MAC-ParametersCommon): LCG extension;

- MR-DC parameters (included in GeneralParametersMRDC): F1 over NR RRC;

We also prepared the TP based on 5.1.12 NR_IAB in 3GPP 38.822 [2], as detailed in Annex.

Conclusion

This paper mainly discusses the enhancements on R17-UE capability and propose:

Proposal 1: For RAN2 specific eIAB capability, introduce below optional parameters/features:

- General parameters (included in UE-NR-Capability):

- Type2 indication;

- Type3 indication;

- BAP parameters (included in BAP-Parameters-r16):

- Header rewriting based inter-topology routing (including Inter-CU IAB-MT migration (e.g. BAP address related));

- Header rewriting based local re-routing;

- MAC parameters (included in MAC-ParametersCommon): LCG extension;

- MR-DC parameters (included in GeneralParametersMRDC): F1 over NR RRC;

Similar to 5.1.12 NR_IAB in 38.822, this proposal can be captured in specs, as shown in Annex.

Reference

- Chairman notes of 3GPP TSG-RAN WG2 meeting #116-e

- 3GPP TR 38.822 V16.2.0 (2021-12): NR; User Equipment (UE) feature list (Release 16)

- Annex: Text Proposal for TS 38.822

START OF CHANGE

X.X.X	NR_eIAB

Table xxx: Layer-2 and Layer-3 feature list for NR_eIAB-Core

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | General Parameters | 1) Support Type2 indication 2) Support Type3 indication |  | type2Indication-IAB-r17 type3Indication-IAB-r17 | UE-NR-Capability | No | No | IAB-MT impact | Optional with capability signalling. |
|  |  | BAP Parameters | 1) Support header rewriting based inter-topology routing, including Inter-CU IAB-MT migration 2) Support header rewriting based re-routing |  | bap-HeaderRewritingRouing-r17 bap-HeaderRewritingLocalRerouing-r17 | BAP-Parameters-r16 | No | No | IAB-MT impact | Optional with capability signalling. |
|  |  | MAC parameters | Support LCG extension |  | paraMAC-LCGExtension-IAB-r17 | MAC-ParametersCommon | No | No | IAB-MT impact | Optional with capability signalling. |
|  |  | MR-DC Parameters | Support F1 over NR RRC |  | mr-DC-F1overRRC-IAB-r17 | GeneralParametersMRDC | No | No | IAB-MT impact | Optional with capability signalling. |