3GPP TSG-RAN WG2 Meeting #116bis electronic   						    	R2-2201376

E-Meeting, 17th Jan – 25th Jan, 2022

Source: 	Xiaomi Communications

Title:  	Clarification on the area configured for ROHC continuity

Agenda Item:	8.6.3

Document for:	Discussion and Decision

# Introduction

According to the RAN2#116-e meeting discussion, RAN2 made the following agreement for the ROHC continuity:

- For SDT, ROHC continuity functionality can be configurable between the cell and RNA. Send LS to RAN3

In the LS [2] to RAN3, the above RAN2 agreement is clarified that “the ROHC continuity function for SDT DRB can be configured as applicable for either the same cell where the RRC connection was suspended or the whole RNA”. In this contribution, we provide further clarifications on the area configured for ROHC continuity.

# Discussion

## Area configured for ROHC continuity

According to the email discussion (i.e. [Post116-e][506][SDT] RRC running CR (ZTE)) on the running 38.331 CR [3] of SDT, the above RAN2 agreement is captured as follows:

sdt-DRB-ContinueROHC ENUMERATED { cell, rna } OPTIONAL -- Need R

sdt-DRB-ContinueROHC Indicates whether the PDCP entity for the radio bearers configured for SDT, during SDT operation continues or resets the ROHC header compression protocol during PDCP re-establishment, as specified in TS 38.323 [5]. Value cell indicates that ROHC header compression continues when the UE resumes in the same cell as the cell in which the previous RRCRelease message is received. Value rna indicates that ROHC header compression continues when the UE resumes in a cell belonging to the same RNA as the cell in which the previous RRCRelease message is received.

As the running CR quoted above, when the value of “sdt-DRB-ContinueROHC” is set to “cell”, the cell for ROHC continuity is where the UE receives the RRCRelease message. From our understanding, the cell where the RRC connection was suspended means a cell of RNA.

Observation 1: The cell where the RRC connection was suspended means a cell of RNA, which does not have to be the cell where the UE receives the RRCRelease message.

Observation 2: According to the running RRC CR, when the value of “sdt-DRB-ContinueROHC” is set to “cell”, the cell for ROHC continuity is where the UE receives the RRCRelease message.

Figure 1: RLC segments for RRCRelease

As the Figure 1 illustrated above, the RRCRelease message via SRB1 (i.e. with RLC AM mode) can have more than one segments which are transmitted via more than one cells (i.e. one RLC segment of RRCRelease message via PCell and one RLC segment of RRCRelease message via SCell). If the cell for ROHC continuity is where the UE receives the RRCRelease message, it is not clear in which cell the UE should use the ROHC continuity for SDT when the RRCRelease message with segments are transmitted via more than one cells.

Observation 3: It is not clear in which cell the UE should use the ROHC continuity for SDT when the RRCRelease message with segments are transmitted via more than one cells.

Figure 2: RNA cell different from the cell transmitting RRCRelease

As the Figure 2 illustrated above, the RRCRelease message could be sent via PCell, but the RNA cell indicated in the RRCRelease message may not be the PCell, or may not even be the serving cell as the RRC_CONNECTED. As quoted below, the gNB can configured a list of cells for RNA, which may/may not be the UE’s serving cell.

38.331: RAN-NotificationAreaInfo ::= CHOICE { cellList PLMN-RAN-AreaCellList, ran-AreaConfigList PLMN-RAN-AreaConfigList, ... }

Observation 4: The cell where the RRCRelease message is transmitted may not be the RNA cell.

To resolve the issues as given in Observation 2/3, we think that the gNB can provide an explicit cell indication for the ROHC continuity.

Proposal 1: The cell where the ROHC continuity is applied is indicated via an explicit cell identity in RRCRelease message.

As the running CR quoted above, when the value of “sdt-DRB-ContinueROHC” is set to “rna”, the cell for ROHC continuity belongs to the RNA, in which the RRCRelease message has to be transmitted via a cell of this RNA.

Observation 5: According to the running RRC CR, when the value of “sdt-DRB-ContinueROHC” is set to “rna”, the cell for ROHC continuity belongs to the RNA, in which the RRCRelease message has to be transmitted via a cell of this RNA.

According to Observation 4, the cell where the RRCRelease message is transmitted does not have to a cell of the RNA. To align with the RAN2 agreement and to avoid the too restrictive configuration in which the RRCRelease message has to be transmitted via a cell of the RNA, we think that when the value of “sdt-DRB-ContinueROHC” is set to “rna”, the UE simply apply the ROHC continuity for SDT in the RNA as indicated in the RRCRelease message (i.e. ran-NotificationAreaInfo).

Proposal 2: The RNA where the ROHC continuity is applied is the same RNA as indicated via ran-NotificationAreaInfo in RRCRelease message.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation 1: According to the running RRC CR, when the value of “sdt-DRB-ContinueROHC” is set to “cell”, the cell for ROHC continuity is where the UE receives the RRCRelease message.

Observation 2: The cell where the RRC connection was suspended means a cell of RNA, which does not have to be the cell where the UE receives the RRCRelease message.

Observation 3: It is not clear in which cell the UE should use the ROHC continuity for SDT when the RRCRelease message with segments are transmitted via more than one cells.

Observation 4: The cell where the RRCRelease message is transmitted may not be the RNA cell.

Observation 5: According to the running RRC CR, when the value of “sdt-DRB-ContinueROHC” is set to “rna”, the cell for ROHC continuity belongs to the RNA, in which the RRCRelease message has to be transmitted via a cell of this RNA.

Proposal 1: The cell where the ROHC continuity is applied is indicated via an explicit cell identity in RRCRelease message.

Proposal 2: The RNA where the ROHC continuity is applied is the same RNA as indicated via ran-NotificationAreaInfo in RRCRelease message.

# Reference

[1] RAN2#116-e meeting minutes.

[2] R2-2111446, LS to RAN3, “LS on the ROHC continuity for SDT”.

[3] R2-21xxxxx, running 38.331 CR of SDT, ZTE Corporation, “Introduction of SDT”