3GPP TSG-RAN WG2 Meeting #116bis electronic   						    	R2-2201385

E-Meeting, 17th Jan – 25th Jan, 2022

Source: 	Xiaomi Communications

Title:  	Introduction of new FR2 CA bandwidth classes

Agenda Item:	8.24.1

Document for:	Discussion and Decision

# Introduction

According to the LS [1] from RAN4, RAN4 agreed to introduce new FR2 CA bandwidth classes (i.e. R, S, T, U) as release independent from REL-15), and asks RAN2 to evaluate whether this is possible. In this contribution, we provide our understandings on the introduction of new FR2 CA bandwidth classes (i.e. R, S, T, U) as release independent from REL-15.

# Discussion

## New FR2 CA bandwidth classes

According to the RRC specification 38.331 [1] and the UE capability specification 38.306 [2], the “ca-BandwidthClassDL-NR” and “ca-BandwidthClassUL-NR” which are indicated per band per band combination can indicate the legacy bandwidth class from “a” to “q” (i.e. 17 bandwidth classes with 5 bits of 32 code points) as follows:

38.331: CA-BandwidthClassNR ::= ENUMERATED {a, b, c, d, e, f, g, h, i, j, k, l, m, n, o, p, q, ...}

Figure 1：New FR2 CA bandwidth classes

According to the LS from RAN4 as quoted above, the new FR2 CA bandwidth classes are introduced for FBG2 (Fallback Group 2).

According to the RAN4 specification of 38.101-1 as quoted above, the UE is only mandated to fallback to lower order CA bandwidth class configuration that belongs to the same fallback group. This means that when the UE indicates the support of bandwidth class R in FBG2, the UE still needs to provide extra capability signalling to indicate the support of other bandwidth classes in other FBG(s).

Observation 1: When the UE indicates the support of new bandwidth class (i.e. R, S, T, U) in FBG2, the UE still needs to provide extra capability signalling to indicate the support of other bandwidth classes in other FBG(s) different from FBG2.

When the UE indicates the bandwidth class R, the legacy gNB without updating the ASN.1 is not able to understand the indication of the bandwidth class R and the fallback bandwidth of the bandwidth class R in FBG2. As such the UE would have to indicate the legacy bandwidth class F in addition to the bandwidth class R.

Observation 2: When the UE indicates a new bandwidth class, the legacy gNB without updating the ASN.1 is not able to know the indication of the new bandwidth class and the fallback bandwidth (e.g. F) of the new bandwidth class in FBG2.

Proposal 1: When the UE indicates a new bandwidth class (i.e. R, S, T, U), the UE shall also indicate bandwidth class F.

To facilitate the RRC signalling design as given by Proposal 1, we consider that the UE can indicate the new bandwidth classes via new “ca-BandwidthClassDL-NR-v17xy” or “ca-BandwidthClassUL-NR-v17xy”, which would allow the indication of the new bandwidth classes (i.e. R, S, T, U) via new “ca-BandwidthClassDL-NR-v17xy” or “ca-BandwidthClassUL-NR-v17xy”, and the indication of the legacy bandwidth class F via the legacy “ca-BandwidthClassDL-NR” or “ca-BandwidthClassUL-NR” at the same time.

Proposal 2: The indication of the new bandwidth classes (i.e. R, S, T, U) is via new capability signalling of ca-BandwidthClassDL-NR-v17xy/ ca-BandwidthClassUL-NR-v17xy.

To align with the RAN4 agreements of allowing the capability signalling for the new bandwidth classes (i.e. R, S, T, U) as release independent, we consider that the 38.331 specification can capture that the indication of the new bandwidth classes (i.e. R, S, T, U) is allowed for early implementation.

Proposal 3: The indication of the new bandwidth classes (i.e. R, S, T, U) is allowed for early implementation from Rel-15.

# Conclusions

According to the analysis given above, we have the following Observations and Proposals:

Observation 1: When the UE indicates the support of new bandwidth class (i.e. R, S, T, U) in FBG2, the UE still needs to provide extra capability signalling to indicate the support of other bandwidth classes in other FBG(s) different from FBG2.

Observation 2: When the UE indicates a new bandwidth class, the legacy gNB without updating the ASN.1 is not able to know the indication of the new bandwidth class and the fallback bandwidth (e.g. F) of the new bandwidth class in FBG2.

Proposal 1: When the UE indicates a new bandwidth class (i.e. R, S, T, U), the UE shall also indicate bandwidth class F.

Proposal 2: The indication of the new bandwidth classes (i.e. R, S, T, U) is via new capability signalling of ca-BandwidthClassDL-NR-v17xy/ ca-BandwidthClassUL-NR-v17xy.

Proposal 3: The indication of the new bandwidth classes (i.e. R, S, T, U) is allowed for early implementation from Rel-15.

# Reference

[1] R4-2119966, LSin from RAN4 to RAN2, “LS on release independence aspects of newly introduced FR2 CA BW Classes and CBM/IBM UE capability “both””.

[2] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”

[3] 3GPP TS 38.306, “NR; User Equipment (UE) radio access capabilities”