3GPP TSG-RAN WG2 Meeting #116bis-e,					R2-2201524

# Online, January 17 – January 25, 2022

Agenda Item:	8.16.3

Souce:		Samsung

Title:		Discussion on UE Capabilities relating to Rel17 eNPN features

Document for:	Discussion

# 1. Introduction

In Release 17, RAN2 discussed the support of following features as part of eNPN Work Item:

- Supporting access of external credential holders through SNPNs

- Onboarding and provisioning in SNPNs

- Emergency call/services  support in SNPNs

This contribution discusses whether any new UE capability parameter needs to be introduced to aid in realizing the eNPN features.

# 2. Discussion

The UE knows about the SNPNs providing services from external CH based on the broadcast messages (indicator in SIB1 and SIB broadcasting supported GIDs). The UE can perform cell selection based on this available broadcast information and configured subscription data, which provides priority information to perform network selection.

Since no other additional information is needed at network side regarding the UE’s capability indicating support to implement this feature, there is no need to introduce any new UE capability bit to indicate support of external CH access via SNPNs.

Proposal 1: No new UE capability bit is introduce to indicate UE’s support for external CH access via SNPN

The support of onboarding and provisioning in SNPN is another objective of Release 17 eNPN WI. The process of onboarding and provisioning is a one-time process. Similar to external CH access feature, all the information needed for UE to initiate this procedure is broadcasted as part of System Information messages. Thus, no new capability bit needs to be introduced to indicate UE’s support of onboarding and provisioning over SNPN.

Proposal 2: No new UE capability bit is introduce to indicate UE’s support of onboarding and provisioning over SNPN.

# 3. Conclusion

Proposal 1: No new UE capability bit is introduce to indicate UE’s support for external CH access via SNPN

Proposal 2: No new UE capability bit is introduce to indicate UE’s support of onboarding and provisioning over SNPN.