# 3GPP TSG RAN WG2 Meeting #116bis-e                     	    R2-2201625
E-meeting, 17 - 25 Jan. 2022

Agenda item:	8.15.3

Source:	Qualcomm Incorporated

Title:	Discussion on Inter-UE Coordination

WID/SID:	NR_SL_enh – Release 17

Document for:	Discussion and Decision

# Introduction

RAN2 made the following agreements at RAN1 #107e meeting [1].

Agreement

For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

- Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

  - For the indication of resource set, the following is supported:

    - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

      - First resource location of each TRIV is separately indicated by the inter-UE coordination information

    - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

      - FFS: UE capability details

      - 2nd SCI is UE RX optional

- Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B.

  - For the indication of resource set, the following is supported:

    - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

      - First resource location of each TRIV is separately indicated by the inter-UE coordination information

- FFS: Whether/How to use resource reservation information as coordination information

RAN2 made the following agreements at RAN1 #106e meeting [2].

Agreement

- In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

  - A UE that sends an explicit request for inter-UE coordination information can be UE-B

  - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

  - Working assumption At least a destination UE of a TB transmitted by UE-B can be UE A

  - The above feature can be enabled or disabled or controlled by (pre-)configuration

  - FFS: Details on how to support this, including (pre-)configuration signaling granularity

  - FFS: Additional details and conditions on UE-A and UE-B

- Working Assumption In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

  - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

  - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

  - The above feature can be enabled or disabled or controlled by (pre-)configuration

    - FFS: Details on how to support this, including (pre-)configuration signaling granularity

  - FFS: Additional details and conditions on UE-A and UE-B

In this contribution, we discuss at high level on inter-UE coordination.

# Discussion

It was agreed at RAN1 #107e meeting that MAC CE is always used for inter-UE coordination information for either preferred resources or non-preferred resources.

For inter-UE coordination message, low latency is required to maximize the efficacy of inter-UE coordination information. Therefore, resource sensing and selection should be avoided for transmitting an inter-UE coordination MAC CE,

To ensure that the inter-UE coordination information is not stale when received at other UEs, the inter-UE coordination MAC CE may be multiplexed with a data available to be transmitted if the MAC entity has sidelink resources allocated for the transmission and the allocated resources can accommodate the inter-UE coordination MAC CE.

Proposal 1. For inter-UE coordination MAC CE, multiplexing with an available data to be transmitted at an allocated resource is supported.

To ensure that the inter-UE coordination information is not stale when received at other UEs, the inter-UE coordination MAC CE may be transmitted as a standalone MAC CE on a dedicated resource if there is no available data transmission with sidelink resources allocated by the MAC entity already, so that the receiving UE knows where to monitor the inter-UE coordination MAC CE. The dedicated resources may be pre-co figured or configured.

Proposal 2. For inter-UE coordination MAC CE, transmitting on a dedicated resource is supported when no available data to be transmitted at an allocated resource.

It was agreed as a working assumption for UE-A at RAN1 #106e meeting that at least a destination UE of a TB transmitted by UE-B can be UE A, which may imply that any UE receives a TB transmission from UE-B can become a UE-A. For example, when sharing preferred resources for a unicast transmission, the unicast peer receiving the transmission becomes the UE-A and the other UE becomes the UE-B for this link. For non-preferred resources using groupcast or broadcast, there is no predefined relation between a UE-A and a UE-B, e.g., any UE may a UE-A when receives a TB transmission from a UE and any UE receiving this inter-UE message is a UE-B.

Proposal 3. The case, when IUC is triggered by an explicit request and destination UE of a TB transmission by UE-B is UE-A, is supported for unicast only.

# Conclusion

In this contribution, we discuss at high level on inter-UE cooperation and concluded with the following proposals.

Proposal 1. For inter-UE coordination MAC CE, multiplexing with an available data to be transmitted at an allocated resource is supported.

Proposal 2. For inter-UE coordination MAC CE, transmitting on a dedicated resource is supported when no available data to be transmitted at an allocated resource.

Proposal 3. The case, when IUC is triggered by an explicit request and destination UE of a TB transmission by UE-B is UE-A, is supported for unicast only.

# References

[1] “Draft Report of 3GPP TSG RAN WG1 #107-e v0.1.0”, RAN1 #107-e, 11th - 19th November 2021

[2] R1-2110434, “Final Report of 3GPP TSG RAN WG1 #106-e v3.0.0”, RAN1 #106bis-e, October 11th – 19th, 2021.