3GPP TSG RAN WG2#117-e	R2-2202110

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG-RAN WG1 Meeting #107-e	R1-2112862

E-Meeting, November 11–19, 2021

Title:	Reply LS on UL skipping with LCH prioritization

Response to:	R1-2106409(R2-2106746), R1-2110755 (R2-2109085)

Release:	Rel-16

Work Item:	NR_IIOT-Core, NR_L1enh_URLLC-Core

Source:	RAN1

To:	RAN2

Contact Person:

#### Name:	Lihui Wang

E-mail Address:	wanglihui@vivo.com

1. Overall Description:

RAN1 thanks RAN2 for the LSs for UL skipping with LCH prioritization in R1-2106409(R2-2106746) and R1-2110755 (R2-2109085). For the following WAs:

Working assumption: When lch-BasedPrioritization is not configured and Rel-16 CG/DG PUSCH skipping is enabled, DG always overrides CG. This working assumption is not agreed until confirmed by RAN1. Confirm the WA that LCH based prio has higher priority than UL skipping still applies, and we expect that if there are issues, RAN1 will come-back.

RAN1 discussed and would like to inform RAN2 following:

For the 1st WA, RAN1 made following agreement:

Agreement

RAN1 confirms RAN2’s following working assumption.

- When lch-BasedPrioritization is not configured and Rel-16 CG/DG PUSCH skipping is enabled, DG always overrides CG.

For the second WA, RAN1 cannot confirm RAN2’s WA on LCH based prio has higher priority than UL skipping, and would like to inform RAN2 that RAN1 has concluded that when lch-basedPrioritization is configured, Rel-16 UL skipping cannot be enabled in Rel-16. RAN1 expects RAN2 to capture above configuration restriction in TS 38.331.

In addition, RAN1 confirms that the following intended UE behavior can be supported:

- Given the understanding in RAN1 that when lch-basedPrioritization is configured and Rel-16 UL skipping cannot be enabled in Rel-16, for the case of overlapping PUSCH and SR with equal L1 priority and MAC has not yet delivered MAC PDU for the PUSCH to PHY, if SR is prioritized in MAC, MAC shall not deliver the MAC PDU for the PUSCH and shall instruct PHY for SR transmission.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully ask RAN2 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #107bis-e	17 – 25 January 2022	E-Meeting

TSG-RAN WG1 Meeting #108-e	21 February – 03 March 2022	E-Meeting