3GPP TSG RAN WG2#117-e	R2-2202115

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG RAN WG1 #107bis-e	R1-2200800

e-Meeting, January 17th – 25th, 2022

Title:	LS on Paging Enhancement

Response to:

Release:	Rel-17

Work Item:	NR_UE_pow_sav_enh-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

#### Name:	Weide Wu

#### E-mail Address:	Weide.wu@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

During RAN1 #107-bis-e meeting, RAN1 has discussed and agreed the following agreements related to new RNTI for PEI DCI (DCI format 2_7) and PEI occasion (PEI-O) design:

Agreement A new PEI-RNTI is supported for DCI format 2_7.

Agreement For the value PEI-RNTI, decide one of the following: - Alt-1: PEI-RNTI is of fixed value, and value design is up to RAN2.

Agreement - The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-O.

Agreement For whether and how to accommodate PEI-O location determination for the case POnumPerPEI is smaller than Ns, decide one of the following alternatives - Alt-2: It is supported, and UE applies the single value in PEI-F_offset for the frame-level offset and the (flooris/POnumPerPEI+1) -th value out of Ns/POnumPerPEI configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset Note: The number of PO mapping to one PEI should be multiple of Ns when POnumPerPEI is larger than Ns

Agreement If one PEI-O is associated with POs of 2 PFs, - The two PFs are consecutive PFs configured in SIB - FFS: two PFs are consecutive PFs within the same paging cycle - Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - iPO/Ns⋅T/N

Working assumption - SearchSpaceId = 0 can be configured forpeiSearchSpace for the case of CORESET multiplexing pattern 2 or 3

2. Actions:

To RAN2:

RAN1 respectfully asks RAN2 to take the above information into account for RAN2’s future work.

3. Date of Next RAN1 Meetings:

TSG RAN WG1 Meeting #108-e	21 Feb. - 03 Mar., 2022	e-Meeting

TSG RAN WG1 Meeting #109-e	16 May - 27 May, 2022	e-Meeting