3GPP TSG RAN WG2#117-e	R2-2202156

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG-RAN WG4 Meeting # 101bis-e	R4-2202420

# Electronic Meeting, Jan. 17- 25, 2022

Title:	LS to RAN2 on UL gap in FR2 RF enhancement

Release:	Rel-17

Work Item:	NR_RF_FR2_req_enh2

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Huaning Niu

#### E-mail Address:	 huaning_niu@apple.com

1. Overall Description:

In RAN4#101bis-e, RAN4 has been working on UL gaps for Tx power management (P-MPR) and made the following agreements.

On UL gap configuration:

The UL gap configuration is updated for UL MGP #3 based on SCS.

|  | UGL [ms] | UGRP [ms] |
| --- | --- | --- |
| UL MGP #0 | 1.0 | 20 |
| UL MGP #1 | 1.0 | 40 |
| UL MGP #2 | 0.5 | 160 |
| UL MGP #3 | 0.125 when SCS of active UL BWP =120kHz 0.25 when SCS of active UL BWP =60kHz | 5 |

All UL gap configurations are optional.

On UL gap activation:

MAC-CE based activation and deactivation is not supported.

On P-MPR reporting:

When UL gap is not configured/activated, P bit in PHR should be 1 during the UL gap test. There is no consensus that UE is mandated to report P-MPR when UL gap is not configured/activated.

When UL gap is configured/activated, previous agreement applies, i.e., the reported absolute P-MPR should be 0-3dB.

On UE capability:

On UL gap for Tx power management UE capability, it was agreed the UL gap capability should be defined per band.

On procedures to be prioritized over UL gap,

It was agreed that all the RACH procedure should be prioritized. FFS for other procedures.

On UE indication to NW on “need for UL gap” and “no need for UL gap”,

Enable explicit indication from UE to NW on “need for UL gap” and “no need for UL gap” is supported.

RAN4 will inform RAN2 of any further updates in future meetings if needed.

2. To RAN WG2 group.

ACTION: 	RAN4 respectfully asks RAN2 to take RAN4 conclusions in consideration for further work.

3. Date of Next TSG-RAN WG4 Meetings:

TSG-RAN4 Meeting #101-bis-e	Jan 17 - 25, 2022	Online