3GPP TSG-RAN WG2 Meeting #117 electronic	R2-2202348

Online, February 21st –March 3rd, 2022

Agenda item:	8.17.4.1

Source: 	Fujitsu

Title: 	System Information provisioning for inter-cell beam management

Document for:	Discussion and decision

Introduction

At the last RAN2 meeting, the following agreement regarding System Information provisioning for inter-cell beam management was reached [1]:

Allow NW to update UE SI information either via dedicated configuration, or via switching UE to pTRP for SI reception. FFS if these require specification modifications and whether there are critical issues with the mechanisms.

However, there are some remaining issues need to be handled. In this paper, we would like to discuss this issue and provide our proposals.

Discussion

Based on RAN2 agreement, the network can update SI information for the UE in RRC_CONNECTED either via dedicated configuration, or via switching UE to pTRP for SI reception. According to the mechanism for system information handling captured in 7.3 of TS38.300 [2], system information consists of a MIB, SIB1 and Other SI. So, the first issue is whether to allow network to update MIB, SIB1 or Other SI to UE in RRC_CONNECTED via dedicated configuration or via switching UE to pTRP for SI reception.

As shown in the figure below, MIB is periodically broadcast on BCH, SIB1 is periodically broadcast on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED, while Other SI can be periodically broadcast (on-demand) on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED.

Figure 7.3-1: System Information Provisioning [2]

In other words, the network can provide SIB1 or Other SI for the UE in RRC_CONNECTED in a dedicated manner. However, MIB cannot be provided in a dedicated manner.

Observation: According to existing mechanism, the network cannot provide MIB via dedicated configuration.

In previous discussions for BWP, MIB via dedicated configuration is not introduced with the following reasons:

- MIB only contains cell barred status information and essential physical layer information required to receive further system information

- Except SFN, there is a low possibility to modify the content of MIB

These reasons are applicable for the mechanism to switch UE to pTRP for SI reception. So, we propose to revise RAN2 agreement to:

The network can update SIBn information for the UE in RRC_CONNECTED either via dedicated configuration, or via switching UE to pTRP for SI reception.

Proposal 1: RAN2 agreement is revised:

The network can update SIBn information for the UE in RRC_CONNECTED either via dedicated configuration, or via switching UE to pTRP for SI reception.

It means that the network can provide the updated SIBn information for the UE in RRC_CONNECTED when the SIBn changes. The next issue is, in addition to SIBn modification, whether and when the NW is allowed to provide UE SI information either via dedicated configuration, or via switching UE to pTRP for SI reception.

According to existing mechanism [3], the UE may use a valid stored version of SI except MIB, SIB1, SIB6, SIB7 or SIB8 e.g. after cell re-selection and thus the SI acquisition procedure is skipped. The UE applies the SI acquisition procedure upon cell selection, cell-reselection, return from out of coverage, after reconfiguration with sync completion, after entering the network from another RAT, upon receiving an indication that the system information has changed, upon receiving a PWS notification, upon receiving request from upper layers and whenever the UE does not have a version of a stored SIB or posSIB or a valid version of a requested SIB. Therefore, the network needs to provide SIBn to UE in RRC_CONNECTED when the UE has no valid SIBn so that the UE will not apply the SI acquisition procedure.

The UE will delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid. So, from gNB point of view, the network needs to provide SIBs after 3 hours since last SI transmission.

Proposal 2: In addition to SI update, the NW needs to provide the SIB for UE in RRC_CONNECTED after 3 hours since last SIB transmission.

Based on these, the network needs to provide the SIB for UE in RRC_CONNECTED upon its change or after 3 hours since last SIB transmission either via dedicated configuration, or via switching UE to pTRP for SI reception. The next issue is which mechanism is used, dedicated configuration or switching UE to pTRP for SI reception. For dedicated configuration, signaling overhead needs to be considered. For switching UE to pTRP for SI reception, the network needs to take delay and radio link quality into account. It can be up to gNB implementation.

Proposal 3: When needed, it is up to NW to provide the SIB either via dedicated configuration, or via switching UE to pTRP for SI reception.

At the last RAN2 meeting, there is still an FFS for SI reception, i.e., if these mechanisms require specification modifications.

Regarding System Information Handling, it was captured in 7.3 of TS38.300 [2] that:

System Information (SI) consists of a MIB and a number of SIBs, which are divided into Minimum SI and Other SI: - Minimum SI comprises basic information required for initial access and information for acquiring any other SI. Minimum SI consists of: - MIB contains cell barred status information and essential physical layer information of the cell required to receive further system information, e.g. CORESET#0 configuration. MIB is periodically broadcast on BCH. - SIB1 defines the scheduling of other system information blocks and contains information required for initial access. SIB1 is also referred to as Remaining Minimum SI (RMSI) and is periodically broadcast on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED. - Other SI encompasses all SIBs not broadcast in the Minimum SI. Those SIBs can either be periodically broadcast on DL-SCH, broadcast on-demand on DL-SCH (i.e. upon request from UEs in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED), or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED (i.e., upon request, if configured by the network, from UEs in RRC_CONNECTED or when the UE has an active BWP with no common search space configured). Other SI consists of: …

In addition, regarding System information, it was captured in 5.2.1 of TS38.331 [3] that:

- For a UE in RRC_CONNECTED, the network can provide system information through dedicated signalling using the RRCReconfiguration message, e.g. if the UE has an active BWP with no common search space configured to monitor system information, paging, or upon request from the UE. …

We think that RAN2 needs to implement these agreements in corresponding specification(s), e.g. TS38.300 or TS38.331.

Proposal 4: RAN2 is asked to discuss whether stage-2, stage-3 or both specification modifications are needed to apply these mechanisms.

Conclusion

In this contribution, we have some discussions on the SI provisioning for inter-cell beam management and the following proposals are made:

Proposal 1: RAN2 agreement is revised:

The network can update SIBn information for the UE in RRC_CONNECTED either via dedicated configuration, or via switching UE to pTRP for SI reception.

Proposal 2: In addition to SI update, the NW needs to provide the SIB for UE in RRC_CONNECTED after 3 hours since last SIB transmission.

Proposal 3: When needed, it is up to NW to provide the SIB either via dedicated configuration, or via switching UE to pTRP for SI reception.

Proposal 4: RAN2 is asked to discuss whether stage-2, stage-3 or both specification modifications are needed to apply these mechanisms.

References

- Chairman’s Notes, RAN2 #116bis-e

- 3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2"

- 3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification"