3GPP TSG-RAN WG2 Meeting #117-e	R2-2202376

Online, 21st February–3rd March, 2022

Source: 	Huawei, HiSilicon

Title: 	UE capability issues for eIAB

Agenda Item:	8.4.4

Document for:	Discussion

Introduction

The RAN2#116bis-e meetings have achieved the following agreements on R17-UE capabilities [1].

- [051] Confirm to define a new UE capability for LCG Extension in MAC-ParametersCommon as optional UE capability for IAB-MT.

- [051] Define a new UE capability (1 bit) for ‘BH RLF detection indication and BH RLF recovery indication’ as optional UE capability for IAB-MT.

- [051] Define a new UE capability ‘f1c-OverNR-RRC’ as optional UE capability for IAB-MT. The parent IE of this UE capability is NRDC-Parameters under UE-NR-Capability.

- [051] Define a new UE capability for BAP header rewriting based inter-donor CU routing as optional UE capability for IAB-MT.

- [051] The single UE capability is used for all UL local re-routing trigger conditions.

- [051] Define a new type of feature group for LCG extension.

- [051] Reuse ‘RLF handling’ FG for BH RLF detection and recovery indication in Rel-17 eIAB feature list section.

- [051] Define a new type of feature group for F1-C over NR RRC.

The last meeting also agreed the following FFS issues of Rel-17 eIAB UE capability.

- [051] Following open issues of Rel-17 eIAB UE capability are FFS:

- FFS UE capability for Rel-17 intra-donor DU local-rerouting and inter-donor DU re-routing.

- FFS whether need to differentiate the capability between “inter-donor CU partial migration” and “inter-donor CU routing for topology redundancy”

- FFS the feature group for BAP header rewriting based inter-donor CU routing

- FFS the feature group for local rerouting

- [051] 7 tdocs above are Noted

Based on the above open issues of Rel-17 UE capability, this paper continues to discuss the enhancements on R17-UE capability.

Discussion

The last meeting has agreed the following open issues of Rel-17 eIAB UE capability.

- FFS UE capability for Rel-17 intra-donor DU local-rerouting and inter-donor DU re-routing. - FFS whether need to differentiate the capability between “inter-donor CU partial migration” and “inter-donor CU routing for topology redundancy” - FFS the feature group for BAP header rewriting based inter-donor CU routing - FFS the feature group for local rerouting

For the intra-donor DU local-rerouting and R17 new local rerouting triggers, we suggest not to introduce new capability signaling to describe these capabilities. This is because the new behaviour compared to R16 is only the new trigger, which is not necessary to be reported to the donor-CU, since there is no configuration from CU side corresponding to the triggers. The new UE capability should be added for the BAP header rewriting based re-routing, to cover both inter-donor-DU and inter-CU re-routing.

Proposal 1: No capability signaling for intra-donor DU local re-routing and the R17 new local re-routing triggers.

Proposal 2: Add new UE capability for BAP header rewriting based re-routing (including both inter-donor-DU and inter-CU re-routing).

Conclusion

This paper mainly discusses the enhancements on R17-UE capability and propose:

Proposal 1: No capability signaling for intra-donor DU local re-routing and the R17 new local re-routing triggers.

Proposal 2: Add new UE capability for BAP header rewriting based re-routing (including both inter-donor-DU and inter-CU re-routing).

Reference

- Chairman notes of 3GPP TSG-RAN WG2 meeting #116bis-e.