3GPP TSG-RAN WG2 Meeting #117 electronic	R2- 2202590

Online, 21st Feb – 3rd Mar, 2022

Agenda item:		8.6.3

Source:		Lenovo, Motorola Mobility

Title:		Analysis on CP open issues of SDT

Document for:		Discussion and Decision

- Introduction

In RAN2#116bis-e, some agreements on periodic RNAU was achieved.

Agreements

- UE shall not perform any periodic RNA update during SDT procedure. The rapporteur will find a simple solution to capture this in the RRC, aiming to follow legacy behavior (i.e. keep T380 running).

And the rapporteur proposed to add a condition that RNAU is only initiated if neither T319 nor Txxx are running (see running CR – section 5.3.13.8) in order to suppress RNAU whilst SDT is ongoing in the email discussion.

In RAN2#112-e, the agreement on UAC was achieved.

Agreements

4	For RACH and CG, the existing UAC procedure to determine whether access attempt is allowed, will be reused for SDT.

Although some agreements on RNAU and UAC were achieved, there are some details need to be discussed in order to support the SDT better.

- Discussion

RNAU on CG resource

According to current specification, RNAU (RAN-based Notification Area Update) should be performed for inactive UE. A UE in the RRC_INACTIVE state can be configured by the last serving NG-RAN node with an RNA and a timer (t380) for RNAU. RNAU is triggered based on the configured timer and the RAN area change. The purpose of RNAU is to inform network that UE is still located in this RAN area and the UE is reachable. RRCResumeRequest including a cause of RNA-update will be transmitted to the RAN node once RNAU is triggered. RACH is used for RRCResumeRequest transmission for RNAU. If CG resource is configured for inactive UE and there is not ongoing SDT, it is natural that CG resource can be used for RRCResumeRequest transmission for RNAU purpose.

Proposal 1: CG resource can be used for RRCResumeRequest transmission for RNAU purpose if there is not ongoing SDT.

UAC

It has been agreed that the existing UAC procedure to determine whether access attempt is allowed, will be reused for SDT. The existing UAC can be reused for the majority cases of SDT. However, there is some difference for the CG based solution. The pre-configured CG resources can be considered as dedicated resources. And it has been agreed that small data transmission is configured by the network on a per DRB/SRB basis. It means there are allocated resources for SDT if the arrival data/signalling corresponds to the configured DRB/SRB which is allowed to perform SDT. If the legacy UAC is performed and the UE is barred for the arrival data/signalling, the dedicated resources are wasted. Obviously, it is not the expected case. Then, we can consider the access attempt as allowed for this case to simplify the implementation.

Even if the packet is segmented and the subsequent data may be transmitted by the network scheduling, the first UL can still be considered as allowed and the subsequent transmission can be barred since the network will not schedule any grant for the UE.

Proposal 2: The access attempt is considered as allowed if the pre-configured CG resources are configured for SDT and the arrival data corresponds to the configured SDT DRB/SRB.

- Conclusion

In this contribution, the CP open issues on CG-SDT are discussed, we have the following proposals:

Proposal 1: CG resource can be used for RRCResumeRequest transmission for RNAU purpose if there is not ongoing SD.

Proposal 2: The access attempt is considered as allowed if the pre-configured CG resources are configured for SDT and the arrival data corresponds to the configured SDT DRB/SRB.

References

RAN2#112-e chairman note.

RAN2#116bis-e chairman note.

[POST116bis-e][511][Sdata] CP open issues (ZTE).