3GPP TSG RAN WG2 Meeting #117-e           		          	R2-2202643

Electronic meeting, 21st Feb – 3rd Mar 2022

Agenda Item:	8.4.3.2

Source:	Intel Corporation

Title:	Discussion on remaining BAP open issues

Document for:	Discussion and Decision

# Introduction

During RAN2 #116bis-e post-meeting email discussion, following open issues are captured for BAP protocol and related procedure handling:

- For inter-donor DU re-routing: It is FFS whether for upstream there would be a configuration optimization such that the “New Routing ID” is the same for all entries (a.k.a. default routing ID) - For header rewriting configuration: For inter-topology routing, the header rewriting configuration to include information that allows the boundary node to determine either the egress topology, or the ingress topology, or the traffic direction of a header-rewriting entry (selection of one of these expected). For inter-to-intra topology re-routing: Down selection between four options: 1) no header rewriting 2) single BAP header rewriting 3) double BAP header rewriting 4) default BAP routing ID.

In this contribution, we mainly focus on these remaining open issues of BAP processing.

# Discussion

## BAP header rewriting

### Configuration and topology information

It was agreed in RAN2 #116bis-e meeting that topology information in configurations (e.g. BH RLC CH mapping configuration, UL mapping configuration, routing configuration) is used to differentiate mapping between topologies at the boundary IAB-node.

- [049] The BH RLC CH mapping configuration of the boundary node includes information for the boundary node to differentiate mappings based on ingress topology and egress topology. - [049] The UL mapping configuration to include information for the boundary node to determine the egress topology of each UL mapping entry. - [049] The routing configuration to include information that allows the boundary node to determine the topology each routing entry applies to. RAN3 to decide on St3-related aspects.

Observation 1: Topology information is used to differentiate each entry in BH RLC CH mapping configuration, UL mapping configuration and routing configuration.

It is still FFS how to select the entry during BAP header rewriting for different BAP header rewriting scenarios based on the topology of (re-)routing. It was also agreed to use terminology “F1-terminating CU’s topology” and “non-F1-terminating CU’s topology” to refer different topologies controlled by two different IAB-donor CUs.

There are three scenarios which requires BAP header rewriting at the boundary IAB-node:

Scenario 1: Inter-donor DU re-routing (upstream only).

Scenario 2: Inter-donor CU partial migration.

Scenario 3: Inter-donor CU topology redundancy.

In scenario 1, both IAB-donor DU are under the same IAB-donor CU. There’s no topology change during BAP header rewriting. For scenario 2 and 3, packet’s topology changes from F1-terminating CU’s topology to non-F1-terminating CU’s topology after BAP header rewriting.

However, the trigger condition of BAP header rewriting of each scenario is still different. For scenario 1, according to RAN2 #116e meeting agreement, the boundary IAB-node will only trigger BAP header rewriting when no available next hop can be found in the routing table. The boundary IAB-node is still a dual-connected IAB-node.

Intra topology - For Upstream, The pre-condition/criteria of “BAP header rewriting for re-routing” is that there is no available next hop found based on BAP routing ID and based on BAP address in the routing table (e.g. due to BH RLF, congestion or type2 indication, etc.), as in R16.

Observation 2: The boundary IAB-node (dual-connected IAB-node) will only perform BAP header rewriting for inter-donor DU re-routing when no available next hop is found.

For scenario 2, BAP header rewriting is performed as the boundary IAB-node is partially migrated to another IAB-donor CU. For this scenario, the boundary IAB-node is a single-connected IAB-node whose F1-C terminated at the source IAB-donor CU and RRC terminated at the target IAB-donor CU. The boundary IAB-node itself is aware of its own status of inter-donor CU partial migration.

Observation 3: The boundary IAB-node is single-connected IAB-node when it performs BAP header rewriting for inter-donor CU partial migration.

For scenario 3, BAP header rewriting can either be performed due to IAB-donor CU’s decision on load balancing or due to BH RLF. The boundary IAB-node is dual-connected to two IAB-donor CUs, while one of the IAB-donor CU plays as the F1-terminating CU to provide F1-C related configurations, including routing configuration, BAP header rewriting configuration, etc.

Observation 4: The boundary IAB-node is dual-connected IAB-node when it performs BAP header rewriting for inter-donor CU topology redundancy.

Based on above observations, the BAP header rewriting table cannot be configured together for intra-topology and inter-topology, as the boundary IAB-node can only support two BH links simultaneously.

Observation 5: The boundary IAB-node can only receive BAP header rewriting configuration for intra-topology or BAP header rewriting configuration for inter-topology, as BAP header rewriting for intra-topology and inter-topology will not exist simultaneously.

When configuring BAP header rewriting table for inter-donor DU re-routing, IAB-donor CU does not need to differentiate the topology, each entry of the “previous routing ID” and the “new routing ID” will not conflict with each other since they are under the same topology.

Proposal 1: BAP header rewriting table for inter-donor DU re-routing does not need to include topology information.

For scenario 2 and 3, as captured in RAN3 BL CR, IAB transport migration management procedure is used to exchange information between the F1-terminating IAB-donor-CU and the non-F1-terminating IAB-donor-CU of a boundary IAB-node, for the purpose of managing the migration of the boundary and descendant IAB-node traffic between the topologies managed by the two IAB-donor-CUs. The request and response messages include BH information of the F1-terminating topology and non-F1-terminating topology, respectively [1]. The BH information includes BAP routing ID as well as BH RLC CH ID in upstream and/or downstream.

Observation 6: RAN3 has agreed to use IAB Transport Migration Management Procedure to exchange boundary IAB-node’s BH information in both topology between F1-terminating IAB-donor CU and non-F1-terminating IAB-donor CU.

Therefore, when providing BAP header rewriting table, it is much easier for the F1-terminating IAB-donor CU to differentiate each entry by topology information. Additionally, it should be noted that the “previous routing ID” in the upstream can be the “new routing ID” in the downstream. To avoid conflict among BAP routing IDs within two topologies, topology information should be carried together with “previous routing ID” and “new routing ID” separately. For example, “F1-terminating topology – previous routing ID” – “non-F1-terminating topology – new routing ID”.

Proposal 2: BAP header rewriting table includes ingress topology information of the previous routing ID and egress topology information of the new routing ID for inter-donor CU partial migration and topology redundancy.

### Inter-donor DU re-routing

As discussed during RAN2 #116bis-e meeting, it is FFS whether for upstream there would be a configuration optimization such that the “New Routing ID” is the same for all entries (a.k.a. default routing ID)

As summarized in [2], there are four options proposed for rewriting mapping for inter-donor DU re-routing:

Option a: No optimization, i.e., inter-donor-DU re-routing uses configurations of (Ingress BAP routing ID, Egress BAP routing ID)-pairs. For this option, we need to resolve the ambiguity between re-routing and inter-topology routing for a boundary node as discussed during [AT116bis-e][049][eIAB].

Option b: Rewriting mapping for inter-donor-DU re-routing is based on a default egress BAP routing ID(s) configured for each parent link.

Option c: Rewriting mapping for inter-donor-DU re-routing is based on the BAP routing IDs included in the routing entries configured for each parent.

Option d: Others.

As analysed in Section 2.1.1, there’s no ambiguity between re-routing and inter-topology routing for the boundary IAB-node, as for a single boundary IAB-node, intra-topology and inter-topology cannot exist simultaneously. A simple mapping between ingress BAP routing ID and egress BAP routing ID would be sufficient.

Proposal 3: Option a “inter-donor-DU re-routing uses configurations of (Ingress BAP routing ID, Egress BAP routing ID)-pairs” is used for BAP header rewriting configuration for inter-donor DU re-routing.

### Inter-to-intra traffic routing

During RAN2 #116bis-e meeting email discussion, there are four options for BAP header rewriting for inter-to-intra topology re-routing [2].

Option 1: No header rewriting is applied, and the upstream packet’s BAP routing ID in the ingress topology contains the BAP address of the IAB-donor-DU in the same topology.

Option 2: Header rewriting is applied based on a header-rewriting entry, which contains the packet’s ingress BAP routing ID and the BAP routing ID of the packet’s egress topology after inter-to-intra re-routing.

Option 3: Header rewriting is applied based on a header-rewriting entry, which contains the BAP routing ID of the packet’s intended egress topology after inter-topology routing and the BAP routing ID of the packet’s egress topology after inter-to-intra re-routing.

Option 4: The boundary node is configured with a default BAP routing ID for each topology via RRC, and such default BAP routing ID can be used as the egress routing ID when applying inter-topology rerouting.

Following existing agreement on how to handle packet routing at the boundary IAB-node, Option 3 is the most straightforward option, as the boundary IAB-node should perform header rewriting first if routing ID matches BAP header rewriting configuration, then perform routing and select egress link according to the new routing ID.

- Go with B, including the following: - - If BAP address matches, deliver to upper layer; - Else: - - If routing ID matches rewriting table, perform the header rewriting; - - perform routing and mapping to BH RLC CH.

However, the inter-to-intra traffic routing scenario can only happen when the BH link of the inter-topology is not available, e.g. BH RLF, congestion, type-2 RLF indication, etc. Therefore, the boundary IAB-node is aware of the unavailable egress link or unavailable egress routing IDs when receiving BH RLF, congestion indication and type-2 RLF indication. If the new routing ID in the BAP header rewriting configuration is the unavailable routing ID, there’s no need to perform header rewriting.

Observation 7: There’s no need to perform BAP header rewriting if the new routing ID is unavailable due to BH RLF, congestion, type-2 RLF indication, etc.

Compared with Option 3, Option 1 can reduce unnecessary BAP header rewriting at the boundary IAB-node.

Proposal 4: No BAP header rewriting is needed during Inter-to-Intra traffic routing.

# Conclusion

In this contribution, we discussed the remaining open issues for BAP protocol.

We propose following observations and proposals:

Observation 1: Topology information is used to differentiate each entry in BH RLC CH mapping configuration, UL mapping configuration and routing configuration.

Observation 2: The boundary IAB-node (dual-connected IAB-node) will only perform BAP header rewriting for inter-donor DU re-routing when no available next hop is found.

Observation 3: The boundary IAB-node is single-connected IAB-node when it performs BAP header rewriting for inter-donor CU partial migration.

Observation 4: The boundary IAB-node is dual-connected IAB-node when it performs BAP header rewriting for inter-donor CU topology redundancy.

Observation 5: The boundary IAB-node can only receive BAP header rewriting configuration for intra-topology or BAP header rewriting configuration for inter-topology, as BAP header rewriting for intra-topology and inter-topology will not exist simultaneously.

Observation 6: RAN3 has agreed to use IAB Transport Migration Management Procedure to exchange boundary IAB-node’s BH information in both topology between F1-terminating IAB-donor CU and non-F1-terminating IAB-donor CU.

Observation 7: There’s no need to perform BAP header rewriting if the new routing ID is unavailable due to BH RLF, congestion, type-2 RLF indication, etc.

Proposal 1: BAP header rewriting table for inter-donor DU re-routing does not need to include topology information.

Proposal 2: BAP header rewriting table includes ingress topology information of the previous routing ID and egress topology information of the new routing ID for inter-donor CU partial migration and topology redundancy.

Proposal 3: Option a “inter-donor-DU re-routing uses configurations of (Ingress BAP routing ID, Egress BAP routing ID)-pairs” is used for BAP header rewriting configuration for inter-donor DU re-routing.

Proposal 4: No BAP header rewriting is needed during Inter-to-Intra traffic routing.

# References

- R3-221551, BL CR to XnAP on Rel-17 eIAB

- R2-2202050, [Post116bis-e][079][eIAB] Open Issues (Qualcomm)