3GPP TSG-RAN WG2 Meeting #117 electronic                                    R2-2202707

Online, February 21 – March 3, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.314 | CR | 0021 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to R16 38.314 on PRB Usage for MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2022-2-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | PRB usage for MIMO was specified in Rel-16 to reflect the PRB usage at the case of MU-MIMO and multiple MIMO layers. Alpha in the matrix was constant integer value. But in the real network, the integer value may not be suitable for some cases. Operator sometime need to configure Alpha with float value. It is also proposed to correct the error in the legacy matrix for PRB usage for MIMO. Since for each sampling occasion (i.e., OFDM symbol), the available PRB is not always the same, due to occupied by reference signalling, SSB, PDCCH etc. So using a constant value N(T) as the number of available PRB is not accurate. Therefore, the number of available PRB should be corrected as the sum of available PRB for each sampling occasion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following are added or corrected to support PRB usage for MIMO - Correct the metrics for PRB usage for MIMO - Correct the granularity for Alpha |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to configure Alpha with float value and to reflect the correct available PRB for each sampling occasion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.7.1, 4.2.1.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

# 4	Layer 2 measurements

### 4.2.1	Measurements valid for all gNB deployment scenarios

<< Partially omitted >>

#### 4.2.1.7	PRB Usage for MIMO

##### 4.2.1.7.1	PDSCH PRB Usage for MIMO in the DL per cell

This measurement provides the total usage (in percentage) of PDSCH physical resource blocks (PRBs) for MIMO in the downlink per cell. The objective of the measurement is to measure usage of time and frequency resources. A use-case is OAM performance observability.

Protocol Layer: MAC, PHY

Table 4.2.1.7.1-1: Definition for PDSCH PRB Usage for MIMO in the DL per cell

| Definition | PDSCH PRB Usage for MIMO in the DL per cell is calculated in the time-frequency domain. Detailed Definition: MT=∀i∀j{M1ij(T)*Lij(T)}∀j{Pj(T)}NT*PT*Alpha*100, where explanations can be found in the table 4.2.1.7.1-2 below. |
| --- | --- |

Table 4.2.1.7.1-2: Parameter description for PDSCH PRB Usage for MIMO in the DL per cell

| M(T) | Total PDSCH PRB usage per cell which is percentage of PRBs used, averaged during time period T with integer value range: 0-100 |
| --- | --- |
| M1ij(T) | A count of PDSCH PRBs used for traffic transmission for UE i on single MIMO layer per cell at sampling occasion j. Counting unit for PRB is 1 Resource Block x 1 symbol. (1 Resource Block = 12 sub-carrier) |
| Lij(T) | The number of MIMO layers scheduled for UE i at sampling occasion j. |
| i | A UE i that is scheduled during time period 𝑇. |
| j | Sampling occasion during time period T. A sampling occasion is 1 symbol. |
| NT |  |
| P(T)Pj(T) | Total number of PDSCH PRBs available for sampling occasion j on single MIMO layer per cell. |
| T | Time Period during which the measurement is performed. |
| Alpha | Constant value configured by OAM with float value range: 1.00-100.00. With this parameter, M(T) should not be larger than 100. |

##### 4.2.1.7.2	PUSCH PRB Usage for MIMO in the UL per cell

This measurement provides the total usage (in percentage) of PUSCH physical resource blocks (PRBs) for MIMO in the uplink per cell. The objective of the measurement is to measure usage of time and frequency resources. A use-case is OAM performance observability.

Protocol Layer: MAC, PHY

Table 4.2.1.7.2-1: Definition for PUSCH PRB Usage for MIMO in the UL per cell

| Definition | PUSCH PRB Usage for MIMO in the UL per cell is calculated in the time-frequency domain. Detailed Definition: MT=∀i∀j{M1ij(T)*Lij(T)}∀j{Pj(T)}NT*PT*Alpha*100, where explanations can be found in the table 4.2.1.7.2-2 below. |
| --- | --- |

Table 4.2.1.7.2-2: Parameter description for PUSCH PRB Usage for MIMO in the UL per cell

| M(T) | Total PUSCH PRB usage per cell which is percentage of PRBs used, averaged during time period T with integer value range: 0-100 |
| --- | --- |
| M1ij(T) | A count of PUSCH PRBs scheduled for traffic transmission for UE i on single MIMO layer per cell at sampling occasion j. Counting unit for PRB is 1 Resource Block x 1 symbol. (1 Resource Block = 12 sub-carrier) |
| Lij(T) | The number of MIMO layers scheduled for UE i at sampling occasion j. |
| i | A UE that is scheduled during time period 𝑇. |
| j | Sampling occasion during time period T. A sampling occasion is 1 symbol. |
| NT |  |
| P(T)Pj(T) | Total number of PUSCH PRBs available for sampling occasion j on single MIMO layer per cell. |
| T | Time Period during which the measurement is performed. |
| Alpha | Constant value configured by OAM with float value range: 1.00-100.00. With this parameter, M(T) should not be larger than 100. |

## << End of change >>