3GPP TSG-RAN WG2 Meeting #117 electronic											R2-2202723

Online, 21st Feb – 3rd Mar, 2022

Agenda item:       6.2.2

Source: 	Huawei, HiSilicon

Title: 	Summary of RRC corrections

Document for:	Discussion and Decision

# Introduction

This document summarizes the contributions submitted to AI 6.2.2. Rapporteur’s recommendations on how to treat them are provided.

# Correction CRs to be discussed together with Rapporteur’s Misc CR

The following correction CRs could be discussed together. The agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR.

Table 1: Correction CRs to be discussed together offline

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2202714 | Miscelleneous CR on 38.331 | Huawei, HiSilicon | - In clause 6.3.2, add the value description of cN-Threshold and remove description of SL-CBR. - Add a field description for IE sl-BWP-Id to clarify that the sidelink bandwidth parts are identified from BWP-Id 1 to maxNrofBWP. - In clause 6.3.5, remove description of SL-Thres-RSRP in the field description, and add in the IE description of SL-Thres-RSRP-List . - Fix the editorial errors. |  |
| R2-2203174 | Clarification on SL power control parameter | vivo |  | This change is based on the assumption that RAN2 can agree that sl-MaxTransPower is used as the parameter to limit the transmitted power of PSSCH/PSCCH for V2X. We can first discuss RAN 4 LS in R2-2200121_R4-2120047 and submitted draft reply LSs.. If the agreement is achieved in RAN2, the adoption of this change is straightforward plus perhaps wording revision. If no agreement (e.g. waiting for RAN1 input), this CR can be postponed. |
| R2-2203286 | Correction on HARQ attribute of SL SRB option1 | ZTE Corporation, Sanechips, OPPO | 11. Since (for 116 meeting agreed change on sl-HARQ-FeedbackEnabled), only RRC signaling is modified, the agreed change can only be applied on SL DRB. As SL SRB are used for sidelink unicast communication, all SL SRBs use specified RB configuration, and it does not include the HARQ attribute. Based on current spec(i.e. only LCH with HARQ disabled can use resource pool without PSFCH), SL SRB still can not use the SL grant associated to resource pool without PSFCH resource pool. Therefore, to align SL DRB configuration, HARQ attribute will always be configured for SL SRB. 2. And to reduce the latency of SL SRB, HARQ disabled is configured in specified SCCH configuration. (for SL-SRB 3 and similarly for SL-SRB 0,1,2) | This and next CR are related to the 116 meeting merged change “Network always includes this field” sl-HARQ-FeedbackEnabled. It is understood that, as long as the value of sl-HARQ-FeedbackEnabled is selected as Enabled or Disabled, there will be no issue for MAC PDU multiplexing, according to the discussion in 116 meeting. It can be discussed whether to fix this value as Disabled in SL-SRB attribute or let UE implementation to select. Another approach can be that, as there seems no reference of sl-HARQ-FeedbackEnabled for SRB in RRC spec, MAC spec can adopt e.g. a NOTE similar to: “NOTE 2: sl-HARQ-FeedbackEnabled is set to disabled for the transmission of a MAC PDU only carrying CSI reporting MAC CE.” in clause 5.22.1.4.1.2. |
| R2-2203287 | Correction on HARQ attribute of SL SRB option2b | ZTE Corporation, Sanechips,vivo | (Another option for the same raised issue is that): To provide more flexible, the HARQ attribute of SL SRB is selected by UE implementation: |  |

Proposal 1: Discuss the CRs listed in Table 1 together, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR.

# Conclusions

The contributions submitted to AI 6.2.2 are summarized, Rapporteur’s recommendations are as follows.

Proposal 1: Discuss the CRs listed in Table 1 together, the agreed changes for RRC spec could be merged into Rapporteur’s miscellaneous correction CR.