3GPP TSG-RAN WG2 Meeting #117-e	R2-2202984

Online, February, 2022

Agenda Item	: 8.15.2 (NR_SL_enh-Core)

Source	: LG Electronics Inc.

Title	: Consideration on the remaining issues for SL DRX

Document for	: Discussion and Decision

1.	Introduction

Before starting RAN2 117e-meeting, RAN2 had some time for discussing the remaining issues via post 116bis e-mail discussion ([POST116bis-e][705][V2XSL] Open issues on SL DRX (OPPO)). In this contribution, we investigated more issues related to the e-mail discussion.

2.	Discussion

2.1 Prior SL DRX configuration when initial SL DRX configuration

In the post-116bis e-mail discussion [1], we’ve discussed whether to apply default SL DRX for BC for the PC5-S messages which are sent after the DCR message and before SL DRX configuration is applied. In addition to this issue, we need to discuss how to handle when RX UE transmits a reject message during the initial SL DRX configuration. In the normal case, the RX UE can use the prior SL DRX configuration after sending a rejection message, but in the initial SL DRX configuration, there is no prior SL DRX configuration between TX UE and RX UE. So, if RX UE transmits a reject message to TX UE, it's not clear what the prior SL DRX configuration between TX UE and RX UE is during the initial setting.  Because SL DRX configuration has never been configured yet. We can assume that the SL DRX configuration applied to the PC5-S messages which are sent after the DCR message and before SL DRX configuration can be regarded as a prior SL DRX configuration. However, RAN2 needs to clarify whether this assumption is valid or not.

Proposal 1: During the initial setting, after RX UE transmits a rejection message to TX UE, it needs to be clarified what the prior SL DRX configuration between TX UE and RX UE is.

-	Using default SL DRX configuration

-	Always-on operation

2.2 RX UE operation connected with mode 1 TX UE of which serving gNB does not have SL DRX capability

According to the previous RAN2 agreement, when TX UE is in mode 1 operation connected with a gNB that has SL DRX capability, the gNB configures SL DRX for peer RX UE. Meanwhile, when TX UE is in mode1 operation connected with gNB that does not have SL DRX capability, we are still discussing via the POST116bis e-mail discussion [1]. If the serving gNB of TX UE doesn't have SL DRX capability, the TX UE may not be able to transmit SL DRX configuration for RX UE. Because the serving gNB of TX UE cannot schedule proper resources on the active time of the TX UE. In this case, we need to consider how the RX UE can know the TX UE cannot transmit SL DRX configuration even the TX UE has SL DRX capability. If the RX UE does not know whether the TX UE can provide SL DRX configuration or not, how long the RX UE should wait for SL DRX configuration? The RX UE has to know the TX UE has no choice but to provide always-on SL configuration for RX UE even the TX UE has SL DRX capability.

Observation 1: When TX UE is in mode 1 operation connected with SL DRX capable gNB, the serving gNB of TX UE provides SL DRX configuration for RX UE.

Observation 2: When TX UE is in mode 1 operation connected with DRX incapable gNB, the TX UE may have no choice but to provide always-on SL configuration even the TX UE has SL DRX capability.

Observation 3: If TX UE informs having SL DRX capability to RX UE, the RX UE expects to receive SL DRX configuration from TX UE. But if the TX UE is in mode 1 operation connected with SL DRX incapable gNB, RX UE may not receive any SL DRX configuration from TX UE.

If the RX UE knows that the TX UE with having SL DRX capability cannot provide SL DRX configuration for RX UE, the RX UE may perform other operations. For example, the RX UE can perform release the SL connection or keep the SL connection as giving up power-saving. It’s all depending on RX UE implementation. But the RX UE can decide its operation by knowing the TX UE’s situation. We think there could be the following alternatives that the RX UE can know the situation of TX UE. One alternative method is TX UE informs always-on indication using PC5-RRC message. Or, RX UE waits for some time period after transmitting assistance information. If there is no SL DRX configuration from TX UE within the time, the RX UE can regard it as the always-on configuration.

Proposal 2: If TX UE is in mode 1 operation connected with SL DRX incapable gNB, the RX UE has to know the TX UE has no choice but to provide always-on SL DRX configuration, even the TX UE has SL DRX capability.

Proposal 3: There could be three alternatives that RX UE can know the TX UE has no choice but to provide always-on SL DRX configuration.

-	Alt1: TX UE informs indication to the RX UE that the serving gNB of TX UE doesn’t have SL DRX capability and TX UE itself is in mode1 operation (via PC5-RRC message).

-	Alt2: TX UE replaces gNB’s SL DRX capability with its own SL DRX capability when the TX UE is in mode1 during capability exchange with RX UE.

-	Alt3: If TX UE doesn’t provide any SL DRX configuration for RX UE within some specified time after RX UE sends assistance information, the RX UE can regard it as an always-on SL DRX configuration.

2.3 Mode transition of TX UE connected with gNB having no SL DRX capability

### 5.8.9 Sidelink RRC procedure #### 5.8.9.1 Sidelink RRC reconfiguration ##### 5.8.9.1.1 General Figure 5.8.9.1.1-1: Sidelink RRC reconfiguration, successful Figure 5.8.9.1.1-2: Sidelink RRC reconfiguration, failure The purpose of this procedure is to modify a PC5-RRC connection, e.g. to establish/modify/release sidelink DRBs, to (re-)configure NR sidelink measurement and reporting, to (re-)configure sidelink CSI reference signal resources and CSI reporting latency bound. The UE may initiate the sidelink RRC reconfiguration procedure and perform the operation in sub-clause 5.8.9.1.2 on the corresponding PC5-RRC connection in following cases: - the release of sidelink DRBs associated with the peer UE, as specified in sub-clause 5.8.9.1a.1; - the establishment of sidelink DRBs associated with the peer UE, as specified in sub-clause 5.8.9.1a.2; - the modification for the parameters included in SLRB-Config of sidelink DRBs associated with the peer UE, as specified in sub-clause 5.8.9.1a.2; - the (re-)configuration of the peer UE to perform NR sidelink measurement and report. - the (re-)configuration of the sidelink CSI reference signal resources and CSI reporting latency bound. In RRC_CONNECTED, the UE applies the NR sidelink communications parameters provided in RRCReconfiguration (if any). In RRC_IDLE or RRC_INACTIVE, the UE applies the NR sidelink communications parameters provided in system information (if any). For other cases, UEs apply the NR sidelink communications parameters provided in SidelinkPreconfigNR (if any). When UE performs state transition between above three cases, the UE applies the NR sidelink communications parameters provided in the new state, after acquisition of the new configurations. Before acquisition of the new configurations, UE continues applying the NR sidelink communications parameters provided in the old state.

According to the above 5.8.9.1.1 in 38.331 documents, when TX UE performs mode transition, TX UE can keep SL communication with RX UE. But, in the case of SL DRX, we need to consider when the TX UE performs mode transition with gNB having no SL DRX capability.

If the TX UE in mode 1 connected with a gNB not having SL DRX capability transits to mode2, the TX UE operation needs to be clarified. When TX UE is in mode1, the TX UE might not provide SL DRX configuration for RX UE, even the RX UE transmits assistance information. because the serving gNB of TX UE doesn't have SL DRX capability. Therefore, the TX UE and RX UE may communicate with each other in an always-on configuration. But when the TX UE becomes to transit from mode1 to mode2, the TX UE may SL DRX configuration for the RX UE based on the latest assistance information.

Proposal 4: When the TX UE in mode 1 connected with SL DRX incapable gNB transits to mode2, the TX UE can provide SL DRX configuration for RX UE based on the received latest assistance information from RX UE.

Meanwhile, if the TX UE in mode2 SL DRX transits to mode 1 with not capable SL DRX gNB, the TX UE and RX UE may not communicate with each other using SL DRX anymore. In this case, TX UE should inform the RX UE that only an always-on configuration becomes available to keep the connection. The RX UE receiving this information can make a decision whether to keep the connection without SL DRX or release. When the SL DRX TX UE in mode 2 transits mode 1 via a gNB that is not capable SL DRX, the TX UE can inform always-on configuration to the RX UE directly via PC5-RRC message.

Proposal 5: When SL DRX TX UE in mode 2 transits to mode 1 connected with SL DRX incapable gNB, the TX UE should inform the RX UE that only always-on configuration becomes available.

Proposal 6. RX UE receiving the information that only always-on configuration becomes available can make a decision whether to keep the link without power saving or release.

3.	Conclusion

Proposal 1: During the initial setting, after RX UE transmits a rejection message to TX UE, it needs to be clarified what the prior SL DRX configuration between TX UE and RX UE is.

-	Using default SL DRX configuration

-	Always-on operation

Observation 1: When TX UE is in mode 1 operation connected with SL DRX capable gNB, the serving gNB of TX UE provides SL DRX configuration for RX UE.

Observation 2: When TX UE is in mode 1 operation connected with DRX incapable gNB, the TX UE may have no choice but to provide always-on SL configuration even the TX UE has SL DRX capability.

Observation 3: If TX UE informs having SL DRX capability to RX UE, the RX UE expects to receive SL DRX configuration from TX UE. But if the TX UE is in mode 1 operation connected with SL DRX incapable gNB, RX UE may not receive any SL DRX configuration from TX UE.

Proposal 2: If TX UE is in mode 1 operation connected with SL DRX incapable gNB, the RX UE has to know the TX UE has no choice but to provide always-on SL DRX configuration, even the TX UE has SL DRX capability.

Proposal 3: There could be three alternatives that RX UE can know the TX UE has no choice but to provide always-on SL DRX configuration.

-	Alt1: TX UE informs indication to the RX UE that the serving gNB of TX UE doesn’t have SL DRX capability and TX UE itself is in mode1 operation (via PC5-RRC message).

-	Alt2: TX UE replaces gNB’s SL DRX capability with its own SL DRX capability when the TX UE is in mode1 during capability exchange with RX UE.

-	Alt3: If TX UE doesn’t provide any SL DRX configuration for RX UE within some specified time after RX UE sends assistance information, the RX UE can regard it as an always-on SL DRX configuration.

Proposal 4: When the TX UE in mode 1 connected with SL DRX incapable gNB transits to mode2, the TX UE can provide SL DRX configuration for RX UE based on the received latest assistance information from RX UE.

Proposal 5: When SL DRX TX UE in mode 2 transits to mode 1 connected with SL DRX incapable gNB, the TX UE should inform the RX UE that only always-on configuration becomes available.

Proposal 6. RX UE receiving the information that only always-on configuration becomes available can make a decision whether to keep the link without power saving or release.

4.	Reference

[1] POST116bis-e][705][V2XSL] Open issues on SL DRX (OPPO)