3GPP TSG-RAN WG2 Meeting #117-e	R2-2203128

# Online, 21 February – 03 March 2022

Agenda item:	8.19.2

Source: 	Huawei, HiSilicon

Title: 	On measurement gap handling for Msg3 repetitions

Document for:	Discussion and Decision

- Introduction

In the last RAN2 meeting, the MAC running CR for NR CE was discussed and noted in [1]. In this contribution, we identify one missing points on measurement gap handling for Msg3 repetitions and provide our suggested corrections to the MAC Running CR.

- Discussion

5.14	Handling of measurement gaps

During a measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]:

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 or the MSGA payload as specified in clause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running:

2>	monitor the PDCCH as specified in clauses 5.1.4 and 5.1.5.

1>	else:

2>	not monitor the PDCCH;

2>	not receive on DL-SCH.

According to the following MAC spec, the Msg3 and MsgA payload is not subject to the measurement gap. Provided Msg3 repetition is introduced, it is unclear what to do with Msg3 repetition within a bundle after initial Msg3 transmission. From our understandings, Msg3 repetition is used to enhance coverage that is even more critical than inter-RAT/frequency measurement during the gap. Thus we think the Msg3 repetition should not be subject to the measurement gap, i.e. the MAC entity can transmit on UL-SCH for Msg3 including all the repetitions during a measurement gap.

Proposal 1: During a measurement gap, the MAC entity shall transmit on UL-SCH for Msg3 including all repetitions of the Msg3 transmission.

- Conclusion

In this contribution, we identity some miscellaneous MAC impacts for CE.

Proposal 1: During a measurement gap, the MAC entity shall transmit on UL-SCH for all repetitions of the Msg3 transmission.

And a TP to the current MAC spec is provided in the Annex.

- Reference

- R2-2201966 Running 38.321 CR for NR coverage enhancements, ZTE Corporation

- Annex TP

Start of changes

5.14	Handling of measurement gaps

During a measurement gap, the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]:

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 (including all the repetitions within a bundle) or the MSGA payload as specified in clause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer or the msgB-ResponseWindow is running:

2>	monitor the PDCCH as specified in clauses 5.1.4 and 5.1.5.

1>	else:

2>	not monitor the PDCCH;

2>	not receive on DL-SCH.

End of changes