3GPP TSG-RAN WG2 Meeting #117-e	R2-2203134
Electronic, 21 February – 3 March 2022

Agenda item:	8.24.1

Source:	Huawei, HiSilicon

Title:	Discussion on the DC location report for more than 2CCs

Document for:	Discussion

- Introduction

A LS [4] is received from RAN4 on additional DC location reporting for more than 2 CC are configured for UE, where RAN2 is asked to implement the signalling. RAN2 made the following agreement based on the discussion in RAN2-116bise as following:

For default DC location derivation, the UE signals:

- 1. the choice of frequency component, among {Activated CC, Configured CC, Activated BWP, Configured BWP}.

- 2. the choice of UL and/or DL for frequency component, among {UL, DL, Edge most frequencies among any DL and UL}

The network specifies the radio resource configuration (including BWP / CC activation state) for which the UE is requested to report the offset to default DC location. FFS how the radio resource configuration is specified.

Introduce a new release-17 network request for the extended DC location reporting for more than 2 UL CCs.

Upon a new release-17 network request, the UE reports the extended DC location reporting for more than 2 UL CCs, i.e. the release-17 network request does not trigger the reporting of reportUplinkTxDirectCurrent and reportUplinkTxDirectCurrentTwoCarrier-r16

In this contribution, we will further discuss the signalling report for DC location offset report.

- Discussion

For the options where default DC location is calculated by the configured carrier or configured BWP, there is only one default DC location and at most two corresponding offsets(for dual PA scenario) are needed for such options. Therefore, the offsets corresponding to default DC locations calculated by the configured carrier or configured BWP can be directly included in RRC Complete messages without need of optimization on the signaling.

Proposal 1: the offsets corresponding to default DC locations options calculated by the configured carrier or configured BWP can be directly included in RRC Complete messages without need of optimization on the signalling.

For the options where default DC location is calculated by the active carrier or active BWP, there are quite many(up to several thousands) possible default DC locations based on the combination of active BWP/carrier in uplink and downlink, and it is not realistic to report all the possible offsets corresponding to each possible default DC locations. It has been agreed that the network specifies the radio resource configuration (including BWP / CC activation state) for which the UE is requested to report the offset to default DC location. The specified radio resource configuration should include the BWP combinations of all configured CC where each CC/BWP can be in active/deactivate state, one example of the requested combination can be encoded as below:

requestedCombinationList::=          SEQUENCE (SIZE(1..maxCombinanation)) OF RequestCombination

RequestCombination::=                 SEQUENCE (SIZE(1..maxCC)) OF CC-State

CC-State::=                              SEQUENCE {

dlCarrier          carrierState  OPTIONAL,

ulCarrier          carrierState  OPTIONAL

}

carrierState::=                 Choice {

deActivated          NULL,

Activated             ActiveCarrierState,

}

ActiveCarrierState::=                              SEQUENCE {

activeBWP             INTEGER (0..maxNrofBWPs-1)  OPTIONAL

}

Proposal 2: in case the default offsets is calculated by the active carrier or active BWP, the network requests DC location offsets by including the BWP combinations of all configured CC where each CC/BWP can be in active/deactivated state.

- Conclusion

Based on the above discussion, we recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: the offsets corresponding to default DC locations options calculated by the configured carrier or configured BWP can be directly included in RRC Complete messages without need of optimization on the signalling.

Proposal 2: in case the default offsets are calculated by the active carrier or active BWP, the network requests DC location offsets by including the BWP combinations of all configured CC where each CC/BWP can be in active/deactivated state.

Reference

- [R4-2011906], LS on additional DC location reporting for intra-band UL CA