3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-2203199

Elbonia, 21 February – 3 March 2022

Agenda item:	8.11.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Reporting of GNSS Positioning Integrity Result

WID/SID:	NR_pos_enh - Release 17

Document for:	Discussion and Decision

# 1	Introduction

Based on the investigation in the SI phase, specification support for GNSS-based positioning integrity will be examined as an objectives of Rel-17 WI [1]:

RP-210903: Revised WID on NR Positioning Enhancements (RAN Plenary #91e, March 2021) …… - Specify the signalling, and procedures to support GNSS positioning integrity determination, including [RAN2, RAN3]: - The assistance information that will be used to support integrity determination - The information that will be used to provide the positioning integrity KPIs and integrity results - Support of integrity for UE-based and UE-assisted A-GNSS positioning. Note: This objective is applicable to NR and E-UTRA. ……

In RAN2 #115e, some key agreements relating to signaling exchange between LMF and UE for integrity support has been reached:

Agreements:

Proposal 1: Agree that the GNSS feared events will be addressed in the WI.

Proposal 2 (modified): Agree that all for A-GNSS positioning methods, positioning integrity determination is supported in LPP.

Proposal 3: Agree that additional IEs are needed in LPP to support A-GNSS positioning integrity determination.

Proposal 4: The specific algorithms used for positioning integrity shall be up to implementation.

Proposal 5: For interoperability, the use of “hard-coded” parameters should be minimized and instead the needed parameters should be sent explicitly in the assistance data.

Proposal 6: RAN2 agrees that the PL will be reported in the Integrity Results. It is FFS whether Mode 2 and the TIR, AL, TTA that were used in the integrity calculation will also be reported in the integrity results.

Proposal 8: Agree that the UE feared events will be handled in the implementation for UE-based (network-assisted) methods of positioning integrity determination.

Proposal 10: Agree that the LMF feared events can be handled via implementation for the UE-based (network-assisted) and UE-assisted (LMF-based) methods of positioning integrity determination.

Proposal 11: RAN2 agrees to use Common Positioning IEs to transfer the KPIs and Integrity Results.

Proposal 12: RAN2 agrees that the LPP procedures can be used to transfer the KPIs and Integrity Results. For UE-assisted, the LCS procedures remain FFS in the case of MO-LR.

Agreements:

In Rel-17, we do not address the data transmission feared event (i.e. we rely on the system’s existing methods for assuring data integrity).

Agreements:

Proposal 1:	The support of GNSS integrity is enabled by using existing NG-RAN positioning architecture.

Proposal 2:	Any additional functional elements, positioning/integrity modes, etc. should be introduced only when needed.

Agreements:

Proposal 3 (modified):	Separate procedures for "A-GNSS Positioning Integrity" as proposed in R2-2107503 will not be defined; the existing A-GNSS (and general location) Procedures are applicable/sufficient.

Proposal 4 (modified):	RAN2 confirms that LPP messages RequestLocationInformation and ProvideLocationInformation are used to transfer integrity KPIs/results, respectively, for GNSS positioning at least for UE-based mode.

Proposal 5 (modified):	RAN2 confirms that LPP messages RequestAssistanceData and ProvideAssistanceData are used to transfer integrity assistance data for GNSS positioning at least for UE-based mode.

This paper aims to address the follow up of this agreement which has some FFS:

RAN2 agrees that the PL will be reported in the Integrity Results. It is FFS whether Mode 2 and the TIR, AL, TTA that were used in the integrity calculation will also be reported in the integrity results.

In the email discussion [Pre117-e][POS][610], the open issue relating to integrity requirement information in LPP has been discussed. In addition, whether Reporting Mode 2 should be supported (i.e. Integrity Event Flagging) has not been labelled as a critical issue for WI completion. Hence, we discuss whether RAN2 should consider supporting Integrity Reporting Mode 2.

# 2	Discussions

In TR 38.857, we have identified two modes of positioning integrity result reporting:

- Mode 1 of Integrity Result Reporting : PL Reporting The integrity computing entity calculates the PL, based on the measurement, assistance information and TIR. Then, the calculated PL is directly reported to where the LCS client resides (Network or UE). Hence, the integrity computing entity does not judge whether the positioning system is still available, it simply provides whatever PL value it has obtained. It is left to the LCS client itself to determine if the positioning system is still available based on the reported PL. - Mode 2 of Integrity Result Reporting : Integrity Event Flagging The integrity computing entity calculates the PL, based on the measurement, assistance information and TIR. Then, the integrity computing entity further compares the calculated PL with the given AL to determine if the positioning system is still available to offer trustable position estimation. Thus, the integrity computing entity may only have to report a binary flag (0 and 1) to indicate whether the positioning system is available or not. Thus, in this case the LCS client can be directly informed about the system availability, without conducting further evaluation by itself.

RAN2 has agreed that at least Mode 1 (PL reporting) will be supported, but this is FFS on whether Mode 2 should be supported too. We think both of these modes should be supported, and the mode could be selected based on the request. Several companies have argued that Mode 2 is not needed because the LCS client can derive by itself. Nevertheless, we think Mode 2 is beneficial in terms of latency reduction, where the LMF can quickly learn that there is an integrity event and some positioning quality boosting mechanism should be done (e.g. by implementation the LMF may determine that RAT-D positioning schemes should be involved as well), without having to further interact with the LCS client.

Proposal 1: The reporting mode of  “integrity event flagging”  (Mode 2) should be supported.

For UE-based integrity in MT-LR cases, the LMF may first send a request of reporting to the UE via LPP for integrity information. This is already captured in the LPP running CR, where CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message:

…

IntegrityInformationRequest-r17 ::= SEQUENCE {

-- FFS

}

…

It is generally agreeable that integrity requirement information such as TIR, AL, and TTA could be included in this IE. Moreover, if Proposal 1 above is agreed, then we think this IE should further include an indicator that allows the UE to know which of the reporting mode it should apply. Upon the reception of this request, the UE derives and reports the integrity result (PL or flagging) accordingly.

Proposal 2: The IE IntegrityInformationRequest-r17 should further include a reporting mode indicator that allows the UE to know whether Integrity Reporting Mode 1 or Mode 2 it should apply for this integrity request.

# 3	Conclusion

In this paper, we have discussed our views on GNSS positioning integrity reporting, the following proposals are drawn:

Proposal 1: The reporting mode of  “integrity event flagging” (Mode 2) should be supported.

Proposal 2: The IE IntegrityInformationRequest-r17 should further include a reporting mode indicator that allows the UE to know whether Integrity Reporting Mode 1 or Mode 2 it should apply for this integrity request.