3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-2203260

Elbonia, 21 February – 03 March 2022

Agenda item:	8.22.3.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on open issues for pre-configured MG

WID/SID:	NR_MG_enh-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In RAN2-116bis meeting, a list of open issues was identified as summarized in [1]. In this contribution, we would like to provide our view to address below Open Issues for Pre-Configured MG.

Pre-configured MG: Company tdocs invited: P1-1: Discuss support of case 4 where NW signals the pre-configured gap and BWP status via RRC, then UE follows BWP status to activates/deactivates gap upon BWP switching P1-2: Support pre-configured MG under CA based on BWP switching on a single CC

# 2	Discussion

## 2.1	Support of NW-controlled (de)activation pre-configured measurement gap

In our understanding, it is RAN4’s intention to support both NW-Controlled activation/deactivation mechanism and UE autonomous activation/deactivation mechanism as indicated in RAN4-100e meeting LS [2]. Furthermore, at RAN4-101e meeting, RAN4 clarified in LS [3] that the indicator(s) of MG status (activation/de-activation) per BWP can be used as the flag to indicate whether NW-Controlled or UE autonomous pre-MG (de)activation should be used by UE.

If the network provides the activation/deactivation status via RRC signalling, the UE will not use autonomous rules to determine the activation/deactivation status of the pre-configured MG. It will follow the per-BWP status indicated by the network.

Observation 1: NW-Controlled (RRC-based) Pre-MG activation/deactivation mechanism is already supported by RAN4.

For RAN2’s signalling design, it was agreed in RAN2-116e that the UE autonomous (de)activation mechanism should be supported while it is FFS for the NW-Controlled activation/deactivation mechanism.

In our view, RAN2 should follow the RAN4 agreement to support NW-Controlled activation/deactivation mechanism unless there are critical technical issues spotted by RAN2. Obviously, from signalling point of view, it is feasible that NW signals the pre-configured gap and MG status per BWP to UE via RRC, then UE follows per-BWP status to activates/deactivates gap upon BWP switching.

Furthermore, the complexity of gap enhancements in NR is exploding (e.g. gap enhancements discussed in two RAN2-led WIs (MUSIM and NTN) , two RAN4-led WIs (MG enhancements, FR2 enhancements) and one RAN1-led WI (ePos gap enhancement)),  we believe it is so complex that it is impossible to account all inter-actions between all configuration options now. The NW indicated per-BWP gap status facilitate NW to control gap ON/OFF to avoid any misunderstanding between NW and UE. This is quite important for different MG enhancements interworking. Hence, NW controlled mechanism is always helpful for feature interactions and future feature design.

Proposal 1: RAN2 to define the signalling and procedure for NW-Controlled Pre-MG activation/deactivation mechanism.

For NW-Controlled activation/deactivation mechanism, the NW should indicate the MG status (active/de-active) per BWP to UE via RRC. Then UE follows MG status per BWP to activates/deactivates gap upon BWP switching (or other event triggers which is up to RAN4). We share the view that the MG status (active/de-active) per BWP can be delivered to UE either in the same RRC message with Pre-MG configuration (which configures the Pre-MG parameters) or in a different RRC message.

Proposal 2: For NW-Controlled activation/deactivation mechanism, NW may indicate the MG status (active/de-active) per BWP to UE in the same RRC message with Pre-MG configuration or in a different RRC message.

Since the MG status (active/de-active) per BWP is mandatory for NW-Controlled activation/deactivation mechanism, as indicated in LS [3], RAN4 agreed that if the network provides the activation/deactivation status via RRC then the UE will not use autonomous rules to determine the activation/deactivation status of the pre-configured MG. This means the indicator(s) of MG status (active/de-active) per BWP can be used as the flag to indicate whether NW-Controlled or UE autonomous pre-MG (de)activation should be used by UE.

Proposal 3: RAN2 to confirm that the indicator(s) of MG status (active/de-active) per BWP can be used as the flag to indicate whether NW-Controlled or UE autonomous pre-MG (de)activation should be used by UE.

## 2.2	Support pre-configured MG under CA

RAN4 agreed that pre-configured MG can be applied under CA in Rel-17 as indicated in LS [3].

- Support pre-configured MG under CA but based on BWP switching on a single CC

In the open issue discussion in RAN2[1], there are two possible interpretations on RAN4 request especially on “a single CC”.

- Interpretation 1: Pre-MG status is determined based on the combination of active BWP(s) on multiple CC(s). RAN4 will not touch the case where BWP(s) on multiple CC(s) are being switched at the same time.

- Interpretation 2: Pre-MG status is based on the BWP on one certain CC.

As RAN4 agreed “UE will combine the pre-MG status indication from NW for each induvial CC for all configured measurement objects” in WF [5], our understanding is that interpretation 1 is the intention agreed in RAN4.

Observation 2: For Pre-MG applied under CA, UE’s Pre-MG status is determined based on the combination of active BWP(s) on multiple CC(s).

For the UE autonomous activation/deactivation mechanism, RAN4 agreed in WF [5] that, the same criteria as those for single CC rule-based pre-MG activation/deactivation should be applied under CA case.  If the target MOs can be covered by multiple BWPs in different CC(s), the MG can be deactivated. From RAN2 point of view, we think there is no additional effort needed to support this case.

Criteria for the rule-based pre-MG (de)activation under CA Same criteria as these for single CC rule-based pre-MG activation/deactivation [refer to Issue 2-2 Option 1] Pre-MG activation/deactivation criteria - The general applicability rules in TS38.133 for measurement gap for SSB, CSI-RS L3 in TS38.133 can be reused as the rules for UE to autonomously determine the pre-MG activation/deactivation status - If MG is not required by any of the configured measurements, the MG is deactivated. - If MG is required by one or more of the configured measurements, the MG is activated.

Proposal 4: For rule-based Pre-MG (de)activation under CA, RAN2 has no extra effort to support the Pre-MG in CA in comparison with single CC. UE should follow RAN4 criteria to decide the Pre-MG status on multiple CC(s).

For NW-Controlled (RRC-based) Pre-MG activation/deactivation mechanism, the key aspect for RAN2 is how to indicate the MG status (active/de-active) per BWP because there can be multiple active BWPs in multiple CC(s) at the same time. According to WF [5], RAN4 agreed that NW should indicate the per-BWP Pre-MG status to UE which is defined for each of individual CC. And then UE will combine the NW indicated Pre-MG status indication for each individual CC and decide the UE’s Pre-MG activation status under CA.

Criteria for the signaling-based pre-MG (de)activation under CA The principle for the signaling-based Pre-MG (de)activation under CA can be based on: - NW will forward the per-BWP pre-MG status indications to UE by RRC message, which is defined for each of induvial CCs (e.g. Table S1, S2 for the CC1 and CC2 respectively) - UE will combined the pre-MG status indication from NW for each induvial CC for all configured measurement objects. - FFS on the rules on how to combine the individual per-BWP pre-MG status . - Option 1a - When configured with per-UE gap, - assume the per-UE gap is ON as long as the pre-MG status of active DL BWP in one of the CCs is ON, - and assume the per-UE gap is OFF only if the pre-MG status of active DL BWP in all CCs are OFF. - When configured with per-FR gap, - assume the gap is ON as long as the pre-MG status of active DL BWP in one of the CCs in the same FR is ON, - and assume the gap is OFF only if the pre-MG status of active DL active DL BWP in all CCs in the same FR are OFF - Option 1b - For per-UE pre-MG, - UE assumes the pre-MG is ON as long as the pre-MG status for active DL BWP in one of the activated CCs or for one of the deactivated SCCs is ON, - and assume the pre-MG is OFF only if the pre-MG status for active DL BWP in all activated CCs and for all deactivated SCCs are OFF. - For per-FR pre-MG, - UE assumes the pre-MG is ON as long as the pre-MG status for active DL BWP in one of the activated CCs or for one of the deactivated SCCs in the same FR is ON, - and assume the pre-MG is OFF only if the pre-MG status for active DL BWP in all activated CCs and for all deactivated SCCs in the same FR are OFF

It is still open in RAN2 how to indicate pre-BWP pre-MG status for single CC scenario, it can be put inside the BWP configuration or inside the gap configuration. For CA case, the same message format as for single CC case can be applied but NW should include the status indication per BWP for each CC.

Proposal 5: For NW controlled (RRC-based) Pre-MG (de)activation under CA, NW should indicate per-BWP pre-MG status indications to UE for each of individual CCs, which uses the same message format as for single CC case.

Regarding how NW decides the per-BWP Pre-MG status for each of individual CC when UE is in CA mode, we think it is up to NW implementation. For each BWP, the NW may consider only the measurements with RSs within the CC which the BWP belongs to and whether all RSs to be measured are within all the configured CC as suggested by [6]. However, we don’t think there is a need to capture that in specification which will restrict the NW implementation.

Proposal 6: In CA mode, how to decide per-BWP per-MG status indication for each of induvial CCs is up to NW implementation.

For UE behavior when BWP switching happens, RAN4 agreed that UE will combine the NW indicated Pre-MG status indication for each individual CC and decide the UE’s Pre-MG activation status. But it is still FFS in RAN4 on the rules of combination.

Observation 3: For NW controlled (RRC-based) Pre-MG (de)activation under CA, the principle of how the UE combines each per BWP status indication across all serving cells is up to RAN4.

# 3	Conclusion

This document has made the following observations:

Observation 1: NW-Controlled (RRC-based) Pre-MG activation/deactivation mechanism is already supported by RAN4.

Observation 2: For Pre-MG applied under CA, UE’s Pre-MG status is determined based on the combination of active BWP(s) on multiple CC(s).

Observation 3: For NW controlled (RRC-based) Pre-MG (de)activation under CA, the principle of how the UE combines each per BWP status indication across all serving cells is up to RAN4.

And proposed the following:

Proposal 1: RAN2 to define the signalling and procedure for NW-Controlled Pre-MG activation/deactivation mechanism.

Proposal 2: For NW-Controlled activation/deactivation mechanism, NW may indicate the MG status (active/de-active) per BWP to UE in the same RRC message with Pre-MG configuration or in a different RRC message.

Proposal 3: RAN2 to confirm that the indicator(s) of MG status (active/de-active) per BWP can be used as the flag to indicate whether NW-Controlled or UE autonomous pre-MG (de)activation should be used by UE.

Proposal 4: For rule-based Pre-MG (de)activation under CA, RAN2 has no extra effort to support the Pre-MG in CA in comparison with single CC. UE should follow RAN4 criteria to decide the Pre-MG status on multiple CC(s).

Proposal 5: For NW controlled (RRC-based) Pre-MG (de)activation under CA, NW should indicate per-BWP pre-MG status indications to UE for each of individual CCs, which uses the same message format as for single CC case.

Proposal 6: In CA mode, how to decide per-BWP per-MG status indication for each of induvial CCs is up to NW implementation.

# Reference

[1] R2-2202054 "[Post116bis-e][085][MGE] Open Issues", Intel

[2] R4-2115438 “LS on R17 NR MG enhancements – Pre-configured MG”, Huawei

[3] R4-2202615 "LS on R17 NR MG enhancements – Pre-configured MG", Intel

[4] R2-2201925 “[AT116bis-e][062][MGE] pre-configured measurement gap”, Intel

[5] R4-2202614 “WF on R17 NR MG enhancements – Pre-configured MG”, Intel

[6] R2-2200832 “Discussion on Pre-configured MG”, Huawei