3GPP TSG-RAN2 #117-e  	R2-2203265

# Electronic meeting, February, 2022

Agenda item:		8.4.5 (NR_IAB_enh-Core)

Source:	LG Electronics Inc.

Title: 	Resolving open issues on BH RLF indications

Document for:	Discussion and Decision

1. Introduction

This contribution aims to resolve stage-2 open issues on BH RLF indications.

2. Discussion

Partial local re-routing and type-2 triggering

In the offline [AT-116bis][ [048], the following proposal has significant support but not agreed mainly due to the potential complexity introduced by supporting partial local re-routing.

Proposal 1b: [OPEN] partial re-routing upon BH RLF results in no triggering of type-2 indication, as per the current agreement.

Note, however, that RAN2 agreed that upon failure of a BH, a full local re-routing is not mandatory for a node capable of re-routing as captured below:

[048] Execution of local re-routing of all affected traffic among re-routable traffic upon BH RLF is not mandatory for a node capable of local re-routing. This can be revisited if there is a severe issue.

Since RAN2 agreed to support partial local re-routing, we can confirm the statement: if a node upon detecting a BH RLF performs partial local re-routing, it does not trigger type-2 indication.

Proposal 1: To confirm that if a node upon detecting a BH RLF performs partial local re-routing, it does not trigger type-2 indication

This statement in proposal1 is used as baseline for further discussion below.

Further propagation of type-2/3 indications and contents of the indications

The most outstanding FFS is whether to support further propagation of type-2/3 indications. This FFS also affects the content of typ-2/3 indications. There has been extensive discussion on these issues in the past meetings. There are two possible sets of alternatives that RAN2 can choose:

- Option 1: Not support further propagation of type-2/3 indication.

- Option 2: Support further propagation of type-2/3 indication.

Companies provided the following input for their preference during the AT-116bis#48 offline,:

- Option1 is preferred since

  - Option1 is simpler.

  - The time interval between type-2 and type-3/4 will not be long, hence the propagation just brings overhead and no clear gain. By the time a descendant node gets the type-2, a parent node may have already recovered its link or re-established.

  - The Child IAB-node can trigger Type2 indication by its own condition. So we think RAN2 don’t need to discuss any other additional condition for further forwarding the Type2 indication.

  - think RLF indication only reflects its own BH link radio condition. Also, it is highly possible that BH RLF may be recovered soon, then the original link could be reused. Further propagating type-2 RLF indication to descendant IAB-nodes would introduce extra overhead for sending type-2/3 RLF indication via many hops during a very short period. For the complex and mixture scenario mentioned by rapporteur above, if there are many child IAB-nodes in the middle of the topology are single-connected nodes, it is also possible that when the original IAB-node is recovered from the BH RLF, the propagated type-2 RLF is still passing to the descendant IAB-node. However, this would be a wrong indication due to multi-hop delay.

- Option2 is preferred since

  - Option2 allows for more chance of re-routing at lower descendent nodes to circumvent upper node(s) experiencing BH failure(s), i.e., Option 2 would provide better performance.

  - The situation at descendant node is similar as the IAB node which detects BH RLF. Specifically, the descendant node may cannot perform local rerouting for affected traffic, e.g. inter-donor rerouting is not available, UP link fails in EN-DC or CU-UP separation case. In these cases, descendant node needs to propagate type 2 indication to its child nodes.

  - DC may not be available in wider scale in the IAB networks, hence Opt.2 would provide benefits. Furthermore, propagation enables descendant nodes stop/slow down UL traffic which may be lost if RLF/re-routing eventually happens.

RAN2 should notice that option 2 needs to further resolve the following issues:

- Open issue 2a) In case a node receives a type-2 indication, it is FFS whether to i) simply forward the received type-2 indication without regenerating a type-2 indication to send or ii) whether to re-generate a type-2 indication to send (based on updated availability of the routing IDs)

- Open issue 2b) For the routing information to be included in the type-2/3 indication, it is FFS i) whether it should be routing IDs or ii) whether it should be destination IDs being unavailable/available.

For the open issue 2a), it is our view that a simple forwarding would be sufficient. The reasoning is given as follows.

- Note that re-generation of type-2 indication by a node is needed only when the node needs to update the content of the type-2 indication to send. Also note that the update happens only when the unavailable routing IDs in the type-2 indication to send is different from the unviable routing IDs in the received type-2 indication. Such update could happen only if the node were allowed to further propagate the type-2 indication even when the node performs partial re-routing triggered by the received type-2 indication.

- For instance, consider the case that a node receives a type-2 indication including unavailable routing IDs={3,4}, and the node then performs local re-routing only for routing ID={3}. In this case, if the node were allowed to further propagate a type-2 indication to child nodes, it may need to re-generate type-2 indication to only indicate unavailable routing ID={4}. Note however that RAN2 already agreed that upon detection of BH failure, a node cannot trigger type-2 indication if the node is capable of partial re-routing. If the same logic applies to the node which receives a type-2 indication, if the node performs partial local re-routing upon reception of type-2 indication, the node cannot propagate type-2 indication, i.e., there is no such case to update the content of received type-2 indication for further propagation. Therefore, the further propagation should be enabled only by a simple forwarding of received indication without re-generating a type-2/3 indication to send.

Observation1: if further propagation of type-2/3 indications is supported, it is sufficient to enable further propagation by a simple forwarding of received indication without re-generating a type-2/3 indication to send

For the open issue 2b), it is our view that destination ID information is insufficient. The reasoning is given as follows:

- If the type-2 indication only indicates unreachable destination ID information, this information cannot distinguish which routing IDs are actually unreachable. For instance, consider that two routing IDs share the same destination, and one of the two becomes unreachable due to BH RLF, triggering type-2 indication. If destination ID is only included in the type-2 indication, this may trigger a node receiving the type-2 indication to re-route available routing ID as well as unavailable routing ID. i.e., unnecessary local re-routing may be triggered at lower node(s).

Observation2: if further propagation of type-2/3 indications is supported, unavailable/availalbe routing ID should be included in type-2/3 indications.

Based on the observation1 and observation2, we can derive the refined two alternative sets of procedures on the table:

- Option 1: Not support further propagation of type-2/3 indication, where

  - Received type-2 indication is NOT propagated further.

  - Received type-3 indication is NOT propagated further.

  - Type-2 indication does NOT carry any information.

  - Type-3 indication does NOT carry any information.

- Option 2: Support further propagation of type-2/3 indication, where.

  - If a node receives a type-2 indication from a parent node, the node further propagates the received type-2 indication to its child node(s) if it has no alternative path available.

  - If a node receives a type-3 indication from a parent node, the node further propagates the received type-3 indication to its child node(s)

  - For further propagation, a node simply forwards the received type-2/3 indication without regenerating a type-2/3 indication to send.

  - Type-2 indication indicates unavailable routing IDs.

  - Type-3 indication indicates available (recovered) routing IDs.

Now RAN2 need to make a choice. We make the following observations:

- a) Both options work

  - No blocking scenario to preclude a particular option has been identified so far.

- Performance

  - Option 2 has the potential to provide better performance.

  - However, the actual gain largely depends on the topology of the IAB networks. That is, the claimed performance benefit is hard to override the complexity concern of option2

- Complexity and specification effort

  - If option1 is taken, then stage-2 discussion can be closed stably.

  - If option2 is taken, we need to address at least open issue 2-a and 2-b presented above. For these, if the observation#1 and observation#2 are considered acceptable, the complexity concern of option 2 becomes somehow reduced because we have solutions on the open issues.

  - However, it is not clear if there exist other corner cases introduced by option2 that are not identified until now but to be resolved for functional completeness. RAN2 has simply no time to discuss if such case exists or not. Then, non-trivial risk of hidden issues may remain.

Given the observations, we conclude that option 2 is good to have but not essential for Rel-17 and there may exist a hidden risk of unresolved issues, and option1 is the minimum but suboptimal. Given the limited time budget for Rel-17, it is reasonable to take a simpler but workable solution, which is option 1. We believe that more optimal solutions such as option2 and other enhancements can be considered later e.g., in Rel-18 feIAB WI.

Proposal 2: Further propagation of type-2/3 indication is not supported.

4. Conclusion

Proposal 1: To confirm that if a node upon detecting a BH RLF performs partial local re-routing, it does not trigger type-2 indication

Proposal 2: Further propagation of type-2/3 indication is not supported.

5. Reference

[1] R2-2201937 Report of [AT-116bis][048][eIAB] BH RLF indication (LGE) LG Electronics.