3GPP TSG-RAN WG2 Meeting #117-e                       R2-2203271

Online, Feb 21th – March 3rd, 2022

Agenda item:	8.8.2

Source:	Samsung, Qualcomm Incorporated, OPPO

Title:	Text Proposal for 38.304 on cell reselection for RAN slicing

Document for:	Discussion & Decision

# Introduction

In RAN2#116bis-e meeting, RAN2 discussed the following solution 4 options for slice-based cell reselection [1]:

- Solution 4, all NAS-prioritised slices with frequency priorities as well as legacy frequency priorities are consisdered, without iteration - Solution 4, original (UE first uses the frequency priorities of the highest priority slice, and if no cell is found, it will use the priorities of other slices in priority order, and at last it will use legacy priorities) - Solution 4, only highest prio slice considered, then legacy priorities considered.

RAN2 could not agree on any of the above options, and instead RAN2 decided to support a variation of Option A (without formula) [1]:

A (without formula, i.e. specific slice priority value used in evaluation): - Working assumption: We go with proposal A without formula, e.g. as proposed by Samsung or Apple. Exact details to be worked out for the next meeting.

In this contribution, we provided a TP against the running CR for 38.304. Additionally, we provided an accompanying discussion paper in [2].

Proposal 1: RAN2 is kindly asked to agree the proposed TP against the running CR for 38.304 (Annex A).

# References

[1] RAN2#116bis-e Report on LTE legacy, 71 GHz, DCCA, Multi-SIM and RAN slicing, Vice Chairman notes

[2] R2-2203266 Realising Prioritisation rules for option A without Formula, Samsung, Qualcomm Incorporated.

[3] R2-2200043 [Post116-e][242][Slicing] Slice-based cell re-selection algorithm (Ericsson)

[4] R2-2200044 Running 38.304 CR for RAN slicing

# Annex A (TP against the running CR for 38.304 in [4])

Start of changes

### 5.2.4	Cell Reselection evaluation process

#### 5.2.4.1	Reselection priorities handling

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). The UE may also be provided with slice Information including a list of slices or slice groups. For each slice or slice group, slice information can contain a list of frequencies supporting the slice or slice group and for each frequency an associated frequency priority for the slice or slice.

If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information, including slice or slice group specific frequency priorities. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values). If the UE is configured to perform both NR sidelink communication and V2X sidelink communication, the UE may consider the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform NR sidelink communication and not perform V2X communication, the UE may consider the frequency providing NR sidelink communication configuration to be the highest priority. If the UE is configured to perform V2X sidelink communication and not perform NR sidelink communication, the UE may consider the frequency providing V2X sidelink communication configuration to be the highest priority.

For a UE supporting slice-based cell reselection, if the UE is provided with slice priorities from NAS, and if slice or slice group specific frequency priorities are included in the cell reselection information used by the UE, the UE applies slice-based cell reselection priorities as described in clause 5.2.4.x.

NOTE 1:	The frequency only providing the anchor frequency configuration should not be prioritized for V2X service during cell reselection, as specified in TS 38.331[3].

NOTE 2:	When UE is configured to perform NR sidelink communication or V2X sidelink communication performs cell reselection, it may consider the frequencies providing the intra-carrier and inter-carrier configuration have equal priority in cell reselection.

NOTE 3:	The prioritization among the frequencies which UE considers to be the highest priority frequency is left to UE implementation.

NOTE 4:	The UE is configured to perform V2X sidelink communication or NR sidelink communication, if it has the capability and is authorized for the corresponding sidelink operation.

NOTE 5:	When UE is configured to perform both NR sidelink communication and V2X sidelink communication, but cannot find a frequency which can provide both NR sidelink communication configuration and V2X sidelink communication configuration, UE may consider the frequency providing either NR sidelink communication configuration or V2X sidelink communication configuration to be the highest priority.

NOTE 6: The UE is configured with either dedicated cell reselection priorities or slice or slice group specific frequency priorities in the RRCRelease message.

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

In case UE receives RRCRelease with deprioritisationReq, UE shall consider current frequency and stored frequencies due to the previously received RRCRelease with deprioritisationReq or all the frequencies of NR to be the lowest priority frequency (i.e. lower than any of the network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection or SNPN selection is performed on request by NAS (TS 23.122 [9]).

NOTE:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 38.133 [8] are still applicable.

The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters a different RRC state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	the UE receives an RRCRelease message with the field cellReselectionPriorities absent; or

-	a PLMN selection or SNPN selection is performed on request by NAS (TS 23.122 [9]).

NOTE 2:	Equal priorities between RATs are not supported.

The UE shall not consider any black listed cells as candidate for cell reselection.

The UE shall consider only the white listed cells, if configured, as candidates for cell reselection.

The UE in RRC_IDLE state shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e. T320 in NR and E-UTRA), if configured, at inter-RAT cell (re)selection.

NOTE 3:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

Next change

#### 5.2.4.5	NR Inter-frequency and inter-RAT Cell Reselection criteria

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority NR or EUTRAN RAT/frequency fulfils Squal > ThreshX, HighQ during a time interval TreselectionRAT

Otherwise, cell reselection to a cell on a higher priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	A cell of a higher priority RAT/ frequency fulfils Srxlev > ThreshX, HighP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

Cell reselection to a cell on an equal priority NR frequency shall be based on ranking for intra-frequency cell reselection as defined in clause 5.2.4.6.

If threshServingLowQ is broadcast in system information and more than 1 second has elapsed since the UE camped on the current serving cell, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Squal < ThreshServing, LowQ and a cell of a lower priority NR or E-UTRAN RAT/ frequency fulfils Squal > ThreshX, LowQ during a time interval TreselectionRAT.

Otherwise, cell reselection to a cell on a lower priority NR frequency or inter-RAT frequency than the serving frequency shall be performed if:

-	The serving cell fulfils Srxlev < ThreshServing, LowP and a cell of a lower priority RAT/ frequency fulfils Srxlev > ThreshX, LowP during a time interval TreselectionRAT; and

-	More than 1 second has elapsed since the UE camped on the current serving cell.

If the UE performs slice-based cell reselection and if the highest ranked cell, according to neighbouring cell information, does not support the highest priority slice supported by its frequency, the UE only considers the slices supported by the highest ranked cell among the NAS provided slice or slice group for slice specific prioritisation as in section 5.2.4.X for this frequency, until the highest ranked cell changes or NAS provides a new set of slices or slice groups.

Cell reselection to a higher priority RAT/frequency shall take precedence over a lower priority RAT/frequency if multiple cells of different priorities fulfil the cell reselection criteria.

If more than one cell meets the above criteria, the UE shall reselect a cell as follows:

-	If the highest-priority frequency is an NR frequency, the highest ranked cell among the cells on the highest priority frequency(ies) meeting the criteria according to clause 5.2.4.6;

-	If the highest-priority frequency is from another RAT, the strongest cell among the cells on the highest priority frequency(ies) meeting the criteria of that RAT.

New Clause

#### 5.2.4.X	 Slice-based cell reselection prioritisation

The UE applies slice based cell reselection prioritisation using the following rules:

1. The UE considers a frequency supporting a slice or slice group with a higher slicePriority as higher priority than any other frequency supporting only slices or slice groups with lower slicePriority.

2. If there are multiple frequencies supporting a slice or slice group with the same slicePriority, then the UE applies the sliceSpecificFrequencyPriority among such frequencies for slice-based cell reselection prioritization. The UE considers any frequency present in sliceInfo without sliceSpecificFrequencyPriority as lowest priority frequency among frequencies supporting that slice or slice group for Slice-based cell reselection prioritization.

3. The UE considers a frequency not supporting any slice or slice group for which slicePriority is available as a frequency of a priority lower than any other frequency supporting a slice or slice group for which slicePriority is available. When there are multiple frequencies not supporting any slice or slice group, the UE applies the relative priority among such frequencies as per the CellReselectionPriority of the frequency.

Note: It is up to the UE implementation how the UE realises the above rules for prioritisation.

Editor’s Note: slicePriority is the slice or slice group priority provided by NAS. sliceInfo is slice specific cell reselection information used by the UE provided in SIB or RRC release message. sliceSpecificFrequencyPriority is the absolute priority for NR frequency with respect to a slice or slice group. After their definitions are agreed, they will be captured in Section 5.2.4.7.0.

End of Changes