3GPP TSG-RAN2 Meeting #117-e	R2-2203340

# Online, February 21 – March 3, 2022

Agenda item:	8.18.2

Source:	Huawei, HiSilicon

Title:	Further details of RACH procedure with RACH partitioning

Document for:	Discussion

# Introduction

In Rel-17, several features (i.e. SDT, CovEnh, RedCap and RAN slicing) saw the need of RACH partitioning to enable early identification of the feature. Each of the concerned WIs has made some progress.

In this contribution we discuss the remaining issues of RACH procedure in MAC considering the common framework for all involved features.

# Discussion

## 2.1 Fallback rules

At RAN2 #115-e meeting, it is agreed that all RACH retransmissions shall be performed over the same RACH resources and the same carrier as the one selected for initial RACH resource until RACH failure happens.

6. As a baseline, the RA procedure design for Rel-17 should adhere to the following general principles: c: As a general rule, all RACH retransmissions (if any are needed, until RACH failure happens) shall be performed over the same RACH resources (and same carrier – NUL/SUL) as the one selected for initial RACH resource. However, we can discuss fallback on a case by case basis if there is a strong motivation and discuss them together in this AI.

However, it should be noted that there were some agreements in WI-specific discussions which go in another direction. In particular, RAN2 agreed the following for slicing and SDT:

- The following fallback case is supported: - Fallback case 2: Fallback from 2-step slice specific RACH to 4-step common RACH, if 4-step slice specific RACH is not configured. - The following fallback cases are not supported in this release: - Fallback case 1: Fallback from 4-step slice specific RACH to 4-step common RACH - Fallback case 3: Fallback from 2-step slice specific RACH to 2-step common RACH, if neither 4-step slice specific RACH nor 4-step common RACH is configured. - As legacy, UE can be configured to switch from 2-step RA-SDT to 4-step RA-SDT after N times of MsgA transmission

When it comes to the third agreement copied above, this is how legacy RACH procedure works as well and we think this agreement can be extended to all feature combinations in general:

Proposal 1: UE can be configured to switch from 2-step feature (combination) specific RA to 4-step feature (combination) specific RA of the same feature (combination).

However, the first agreement related to slicing is not that straightforward to extend to other features. For example, falling back from 2-step RA-SDT to 4-step legacy RACH would require TB rebuilding and hence is non-preferred. For RedCap, if a RedCap UE falls back to legacy RACH resources when Msg1 RedCap indication is configured in the network, then RACH procedure will likely fail as the network will not be able to deliver RAR in a RedCap specific way (e.g. with narrower bandwidth). Similarly, falling back from CovEnh specific RACH to legacy RACH would not solve the coverage problem nor increase the chances of successful RACH for the UE and hence the following agreement is reached in CovEnh WI:

From CE perspective, if CE RA is selected, then the decision doesn’t change during the entire RACH procedure (i.e. until RACH failure).

Since it has been agreed that for slicing, unified partitioning framework should take priority, and there is no clear motivation for such fallback case for any of other features, we propose to agree the following:

Proposal 2: Fallback from 2-step feature (combination) specific RACH to 4-step common RACH is not supported.

## 2.2 Feature/Feature Combination Selection

In current 38.321 running CR [1] for SDT, RRC layer requests MAC to check the conditions for SDT after upper layers requests the resume of a suspended RRC connection. If the conditions for SDT are met MAC indicates this to RRC. At the same time, MAC directly initiates the RA-SDT procedure if condition for RA-SDT is satisfied as highlighted below, without any further indication from RRC layer.

1> if the data volume of the pending UL data across all RBs configured for SDT is less or equal to sdt-DataVolumeThreshold; and NOTE: For SDT procedure, the MAC entity also consideres the suspended RBs configured with SDT for data volume calculation. It is up to the UE’s implementation how the UE calculates the data volume for the suspended RBs. Size of the CCCH message is not considered for data volume calculation 1> if the RSRP of the downlink pathloss reference is higher than sdt-RSRP-Threshold: 2> if the Serving Cell for SDT is configured with supplementary uplink as specified in TS 38.331 [5]; and 2> if the RSRP of the downlink pathloss reference is less than sdt-RSRP-ThresholdSSB-SUL: 3> select the SUL carrier. 2> else: 3> select the NUL carrier. Omitted... 2> else if RA-SDT is configured on the selected UL carrier: 3> indicate to the upper layers that the conditions for initiating SDT are fulfilled; 3> select RA-SDT on the selected UL carrier according to clause 5.1 for SDT.

Upon receiving the indication from MAC layer that SDT conditions are met, RRC resumes SDT RBs and starts SDT failure detection timer. However, considering that multiple RACH partitions for different features/feature combinations can be configured in SIB1, the UE will have to first check the availability of a RACH partition for its preferred feature combination and if such partition is not available, the UE will use RACH partition/feature priority rules to select another RACH partition. It is then possible that MAC will select a RACH partition which does not include SDT based on the signaled feature priorities. In this case, the resumption of SDT RBs by RRC layer is unnecessary resumed SDT RBs. Similarly, the UE would wrongly start SDT failure detection timer instead of T319 in this case. This has bad impact on UE’s power consumption and would impact the time it would take the UE to determine the resume failure and perform another access attempt.

To avoid this issue, the first point that has to be clarified is which layer is responsible for the feature/feature combination selection, i.e. either RRC or MAC:

- Feature/feature combination selection happens in RRC layer: Upon data arrival, UE would determine its preferred feature/feature combination based on the features enabled in the cell. Then the condition for each feature would be checked, potentially with some inter-layer interactions, e.g., MAC layer is responsible to check the SDT condition including data volume and RSRP. Finally, RRC layer would decide the selected feature/feature combination and indicate to MAC layer the selected feature/feature combination, according to which the MAC layer initializes RACH parameters.

- Feature/feature combination selection happens in MAC layer: Upon data arrival, RRC and MAC will check the conditions for features, e.g., the condition for SDT. Then MAC could perform the feature/feature combination selection during the RA procedure according to the satisfied conditions and the RACH resource for feature/feature combination configured within the cell.

Proposal 3: RAN2 to discuss whether it is MAC layer or RRC layer to perform the feature/feature combination selection.

Based on the above assumption, to avoid that UE unnecessarily resumes SDT RBs and starts SDT failure detection timer instead of T319, the following approaches can be considered:

- If it is RRC layer to perform the feature/feature combination selection, then instead of initiating the SDT procedure directly when SDT conditions are met, MAC layer should first indicate to RRC that the conditions for initiating SDT are fulfilled and only initiate the SDT procedure after the feature/feature selection including SDT is done by RRC layer (which will be indicated by RRC layer based on the selected feature combination). In conclusion, UE only resumes SDT RBs and starts SDT failure detection timer after SDT conditions are met and the feature combination includes SDT.

- If it is MAC layer to perform the feature/feature selection, there can further be two approaches:

  - MAC layer should “indicate to the upper layers that the conditions for initiating SDT are fulfilled”. However, the SDT procedure should only be initiated after the feature/feature combination selection including SDT is done by MAC layer. In this approach, if MAC selects a feature/feature combination without SDT after MAC indicated to RRC that the conditions for initiating SDT were fulfilled, MAC should indicate to RRC layer that feature/feature combination does not include SDT, so that RRC layer would cancel the triggered SDT related actions (e.g. suspend SDT RBs, stop SDT failure timer and start T319 instead).

  - In another approach, MAC layer performs feature/feature combination selection after checking the SDT conditions. Then an indication is sent from MAC layer to RRC layer which includes the selected feature/feature combination and information for whether SDT conditions are satisfied or not can also be included at this step (explicitly or implicitly). In this case, SDT procedure is only initiated when the selected feature/feature combination includes SDT and the issues addressed above can be avoided.

Proposal 4: To avoid that SDT procedure is initiated while the feature/feature combination does not include SDT, RAN2 should consider updating the SDT CRs according to one of the following options:

- Option 1: SDT is only initiated when the selected feature/feature combination includes SDT;

- Option 2: MAC layer indicates to RRC layer that SDT procedure should be cancelled when the selected feature/feature combination does not include SDT.

# Conclusion

Based on the above discussion, we make the following observations and recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: UE can be configured to switch from 2-step feature (combination) specific RA to 4-step feature (combination) specific RA of the same feature (combination).

Proposal 2: Fallback from 2-step feature (combination) specific RACH to 4-step common RACH is not supported.

Proposal 3: RAN2 to discuss whether it is MAC layer or RRC layer to perform the feature/feature combination selection.

Proposal 4: To avoid that SDT procedure is initiated while the feature/feature combination does not include SDT, RAN2 should consider updating the SDT CRs according to one of the following options:

- Option 1: SDT is only initiated when the selected feature/feature combination includes SDT;

- Option 2: MAC layer indicates to RRC layer that SDT procedure should be cancelled when the selected feature/feature combination does not include SDT.

# Reference

- R2-2202041 Running MAC CR for Small Data, Huawei, HiSilicon