3GPP TSG-RAN WG2 #117e	Tdoc R2-2203392

Electronic meeting, February 21 – March 03, 2022

Agenda Item:	8.2.5

Source:	Ericsson

Title:	UE capabilities for Rel-17 MR-DC enhancements

Document for:	Discussion, Decision

# 1	Introduction

The discussion for UE capabilities in [1] contains the following UE feature list as baseline:

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Activation/Deactivation of SCG | Support of activation/deactivation of SCG. |  | No | Yes | Per UE | Optional with capability signalling |
| x-2 | Activation/Deactivation of SCG | RACH-less SCG activation. | FFS | No | Yes | Per UE | Optional with capability signalling |
| x-3 | CPAC | CPA for NR-DC |  | No | No | Per UE | Optional with capability signalling |
| x-4 | CPAC | CPA for (NG)EN-DC |  | No | No | Per UE | Optional with capability signalling |
| x-5 | CPAC | MN initiated CPC in NR-DC | FFS | No | No | Per UE | Optional with capability signalling |
| x-6 | CPAC | MN initiated CPC in (NG)EN-DC | FFS | No | No | Per UE | Optional with capability signalling |

This contribution discusses the remaining aspects related to UE capabilities for Rel-17 MR-DC enhancements.

# 2	Discussion

## 2.1	Activation / deactivation of SCG and SCells

As depicted in the above, the baseline for UE feature list contains RACH-less SCG activation (x-2) as an optional feature. However, RACH-less SCG activation can speed up the overall SCG activation, and it can be a main benefit of the use of this feature compared to SCG addition procedure, hence, this should be part of the basic support of Activation/Deactivation of SCG (x-1).

RACH-less SCG activation is part of the basic support of the feature for Activation/Deactivation of SCG.

The RACH-less SCG activation also depends on Rel-15 RLM/BFD capabilities, since the UE should keep performing such activities in order to perform RACH-less SCG activation. Note this does not impose any new pre-requisites on the Rel-15 capabilities, it just implies that a UE indicating support of Activation/Deactivation of SCG for a given FR should also indicate support of the Rel-15 RLM/BFD capabilities for the given FR. The network should then accordingly configure the UE with RLM/BFD on the deactivated SCG to use the RACH-less SCG activation, this is further discussed in [3].

For both FR1 and FR2, the UE support of Activation/Deactivation of SCG depends on the UE support of Rel-15 RLM/BFD capabilities.

Another aspect to be considered is whether to introduce separate capabilities for activation/deactivation of SCG in RRC Resume and RRC Reconfiguration cases. It has been raised in RAN2#116 that this discussion would be similar to the discussion held on SCell activation in Rel-16 (where it was decided to have separate UE capabilities for RRC Resume and RRC Reconfiguration), and thus we should do the same for SCG activation/deactivation. It should be noted, however, that for SCell activation upon resume (compared to SCell activation upon RRC reconfiguration), there is a clear new feature being triggered i.e. the UE must be able to activate SCells upon receiving an RRCResume message. However, for SCG case, from the legacy Rel-16 behaviour, the UE should already be able to have the SCG activated upon resuming, i.e. if it indicates support for resumeWithStoredSCG-r16.Note that the term “activate SCG” only makes sense if the SCG was previously deactivated, otherwise, an activated SCG is essentially just a regular SCG. Therefore, we cannot follow the same approach as done for SCell activation, considering that the UE should already be able to support SCG activation upon resuming. We could at most cover with a new feature on resume “the UE support for keeping the SCG deactivated upon resuming”; but the activation is already given by resumeWithStoredSCG-r16. Hence, it is simpler to just follow one capability for RRC configuration and RRC resume in general. The support for resume would further depend on whether the UE reported the Rel-16 capabilities for SCG resume.

If the UE indicates support of SCG activation/deactivation, it implies support of the feature for both RRC Resume (if the according Rel-16 capabilities for SCG resume are supported) and RRC Reconfiguration.

Another aspect is that currently UE capabilities for Activation/Deactivation are marked with FR1/FR2 differentiation only in the baseline UE feature list, i.e. no TDD/FDD differentiation. But according to the agreements from RAN2#116bis-e, those should be captured with both R1/FR2 and TDD/FDD differentiation.

From Rel-17 onwards, at least for new capabilities, if a UE capability requires at least FRx or at least xDD differentiation, it is defined with both FRx and xDD differentiation in per band signaling, i.e. no new UE capabilities will be defined in the FRX and XDD capability signaling branches.

The according changes to capture the RAN2 agreement and the proposals above are contained in the Appendixes.

## 2.2	Conditional PSCell change / addition

The baseline UE feature list in [1] contains the following UE capabilities for CPAC:

| x-3 | CPAC | CPA for NR-DC |  | No | No | Per UE | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-4 | CPAC | CPA for (NG)EN-DC |  | No | No | Per UE | Optional with capability signalling |
| x-5 | CPAC | MN initiated CPC in NR-DC | FFS | No | No | Per UE | Optional with capability signalling |
| x-6 | CPAC | MN initiated CPC in (NG)EN-DC | FFS | No | No | Per UE | Optional with capability signalling |

It should be noted that a capability x-3 and x-4 above should be included in different UE capability containers (i.e. x-3 should be included in UE-NR-Capability to indicate to the gNB the UE support for CPA for NR-DC, while x-4 should be included in UE-MRDC-Capability to indicate to the eNB the UE support for CPA for (NG)EN-DC). Hence, it makes sense to have two different capabilities for this feature, which can be confirmed in RAN2. There are also differences in the signalling procedures for NR-DC and (NG)EN-DC for the feature, including e.g. that the configurations are sent in MN format that is then either according to 38.331/NR or to 36.331/E-UTRA. This should thus be covered by different capabilities. The same is applicable for the features MN-initiated CPC (x-5 and x-6 above) and for SN-initiated CPC.

The UE can indicate support of the features CPA, MN-initiated CPC and SN-initiated CPC, respectively, separately for NR-DC and (NG)EN-DC.

The UE support for SN-initiated CPC for NR-DC and (NG)EN-DC, respectively, is missing in the baseline feature list in [1] and corresponding capabilities should thus be introduced, together with the capabilities for CPA and MN-initiated CPC that are already included in the list.

For Rel-16 CPC there are also additional UE capabilities for:

- support of two trigger events (condPSCellChangeTwoTriggerEvents-r16)

- support for CPC between FDD and TDD cells (condPSCellChangeFDD-TDD-r16)

- support for CPC between FR1 and FR2 (condPSCellChangeFR1-FR2-r16).

The condPSCellChangeTwoTriggerEvents-r16 is then mandatory supported if the UE supports condPSCellChange-r16 so the UE thus needs to support the two trigger events if it supports the feature. We therefore think that the two trigger events should be considered as part of the basic support of the corresponding feature and that there is no need for a separate capability indication for this.

Support for two trigger events is part of the basic support of the Rel-17 features CPA, MN-initiated CPC and SN-initiated CPC.

The capabilities for support of CPC between FDD and TDD and FR1 and FR2, respectively, are relevant for the Rel-17 CPC features and should be included as separate capabilities. These capabilities should then be common for the Rel-17 SN-initiated CPC and Rel-17 MN-initiated CPC features.

We thus have the following proposal for CPAC related UE capabilities.

The following separate UE capabilities for CPAC should be introduced:
- SN-initiated CPC for NR-DC
- SN-initiated CPC for (NG)EN-DC
- CPA for NR-DC
- CPA for (NG)EN-DC
- MN-initiated CPC for NR-DC
- MN-initiated CPC for (NG)EN-DC
- Inter-SN PSCell change between FDD and TDD (for NR-DC and (NG)EN-DC)
- Inter-SN PSCell change between FR1 and FR2 (for NR-DC and (NG)EN-DC)

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	RACH-less SCG activation is part of the basic support of the feature for Activation/Deactivation of SCG.

Proposal 2	For both FR1 and FR2, the UE support of Activation/Deactivation of SCG depends on the UE support of Rel-15 RLM/BFD capabilities.

Proposal 3	If the UE indicates support of SCG activation/deactivation, it implies support of the feature for both RRC Resume (if the according Rel-16 capabilities for SCG resume are supported) and RRC Reconfiguration.

Proposal 4	The UE can indicate support of the features CPA, MN-initiated CPC and SN-initiated CPC, respectively, separately for NR-DC and (NG)EN-DC.

Proposal 5	Support for two trigger events is part of the basic support of the Rel-17 features CPA, MN-initiated CPC and SN-initiated CPC.

Proposal 6	The following separate UE capabilities for CPAC should be introduced: - SN-initiated CPC for NR-DC - SN-initiated CPC for (NG)EN-DC - CPA for NR-DC - CPA for (NG)EN-DC - MN-initiated CPC for NR-DC - MN-initiated CPC for (NG)EN-DC - Inter-SN PSCell change between FDD and TDD (for NR-DC and (NG)EN-DC) - Inter-SN PSCell change between FR1 and FR2 (for NR-DC and (NG)EN-DC)

# References

R2-2109676, Report of email discussion [Post115-e][214][R17 DCCA] Capabilities (Intel)

R2-2201096, UE capabilities, Huawei, HiSilicon, 3GPP TSG-RAN WG2 Meeting #116bis-e, Electronic meeting, Jan 14th – 25th, 2022

R2-2203391, Actions at SCG activation and deactivation, Ericsson, 3GPP TSG-RAN WG2 Meeting #117-e, Electronic meeting, February 21th – March 3rd, 2022

# Appendix A: Changes related to UE features list in R2-2109676

| Index | Feature group | Components | Prerequisite feature groups | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x-1 | Activation/Deactivation of SCG | Support of activation/deactivation of SCG (including RACH-less SCG activation). |  | Yes | Yes | Per UE | Optional with capability signalling |
| x-2 | CPAC | CPA for NR-DC |  | No | No | Per UE | Optional with capability signalling |
| x-3 | CPAC | CPA for (NG)EN-DC |  | No | No | Per UE | Optional with capability signalling |
| x-4 | CPAC | MN initiated CPC in NR-DC |  | No | No | Per UE | Optional with capability signalling |
| x-5 | CPAC | MN initiated CPC in (NG)EN-DC |  | No | No | Per UE | Optional with capability signalling |
| x-6 | CPAC | SN initiated CPC in NR-DC |  | No | No | Per UE | Optional with capability signalling |
| x-7 | CPAC | SN initiated CPC in (NG)EN-DC |  | No | No | Per UE | Optional with capability signalling |
| x-8 | CPAC | Inter-SN PSCell change between FDD and TDD for NR-DC | x-5, or x-7 | No | No | Per UE | Optional with capability signalling |
| x-9 | CPAC | Inter-SN PSCell change between FR1 and FR2 for NR-DC | x-5, or x-7 | No | No | Per UE | Optional with capability signalling |
| x-10 | CPAC | Inter-SN PSCell change between FDD and TDD for EN-DC | x-6 or x-8 | No | No | Per UE | Optional with capability signalling |
| x-11 | CPAC | Inter-SN PSCell change between FR1 and FR2 for EN-DC | x-6 or x-8 | No | No | Per UE | Optional with capability signalling |