3GPP TSG-RAN WG2 Meeting #117-electronic	   R2-2203393

Online, February 21st - March 3rd, 2022                                       Revision of R2-2201597

Agenda item:		8.18.1

Source:	vivo

Title:	Further Discussion on RACH Partitioning in RA Configuration Aspect

Document for:		Discussion and Decision

1 Introduction

In the previous RAN2 meeting, the following agreements regarding RA configuration were made.

RAN2#116bis-e meeting: => FFS if we remove the FeatureCombination from RACH common config and only keep 2) => confirmed the above cases as a baseline => RAN2 submits one RRC CR to plenary that captures the RA partitioning feature that covers all common aspects for RA partitioning. The RRC CRs for RedCap, SDT, Coverage enhancements, and Slicing should not have any overlap with this common RRC CR. => Agree to use X-Y solution

In this contribution, we would like to further provide our considerations on RRC signaling configuration design for the RACH partitioning.

2 Discussion

According to the output of [2], an illustration of the ASN.1 structure of Rel-17 RACH partitioning can be generally shown as below,

Figure 1: Configuration structure for Rel-17 RACH partitioning

In our understanding, the current structure is in good shape as it allows the flexibility to configure a RA partitioning via either legacy RACH-ConfigCommon (for shared RO case) or RACH-ConfigCommon-r17 (for separate RO case). No technical drawbacks and issues are found. So we propose:

Proposal 1: RAN2 to confirm that the FeatureCombination is kept in RACH common config.

Furthermore, if all the features are deployed based on the 4-step RA type, the current structure looks perfect. However, in our understanding, the current struct is not so overhead-friendly if the features are deployed based on a 2-step RA type.

For example, assuming both R-15 4-step RA type and Rel-16 2-step RA type are configured in shard RO case. Then [Rel-17 4-step RA type and Rel-17 2-step RA type for feature 1] with [Rel-17 4-step RA type and Rel-17 2-step RA type for feature 2] need to be additionally configured in the same RO. Even though only 1 RO set is used in this case, however, there are 2 entries in the Rel-17 rach-configCommon List. These two entries generally are used for configuring MsgA PUSCH configuration as the current IE MsgA-configCommon cannot be extended (while the legacy IE RACH-ConfigCommon can). If MsgA-configCommon can be extended, then we think there is no need to rach-configCommon List as we only need 1 RO set in this case.

-- ASN1START

-- TAG-MSGACONFIGCOMMON-START

MsgA-ConfigCommon-r16 ::=           SEQUENCE {

rach-ConfigCommonTwoStepRA-r16      RACH-ConfigCommonTwoStepRA-r16,

msgA-PUSCH-Config-r16               MsgA-PUSCH-Config-r16                                      OPTIONAL --Cond InitialBWPConfig

}

-- TAG-MSGACONFIGCOMMON-STOP

-- ASN1STOP

To get rid of this limitation, we think in Rel-17 it is necessary to introduce an extendable MsgA-ConfigCommon, such as

-- ASN1START

-- TAG-MSGACONFIGCOMMON-START

MsgA-ConfigCommon-r17 ::=           SEQUENCE {

rach-ConfigCommonTwoStepRA-r17      RACH-ConfigCommonTwoStepRA-r16,     OPTIONAL, -- Need R

msgA-PUSCH-Config-r17               MsgA-PUSCH-Config-r16                                      OPTIONAL, -- Need R

featureCombinationMsgA-PUSCH-r17		SEQUENCE (SIZE(1..FFS)) OF MsgA-PUSCH-Config-r16			OPTIONAL			-- Need R

}

-- TAG-MSGACONFIGCOMMON-STOP

-- ASN1STOP

With this modification, only one entry was used in the Rel-17 rach-configCommon List to configure the case mentioned, which seems to be simpler. Thus, we propose

Proposal 2: Introduce a new extendable IE MsgA-ConfigCommon-r17 for Rel-17 RACH partitioning.

Figure 2: Proposed configuration structure for RACH partitioning

Next, we would like to focus on the RACH common configuration for cross WIs when ROs are shared with preamble partitioning. As usual, we would like to first summarize what parameters can be commonly used for both 4-step RACH and 2-step RACH in the case of shared RO. Specifically,

Common PHY-aspect parameters when ROs are shared:

prach-ConfigurationIndex, msg1-FDM, and msg1-FrequencyStart for RO configuration;

ssb-perRACH-Occasion for the association between RO and SSB;

msg1-SubcarrierSpacing, restrictedSetConfig, prach-RootSequenceIndex, zeroCorrelationZoneConfig for preamble sequence generation.

preambleReceivedTargetPower and powerRampingStep for preamble power control.

Common MAC-aspect parametes:

rsrp-ThresholdSSB-SUL for UL carrier selection;

ra-ContentionResolutionTimer for contention resolution reception

preambleTransMax for declaring a RA failure;

Then, for the common configuration for multiple Rel-17 features, we generally think the design logic for 2-step RACH can be re-used as the baseline. For example, the above-mentioned PHY-aspect parameters can be commonly configured for those Rel-17 features in the case where shared ROs but preamble partitioning is used. In our understanding, applying this principle helps to relieve potential normative work impacts to R1 WG since no TU is allocated for RAN slicing WI. For the MAC-aspect parameters, we think rsrp-ThresholdSSB-SUL, ra-ContentionResolutionTimer, and preambleTransMax can be commonly configured as well since we cannot observe the additional performance gain brought by separate configuration, taking into account that the fallback back mechanism is supported for RAN slicing and SDT.

Proposal 3: The following PHY-aspects parametrs can be commonly configured for RAN slicing, SDT, RedCap, and CovEnh WIs: prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, ssb-perRACH-Occasion, msg1-SubcarrierSpacing, restrictedSetConfig, prach-RootSequenceIndex, zeroCorrelationZoneConfig, preambleReceivedTargetPower, and powerRampingStep.

Proposal 4: The following MAC-aspects parameters can be commonly configured for RAN slicing, SDT, RedCap, and CovEnh WIs: rsrp-ThresholdSSB-SUL, ra-ContentionResolutionTimer, and preambleTransMax.

3 Conclusion

The contribution is summarized as follows,

Proposal 1: RAN2 to confirm that the FeatureCombination is kept in RACH common config.

Proposal 2: Introduce a new extendable IE MsgA-ConfigCommon-r17 for Rel-17 RACH partitioning.

Proposal 3: The following PHY-aspects parametrs can be commonly configured for RAN slicing, SDT, RedCap, and CovEnh WIs: prach-ConfigurationIndex, msg1-FDM, msg1-FrequencyStart, ssb-perRACH-Occasion, msg1-SubcarrierSpacing, restrictedSetConfig, prach-RootSequenceIndex, zeroCorrelationZoneConfig, preambleReceivedTargetPower, and powerRampingStep.

Proposal 4: The following MAC-aspects parameters can be commonly configured for RAN slicing, SDT, RedCap, and CovEnh WIs: rsrp-ThresholdSSB-SUL, ra-ContentionResolutionTimer, and preambleTransMax.

4 References

[1] R2-2201664, Report for Rel-17 Small data, URLLC/IIoT and RACH partitioning.

[2] R2-2202045, Running CR to 38.331 on RA Partitioning.

5 Annex: Agreements

Rel-17 small data transmission:

      - RAN2#112-e meeting agreements:

- As a baseline, the RACH resource i.e. (RO+preamble combination) is different between SDT and non-SDT

  - If ROs for SDT and non SDT are different, preamble partitioning between SDT and non SDT is not needed.

  - If ROs for SDT and non SDT are same, preamble partitioning is needed

- RAN2 continues to progress the work based the separate RACH resources for SDT (i.e. explicit mechanisms to support common resources won’t be pursued unless there is sufficient support for this. However, use of common RACH resources will not be precluded if possible via implementation

- For RA-SDT, up to two preamble groups (corresponding to two different payload sizes for MSGA/MSG3) may be configured by the network.

      - RAN2#115-e meeting agreements:

- At least the following parameters can be RA-SDT specific.

  - SSB selection related parameters, i.e., rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB.

  - Power control related parameters, i.e., preambleReceivedTargetPower/gA-PreambleReceivedTargetPower, powerRampingStep/msgA-PreamblePowerRampingStep,  msg3-DeltaPreamble/msgA-DeltaPreamble.

  - Preamble group related parameters, i.e., msg3-DeltaPreamble/msgA-DeltaPreamble, messagePowerOffsetGroupB for 2-step RA-SDT and 4-step RA-SDT.

- For shared ROs case, all the following configurations can be allowed:

  - 4-step RA-SDT shares ROs with 4-step RA and/or 2-step RA

  - 2-step RA-SDT shares ROs with 4-step RA and/or 2-step RA

  - 2-step RA-SDT shares ROs with 4-step RA-SDT and/or 4-step RA and/or 2-step RA.

- RA-SDT can be configured on initial BWP.  FFS for non-initial BWP.

- RA prioritization related parameters cannot be configured for RA-SDT, i.e., powerRampingStepHighPriority, scalingFactorBI

      - RAN1#106-e meeting agreements:

- RA-SDT resource cannot be configured on non-initial BWP.

      - RAN1#106bis-e meeting agreements:

- For RA-SDT power control parameters preambleReceivedTargetPower and powerRampingStep:

  - For separate ROs, the power control parameters can be RA-SDT specific

      - RAN1#107-e meeting agreements:

- RAN2 confirms that SDT will be configured only on initial BWP and there is no L1 ACK feedback for CG-SDT. ASK RAN1 to confirm whether it is different from initial BWP and that there is no conflict with the agreement

Rel-17 RAN Slicing,

      - RAN2-113-e meeting agreements:

- Separated PRACH configuration (e.g. transmission occasions of time-frequency domain and preambles) can be configured for slice or slice group. (Agreement 1 above does not imply RAN1 impacts). FFS how many slice groups we can have and how they are indicated.

- Existing RACH parameters prioritization (i.e. scalingFactorBI and powerRampingStepHighPriority ) can be supported as baseline for slices.

- Solution 1 (RACH isolation) & 2 (RACH prioritization) can work independently in a complementary way.

- Both solution 1 and solution 2 for slice-based RACH configuration are recommended for normative work.

      - RAN2-113bis-e meeting agreements:

- RAN2 aims to support both RO partition and preambles partition.

- Slice based RACH configuration can be applied to idle/inactive UE (only applied for CBRA but not for CFRA). And CONNECTED mode is down prioritized and can be considered if time allows.

- Network can configure slices with 4-step or 2-step (or both) RA resources.

- Legacy 2-step RA fallback mechanism is supported.

      - RAN2-114-e meeting agreements:

- RAN2 confirm for a slice group, separated RO and/or separate preamble can be configured within the existing RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA.

- Same as NR Rel-15 conclusion, RAN2 conclude that there is no RA-RNTI collision between slice specific RACH and legacy RACH in shared RO.

- Same as NR Rel-15 conclusion, RAN2 conclude that the RA-RNTI collision between slice specific RACH and legacy RACH may happen in separate RO.

- Working assumption: this can be left to network implementation to resolve it (e.g. network configure RO in different time).

- FFS how many slice groups we can have and how they are indicated.

      - RAN2#115-e meeting agreements:

- Network based solution is introduced to resolve the issue of prioritization parameter collision with MPS/MCS, i.e., Network indicates whether slice override MPS or MPS override slice.

- For slice based RACH prioritization, RAN2 will stick to the current baseline parameters, i.e., scalingFactorBI and powerRampingStepHighPriority, and no additional parameters for this release.

- Reuse the legacy threshold for the selection between 2-step and 4-step slice initiated RACH.

- A new slice grouping mechanism is introduced for RACH configuration. One slice belongs to one and only one slice group. Slice groups are assumed to be only updated when UE does Registration Update.

- 6	For RACH type selection, UE first selects between slice-specific and common RACH, then selects between 2-step and 4-step.

- 9 The following fallback case is supported:

  - Fallback case 2: Fallback from 2-step slice specific RACH to 4-step common RACH, if 4-step slice specific RACH is not configured.

- 10 The following fallback cases are not supported in this release:

  - Fallback case 1: Fallback from 4-step slice specific RACH to 4-step common RACH.

  - Fallback case 3: Fallback from 2-step slice specific RACH to 2-step common RACH, if neither 4-step slice specific RACH nor 4-step common RACH is configured.

- 6, 9, 10 will be aligned to the common RACH partitioning discussion decisions

      - RAN2#116-e meeting agreements:

- In a cell, there may be multiple slice-specific RACH configurations.

- One or more of the slice groups are linked to a slice-specific RACH configuration.

- There may be slice groups that are not linked to a slice-specific RACH configuration (they use the common RACH configuration).

- All slices of a slice group use the slice-specific RACH configuration of the slice group.

- RACH prioritization parameters can be configured per slice group.

Reduced capability,

      - RAN1#105-e meeting Working assumption:

- For 4-step RACH, support the early indication of RedCap UEs at least in Msg

  - The early indication in Msg1 can be configured to be enabled/disabled (e.g. via SIB).

    - FFS details (e.g. separate initial UL BWP, separate PRACH resource, PRACH preamble partitioning).

  - FFS details e.g.

    - separate initial UL BWP

    - separate PRACH resource

    - PRACH preamble partitioning

  - FFS the possibility of supporting Msg3 for the early indication

- if the above working assumption is confirmed, early indication of RedCap UEs in Msg1 can be enabled/disabled via SIB.

      - RAN1#106-e meeting agreement:

- Confirm the following working assumption with the modifications in red:

For 4-step RACH, support the early indication of RedCap UEs at least in Msg1.

  - The early indication in Msg1 can be configured to be enabled/disabled via SIB

    - FFS how to support enable/disable the early indication

  - FFS details e.g. From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP (if supported)

      - separate PRACH resource

      - PRACH preamble partitioning

      - FFS: whether/how to address RA-RNTI overlapping issue

  - FFS the possibility of supporting Msg3 for the early indication

- Conclusion

Whether there is RA-RNTI overlapping issue and how to address RA-RNTI overlapping issue in the early indication of RedCap UEs in Msg1 in Rel-17 is up to RAN2.

      - RAN2#115-e meeting agreements:

- Msg1 identification which can be configured to be enabled/disabled can be specified from RAN2 point of view.

- Solution for early identification for 2-step RACH will be specified.

      - RAN1#106bis-e meeting agreement:

- Conclusion: It is up to RAN2 for PRACH preamble partitioning for Msg1-based early indication.

      - RAN2#116-e meeting agreement:

- In MAC perspective, a RedCap UE uses Msg1 early identification whenever transmitting preamble for CBRA, as long as the Msg1 early identification is configured for RedCap by NW.

- For Msg1 early identification, RAN2 confirm both dedicated ROs and dedicated PRACH preamble can be supported from signalling point of view.

- For RedCap, Msg1 early identification is enabled/disabled implicitly by the presence of dedicate RACH configuration for Msg1 early identification.

      - RAN1#107-e meeting agreement:

- In Rel-17, up to 1 separate initial UL BWP for RedCap can be configured.

- For 2-step RACH, support the early indication of RedCap UEs at least in MsgA PRACH.

  - The early indication in MsgA PRACH can be configured to be enabled/disabled via SIB.

  - From RAN1 perspective, the following methods can be used for early indication both for shared initial UL BWP and separate initial UL BWP

    - separate MsgA PRACH resource

    - MsgA PRACH preamble partitioning

      - RAN2#116bis-e meeting agreement:

- If a RedCap-specific initial UL BWP is configured for RACH, RedCap UEs shall use only the RedCap-specific initial UL BWP to perform RACH.

- If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, measurements are based on CD-SSB for initial RACH resource selection.

- If a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, PDCCH-ConfigCommon of the separate initial DL BWP includes common search space configuration for RAR.

- From RAN2 perspective, if a RedCap UE in idle/inactive mode is configured with a separate initial BWP associated with no SSB (CD or NCD) for RACH, it is up to UE implementation to perform new RSRP measurement in a DL BWP associated with CD-SSB before Msg1/A retransmission.

- RedCap-specific two-step RACH, if configured, and four-step RACH are always configured in the same BWP.

- In MAC perspective, a RedCap UE uses MsgA PRACH early identification when it transmits preamble for CBRA if MsgA PRACH early identification is configured for RedCap by NW.

- For MsgA PRACH early identification, RAN2 confirms both dedicated ROs and dedicated PRACH preamble can be supported from signalling point of view.

- For RedCap, MsgA PRACH early identification is enabled/disabled implicitly by the presence of dedicated RACH configuration for MsgA PRACH early identification.

Coverage Enhancement:

      - RAN1#104b-e meeting:

- For UE requested Msg3 PUSCH repetition with gNB indicating the number of repetitions, a UE can request Msg3 PUSCH repetition via separate PRACH resources (FFS details, e.g., separate PRACH occasion or separate PRACH preamble in case of shared PRACH occasions after SSB association, etc.)

- Whether a UE would request is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

      - RAN1#105-e meeting:

- A UE requests Msg3 PUSCH repetition at least when the RSRP of the downlink pathloss reference is lower than an RSRP threshold. Whether a UE would request is based on some conditions, e.g., measured SS-RSRP threshold, which may or may not have spec impact.

- For requesting Msg3 PUSCH repetition, support the following:

- Use separate preamble with shared RO configured by the same PRACH configuration index with legacy UEs.

- FFS whether to introduce a PRACH mask to indicate a sub-set of ROs associated with a same SSB index within an SSB-RO mapping cycle for requesting Msg3 repetition for a UE.

- FFS definition of shared RO (e.g., whether the shared RO can be an RO with preamble(s) for 4-step RACH only or with preambles for both 4-step RACH and 2-step RACH).

- FFS whether or not to additionally support one (& only one) more option.

- E.g. option 2: Use separate RO configured by a separate PRACH configuration index from legacy UEs

- E.g. option 3: Use separate RO, which indlude

  - the separate RO configured by a separate RACH configuration index from legacy UE, and

  - the remaining RO (if any) configured, by the same PRACH configuration index with legacy UEs, that cannot be used by legacy rules for PRACH transmission).

      - RAN2#115-e meeting agreements:

- RAN2 should focus on Msg3 repetition for 4-step RACH, unless RAN1 makes solid conclusion to support Msg3 repetition for fallbackRAR.

- A separate RSRP threshold is introduced for requesting Msg3 repetition.

- Extension of ra-ResponseWindow and ra-ContentionResolutionTimer are not needed for Msg3 repetition.

- Send an LS to RAN1, saying that support of msg3 repetition on both NUL and SUL is feasible from RAN2 point of view and asking Q1 and Q2 to RAN1. In the LS also indicate that RAN2 thinks that preamble Group B with Msg3 repetition is feasible and ask RAN1 for confirmation.

      - RAN2#116-e meeting agreements:

- Confirm Msg3 repetition is supported on both NUL and SUL, and network can configure different RSRP thresholds for requesting Msg3 repetition on NUL and SUL.

- Group B preambles with Msg3 repetition is supported, it is up to network to decide whether to configure Group B together with Msg3 repetition.

- If Group B preambles with Msg3 repetition is configured, network can configure separate parameters for requesting Msg3 repetition, including ra-Msg3SizeGroupA, messagePowerOffsetGroupB and numberOfRA-PreamblesGroupA (ASN.1 details can be discussed in session on RACH partitioning)

- ra-ContentionResolutionTimer is started or restarted in the first symbol after all Msg3 repetitions.

- In shared RO case, it is not supported to configure a separate set of RACH parameters (preambleReceivedTargetPower, powerRampingStep, preambleTransMax) for requesting Msg3 repetition.

- In shared RO case, it is not supported to separately configure following parameters for requesting Msg3 repetition:

  - prach-ConfigurationIndex,

  - msg1-FDM,

  - msg1-FrequencyStart,

  - zeroCorrelationZoneConfig,

  - totalNumberOfRA-Preambles,

  - ssb-perRACH-OccasionAndCB-PreamblesPerSSB,

  - rsrp-ThresholdSSB-SUL,

  - prach-RootSequenceIndex,

  - msg1-SubcarrierSpacing,

  - restrictedSetConfig,

  - msg3-transformPrecoder,

- In shared RO case, it is up to the common RACH session to decide how to configure the number of preamble per SSB per RO, and how to indicate the start of preamble index for requesting Msg3 repetition.

- A separate rsrp-ThresholdSSB threshold is introduced for requesting Msg3 repetition.

- From CE perspective, carrier selection and BWP selection are performed ahead of CE selection during RACH procedure.

- From CE perspective, UE compares the RSRP of DL path-loss reference with the Msg3 repetition threshold [rsrp-Threshold-Msg3Rep] during the RACH initialization procedure and decides whether to use CE or non-CE RA.

- From CE perspective, if CE RA is selected, then the decision doesn’t change during the entire RACH procedure (i.e. until RACH failure).

      - RAN2#116bis-e meeting agreements:

- Non-CE to CE fallback is not supported in Rel-17.

- UE cannot fallback from CFRA or 2-step RACH to CE-RACH.

- CE-capable UEs use the legacy threshold, rsrp-ThresholdSSB-SUL, in its selection of UL carrier for RACH.

- Working assumption:

From RAN2’s perspective, a dedicated UL BWP can be configured with only CE RACH resources. Its feasibility is to be confirmed by RAN1.

- From CE’s perspective, it does not matter whether UE first selects RA type or CE when initiating a RACH procedure.

- From CE’s perspective, it is confirmed that the eligibility criteria for CE is determined before the selection of RACH partition.

- From CE’s perspective, the RSRP threshold for requesting Msg3 repetition can be configured per BWP on both NUL and SUL.

- When CE is configured in RACH partitions, the configuration granuality for the RSRP threshold for requesting Msg3 repetition should be decided by the common RACH session.

- From CE’s perspective, CE RACH can be configured with a separate RSRP threshold for SSB selection and this threshold can be configured per BWP.

- When CE is configured in RACH partitions, the configuration granularity for the RSRP threshold for SSB selection in a CE RACH procedure should be decided by the common RACH session.

- For now there is no consensus in RAN2 on whether to support CFRA with msg3 repetition or not, based on the assumption that RAN1 should discuss this first.