3GPP RAN WG2 Meeting #117-e	R2-2203422

eMeeting February 21st – March 3rd, 2022

Agenda Item:	8.10.3.2.1

Source:	InterDigital

Title:	Remaining RRC open issues

Document for:	Discussion, Decision

# Introduction

During [Post116bis-e][107][NTN] a number of remaining RRC open issues have been captured. Based on chair guidance, these issues are to be addressed as follows:

Regarding the RRC open issues listed in R2-2201896:

- Issues 1-5, 13, 15-16, 18-20, 21-24 will be handled in offline discussion [Pre117-e][NTN][101] RRC open issues

- Issues 6-10 can be handled via company contributions in AI 8.10.3.2.1

- Issues 11-12 will be handled by CR rapporteur directly in the running CR

- Issues 14 and 17 will be handled in the MAC discussion (in offline discussion [Pre117-e][NTN][103])

- Other RRC issues can be handled via company contributions in AI 8.10.3.2.2

This document discusses remaining RRC open issues 8, 9, and 10, which address open aspects of time-based and location-based conditional handover.

# Remaining issues for conditional handover

## Time-based CHO

From the RRC open issues list, there are the following aspects in Open issue 10 regarding time-based conditional handover (CHO) [1]:

- Does UE start to evaluate the RRM condition at the beginning of the time range?

- Does UE stop evaluating the RRM condition at the end of the time range since CHO cannot be triggered after time window.

From RAN2#114e [2], time-based CHO is defined as follows:

“CHO time trigger event is defined as time duration [t1, t2] associated for each CHO candidate cell. The UE shall execute CHO to that candidate cell during the time duration, if all other configured CHO execution conditions will apply and there is only one triggered candidate cell.”

And from RAN2#115e [3]:

“UE is allowed to perform HO only during T1 to T2”

From the above agreements it seems quite clear that if time-based condition is configured, CHO can only occur during the configured time range. There seems no reason to evaluate RRM outside this range if any other joint time-based condition will not be fulfilled. Furthermore, it is assumed that network will properly set the time range such that the candidate cell remains valid throughout the duration of the time range. There also seems no reason to reduce this validity duration by only evaluating the RRM condition during a portion of the time-based condition.

Proposal 1:	For time-based CHO, UE is only required to evaluate RRM conditions during the configured time range.

Another open issue in RRC CR is upon the expiry of the time range (i.e., after T2), what should the UE do with the corresponding time-based CHO configuration [1]:

Open issue 9: Whether the UE should delete the corresponding time-based CHO configuration after T2

This could depend on the movement of the UE. In NTN, a cell may serve the same geographic area periodically i.e., once the satellite circumnavigates the globe. If the UE is mobile (e.g., located on an airplane), it is likely that when the satellite comes around to the same position after an orbit, the UE will have moved significantly and the configuration it received the last time it was served by the cell may no longer be valid.

However, a stationary UE may be served by the same sequence of satellites within an orbit over and over again. In this case, the UE may store information regarding this satellite sequence once and not need to receive continuous CHO (re)configuration.

In the former case (i.e., mobile UE), the simplest solution would be to delete the corresponding time-based CHO configuration after T2. However in the stationary case, the UE may benefit from storing the CHO configuration, provided it is also given an offset indicating when this configuration is valid again. Although considering the limited time remaining in this Release, this may be considered at a later stage e.g. Release 18.

Proposal 2:	For at least mobile UE, corresponding time-based CHO configuration is deleted after T2. FFS for stationary UE.

## Location-based CHO

In Open issue 10 there is also the following aspect regarding location-based CHO [1]:

- If CHO can be used for failure recovery similar to legacy, UE can continue to keep CHO configuration at the end of time duration. If CHO cannot be used for failure recovery or other purpose, CHO configuration can be released directly at the end of time duration.

This would depend on how the time duration is configured. A reasonable implementation would be to set the time duration such that it expires when the cell reference point is sufficiently far from the UE that it may no longer considered as a valid candidate for mobility. In this case, after timer expiry the candidate may not be a useful for failure recovery anyways. Therefore, we think that the simplest solution would be to release configuration at end of a time duration.

Proposal 3:	UE releases location-based CHO configuration at end of time duration.

Similar to time-based CHO, when the UE starts to evaluate the measurement-based condition when location-based CHO is configured requires further clarification [1]:

- Does UE start to evaluate the measurement-based condition after the location condition is met. Our understanding is that UE starts to evaluate location-based condition but does not evaluate measurement-based condition immediately upon receiving the joint condition of location-based condition and measurement-based condition.

1We share the opinion that UE starts to evaluate location-based condition but is not required to evaluate measurement-based condition until location-based condition is satisfied. The location-based condition should be set such that it shall be satisfied once the neighbouring cell reference point is close enough to be considered a valid candidate for handover. There seems no reason to require UE to evaluate the measurement-based condition if this is not the case.

Proposal 4:	UE is only required to evaluate the measurement-based condition after the location condition is met.

## Joint configuration of time & location-based CHO

Finally, there remains an open issue as to whether location-based CHO and time-based CHO may be jointly configured [1]:

Open issue 8: Is the network allowed to configure location-based CHO and time-based CHO simultaneously?

Both time-based CHO and location-based CHO help improve robustness of measurement-based CHO conditions in an NTN  environment. We see no reason to limit network configuration and restrict simultaneous configuration of time and location-based CHO (in addition with a measurement-based condition).

Proposal 5:	Network may simultaneously configure location-based CHO and time-based CHO.

# Conclusion

In this contribution the following proposals were made regarding remaining open issues in RRC:

Proposal 1:	For time-based CHO, UE is only required to evaluate RRM conditions during the configured time range.

Proposal 2:	For at least mobile UE, corresponding time-based CHO configuration is deleted after T2. FFS for stationary UE.

Proposal 3:	UE releases location-based CHO configuration at end of time duration.

Proposal 4:	UE is only required to evaluate the measurement-based condition after the location condition is met.

Proposal 5:	Network may simultaneously configure location-based CHO and time-based CHO.

# References

R2-2201896 – NTN RRC open issues towards RAN2#117 (Ericsson)

R2-2106901 – Report of 3GPP TSG RAN WG2 meeting #114-e (ETSI MCC)

R2-2109301 – Report of 3GPP TSG RAN WG2 meeting #115-e (ETSI MCC)