3GPP TSG-RAN WG2 #117-e	Tdoc R2-2203465

Electronic meeting, 21st Feb – 3rd Mar 2022

Agenda Item:	8.13.3.2

Source:	Ericsson

Title:	On PSCell MHI and SCG MRO enhancements

Document for:	Discussion, Decision

# Introduction

In this paper, we focus on some issues concerning the following topics:

- Mobility History Information Reporting

- Enhancements to SCG related MRO

# Discussion

## Mobility History Information Related

### PSCell MHI Capability Indicator

One first MHI related issue lifted in the open issue list [2] relates to the need to introduce an explicit capability indicator that indicates that the UE is capable of storing the PSCell related MHI. In release-16, there is no explicit capability indicating the UE’s ability to store the PCell related MHI. Instead, the indication mobilityHistoryAvail, included in RRCSetupComplete and RRCResumeComplete implicitly acts as an indicator of UE capability to store PCell related MHI. However, when the PSCell related information is included in the MHI in release 17, there is no way for the network to know whether a UE is capable of storing PSCell MHI.

Since the legacy MHI is an optional feature without capability indication, the network will become aware of the MHI stored by the UE via RRC messages only when  mobilityHistoryAvail is included.

In Rel.17, without further information in the specification, the network will not know whether the UE may transmit also the PSCell MHI in the MHI report.

Two options has been discussed [1];

- Option-1: Introduce an explicit capability indicator that indicates that the UE is capable of storing the PSCell related MHI.

- Option-2: Introduce an explicit indicator (mobilityHistoryPSCellAvail) in RRCSetupComplete and RRCResumeComplete indicating whether the UE has PSCell related information available in its stored visitedCellInfoList.

In our view, the most straightforward solution would be option-1. This would also avoid the need to introduce new flags in the RRCSetupComplete and RRCResumeComplete messages.

Introduce an explicit capability indicator that indicates that the UE is capable of storing the PSCell related MHI.

It is true that the PSCell MHI capability indicator proposed in Proposal 1 does not guarantee that the UE will really transmit the PSCell MHI, but the network will know at least the UEs for which it is guaranteed that no PSCell MHI will be transmitted.

With the proposal in Proposal 1, the network will know at least the UEs (legacy UEs) for which it is guaranteed that no PSCell MHI will be transmitted.

## Enhancements to SCG Failure related reporting

During the offline discussion [Post116bis-e][833] many companies have expressed their desire to reopen the issue of reporting failedPSCellID, sourcePSCellID and timeSCGFailure in the SCGFailureInformation message. In this section, we address why this is not required and also propose to send an LS to RAN3 in clarifying this issue.

We have provided the explanation for different SCG failure scenarios (Intra-SN PSCell change, Inter-SN change) individually and provided analysis of how the existing information at the network is sufficient for the MRO analysis.

### MN initiated SN Change

The MN initiated SN change procedure is shown in the above signal flow graph. Please note that this is a condensed version of the signal flow graph in TS 37.340 clause 10.5.2. In this procedure, if the UE declares failure at the time of performing the RA procedure towards the Target_SN, then the UE creates the SCGFailureInformation message and sends it to the MN.

Upon receiving this message, the MN can check the contents of the failureType in the SCGFailureInformation.

If the failureType is set to ’ synchReconfigFailureSCG’, then the UE has declared failure while performing a reconfigurationWithSync procedure. However, the MN cannot be sure whether the UE declared SCG change failure while performing the SN change as instructed by MN or is it some other Target_SN initiated SN change failure. But this information is known to the Target_SN as the Target_SN knows whether the UE had completed the RA procedure associated to the MN initiated SN change procedure.

Thus, by forwarding the SCGFailureInformation as received by the MN to the Target_SN, the Target_SN can perform the MRO analysis and decide whether the failure occured at the time of accessing the Target_SN during MN initiated SN change procedure or due to the Target_SN initiated PSCell change procedure. Upon realizing that the UE had declared SCG Change failure at the time of performing SCG change procedure that was from source_SN to the target_SN (too early SN change), the target_SN can inform MN about it (similar to too early HO information being sent from the failed Pcell to the source Pcell in the Failure Indication Message).

The same analysis is applicable even when the UE would have set the failureType to ’ randomAccessProblem’ while T304 for the SCG is running.

For any other failureType, it is very clear that the failure must have occured while being connected to the Target_MN. This is because the RRC(Connection)ReconfigurationComplete message acts as an implicit indication to the MN that the UE has received the SN change command and thus the UE has ’left’ the source cell. From here on, if the UE declares failure because of t310-Expiry or beamFailureRecoveryFailure or t312-Expiry etc. then this must have occurred at the target SN.  Further, the Target_SN also knows how long the UE has spent after completing the RA procedure from the Source_SN (via PSCell related UHI). Thus even if it wants to classify the SN change as a too-early SN change, it can do so.

Existing contents of SCGFailureInformation are sufficient to perform MRO analysis at the time of declaring SCG failure during MN initiated SN change procedure. All the required information is available at Target_SN and upon receiving the SCGFailureInformation from the MN, the Target_SN can indicate whether this was a too early SN change (initiated by MN) or too early PSCell change (initiated by Target_SN after the completion of MN initiated SN change procedure).

If the MN receives the SCGFailureInformation message from the UE, after receiving the RRC(Connection)ReconfigurationComplete message in response to the RRC(Connection)Reconfiguration message instructing the UE to perform the SN change, then the MN can forward the SCGFailureInformation message to the target SN.

### SN initiated SN Change

The SN initiated SN change procedure is shown in the above signal flow graph. Please note that this is a condensed version of the signal flow graph in TS 37.340 clause 10.5.2. In this procedure, if the UE declares failure at the time of performing the RA procedure towards the Target_SN, then the UE creates the SCGFailureInformation message and sends it to the MN.

Upon receiving this message, the MN can check the contents of the failureType in the SCGFailureInformation.

If the failureType is set to ’ synchReconfigFailureSCG’, then the UE has declared failure while performing a reconfigurationWithSync procedure. However, the MN cannot be sure whether the UE declared SCG change failure while performing the SN change as instructed by MN or is it some other Target_SN initiated SN change failure. But this information is known to the Target_SN as the Target_SN knows whether the UE had completed the RA procedure associated to the MN initiated SN change procedure.

Thus, by forwarding the SCGFailureInformation as recieved by the MN to the Target_SN, the Target_SN can perform the MRO analysis and decide whether the failure occured at the time of accessing the Target_SN during MN initiated SN change procedure or due to the Target_SN initiated PSCell change procedure. Upon realizing that the UE had declared SCG Change failure at the time of performing SCG change procedure that was from source_SN to the target_SN (too early SN change), the target_SN can inform MN about it (similar to too early HO information being sent from the failed Pcell to the source Pcell in the Failure Indication Message). The MN can then forward the same to the Source_SN.

The same analysis is applicable even when the UE would have set the failureType to ’ randomAccessProblem’ i.e., if the Target_SN had already received the RRCReconfigurationComplete then the target_SN knows the UE had completed the SN change procedure and the current failure is associated to some other issue. If the Target_SN had not received the RRCReconfiugrationComplete but if the MN had sent the SN Reconfiguration Complete message to the Target_SN, then the Target_SN knows that the UE must have declared SCG failure while performing the SN change procedure. Thus, this is a too early SN change and the target_SN can indicate the same to the MN.

For any other failureType, it is very clear that the failure must have occured while being connected to the Target_MN. This is because the RRC(Connection)ReconfigurationComplete message acts as an implicit indication to the MN that the UE has received the SN change command and thus the UE has ’left’ the source cell. From here on, if the UE declares failure because of t310-Expiry or beamFailureRecoveryFailure or t312-Expiry etc. then this must have occurred at the target SN.

Existing contents of SCGFailureInformation are sufficient to perform MRO analysis at the time of declaring SCG failure during SN initiated SN change procedure. All the required information is available at Target_SN and upon receiving the SCGFailureInformation from the MN, the Target_SN can indicate whether this was a too early SN change (initiated by Source SN) or too early PSCell change (initiated by Target_SN after the completion of SN initiated SN change procedure).

If the MN receives the SCGFailureInformation message from the UE, after receiving the RRC(Connection)ReconfigurationComplete message in response to the RRC(Connection)Reconfiguration message instructing the UE to perform the SN change, then the MN can forward the SCGFailureInformation message to the target SN.

### Intra-SN PSCell Change (with SRB1)

The SN initiated PSCell change procedure using SRB1 is shown in the above signal flow graph. In this procedure, if the UE declares failure at the time of performing the RA procedure, then the UE creates the SCGFailureInformation message and sends it to the MN.

In this case, as the MN was not involved for any SN change procedure, the MN can always send the SCGFailureInformation to the current SN.

Upon receiving the SCGFailureInformation from the MN, the SN can check if the SN had received the S-NODE MODIFICATION CONFIRM message from the MN prior to receiving the SCGFailureInformation or not. If the MN had sent the S-NODE MODIFICATION CONFIRM message then the UE had received the PSCell change command and any failure that has occured must have occured while accessing the target PSCell. If the MN had not sent the S-NODE MODIFICATION CONFIRM message then the UE had not received the PSCell change command and any failure that has occured must have occured in the source PSCell.

Existing contents of SCGFailureInformation are sufficient to perform MRO analysis at the time of declaring SCG failure during intra-SN PSCell change procedure involving SRB1. All the required information is available at the SN and upon receiving the SCGFailureInformation from the MN, the SN can realise whether this was a too early PSCell change or too late PSCell change.

If the MN receives the SCGFailureInformation message from the UE while not involving any SN change procedure, then the MN can forward the SCGFailureInformation message to the SN.

### Intra-SN PSCell Change (with SRB3)

The SN initiated PSCell change procedure using SRB3 is shown in the above signal flow graph. In this procedure, if the UE declares failure at the time of performing the RA procedure, then the UE creates the SCGFailureInformation message and sends it to the MN.

In this case, as the MN was not involved for any SN change procedure, the MN can always send the SCGFailureInformation to the current SN.

Upon receiving the SCGFailureInformation from the MN, the SN can check the contents of the SCGFailureInformation for further analysis. The contents of SCGFailureInformation message are provided below.

FailureReportSCG ::=                       SEQUENCE {

failureType            ENUMERATED {

t310-Expiry, randomAccessProblem,rlc-MaxNumRetx, synchReconfigFailureSCG,

scg-ReconfigFailure, srb3-IntegrityFailure, other-r16, spare1},

measResultFreqList     MeasResultFreqList       OPTIONAL,

measResultSCG-Failure  OCTET STRING (CONTAINING MeasResultSCG-Failure)                OPTIONAL,

...,

[[

locationInfo-r16       LocationInfo-r16            OPTIONAL,

failureType-v1610      ENUMERATED {scg-lbtFailure-r16, beamFailureRecoveryFailure-r16,

t312-Expiry-r16, bh-RLF-r16, spare4, spare3, spare2, spare1} OPTIONAL

]]

}

MeasResultSCG-Failure ::=           SEQUENCE {

measResultPerMOList                 MeasResultList2NR,

...,

[[

locationInfo-r16                    LocationInfo-r16            OPTIONAL

]]

}

MeasResultList2NR ::=               SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR

MeasResult2NR ::=                   SEQUENCE {

ssbFrequency                        ARFCN-ValueNR                           OPTIONAL,

refFreqCSI-RS                       ARFCN-ValueNR                           OPTIONAL,

measResultServingCell               MeasResultNR                            OPTIONAL,

measResultNeighCellListNR           MeasResultListNR                        OPTIONAL,

...

}

Based on the above shown ASN.1 contents, the SN will receive the failureType and SN will also receive the measurements of serving and neighbor cells as configured by the SN.

If the failureType is set to ’ synchReconfigFailureSCG’, then the UE has declared failure while performing a reconfigurationWithSync procedure towards the target PSCell. By knowing this information, the SN would be able to categorize the PSCell change as a too early PSCell change.

If the failureType is set to t310-Expiry or beamFailureRecoveryFailure or t312-Expiry etc., and if the target PSCell had not received the UE yet, then it is very clear that the failure was declared in the source PSCell. Based on this, the SN can categorize this as a too late PSCell change since the failure was declared at the source cell.

If the failureType is set to randomAccessProblem and if the target PSCell had not received the UE yet, then the SN might not be able to identify whether the failure occured while performing the random access towards the source PSCell or towards the target PSCell. However, there are additional information available in the serving cell measurements. It is to be noted that the serving cell measurements are included in a different IE compared to the neighbor cell measurements. For example, if the UE had included the source PSCell as a serving cell, then the SN can interpret that the UE must have declared failure while performing RA procedure in the source PSCell itself. Whereas, if the UE had included the target PSCell as a serving cell, then the SN can interpret that the UE must have declared failure while performing RA procedure in the target cell.

When the failureType is set to randomAccessProblem, the SN can use the measResultSCG-Failure included in the SCGFailureInformation to deduce if the failure occurred while performing RA in the source PSCell  or if the failure occurred while performing RA in the target PSCell  .

Existing contents of SCGFailureInformation are sufficient to perform MRO analysis at the time of declaring SCG failure during intra-SN PSCell change procedure involving SRB3. All the required information is available at the SN and upon receiving the SCGFailureInformation from the MN, the SN can realise whether this was a too early PSCell change or too late PSCell change.

If the MN receives the SCGFailureInformation message from the UE while not involving any SN change procedure, then the MN can forward the SCGFailureInformation message to the SN.

Based on the above analysis of all SN/PSCell change procedures, it is clear that no additional cell identifier information (failedPSCellID, sourcePSCellID) are required to perform the MRO analysis. This can be indicated to RAN3 so that they can use this information in their work.

Further, the network nodes need to maintain the UE history information related information. This information comprises of how long a UE has had a RRC connection in that cell. In Rel-17, the PSCell related UE history information has been introduced in RAN3. Based on this, a connected mode UE’s time of stay in each PSCell is recorded by the SN.

Based on the above analysis of all SN/PSCell change procedures, it is clear that no additional information (failedPSCellID, sourcePSCellID and timeSCGFailure) are required to perform the MRO analysis. This can be indicated to the RAN3 so that they can use this information in their work. Thus, a SN needs to collect the time of stay of the UE. Thus, when a PSCell change is completed the target SN starts a timer and this timer is stopped either upon completing the next PSCell change or at the reception of SCGFailureInformation. This value then represents ‘timeSCGFailure’.

timeSCGFailure is nothing but the time of stay of a UE in a PSCell as recorded in the PSCell UHI (UE history information) as standardized by RAN3 wherein the time of stay related timer is started at the time of UE entering the PSCell and the timer is stopped at the time of reception of SCGFailureInformation that is forwarded by the MN.

Based on the above analysis of all SN/PSCell change procedures, it is clear that timeSCGFailure can be derived by the network node on its own. This can be indicated to the RAN3 so that they can use this information in their work.

RAN2 to send an LS to RAN3 to indicate that failedPSCellID, sourcePSCellID and timeSCGFailure will not be introduced in SCGFailureInformation message.

### RA Information associated to a failed RA procedure for SCG MRO

During RAN2#116 meeting, the following agreements were made.

Report and Content of SCG Failure Information:

The UE needs to include RA information in case that failureType is set to randomAccessProblem or beamFailureRecoveryFailure-r16

RA-InformationCommon-r16 is used as a baseline to indicate random-access related information set by the PSCell.

The parameter connectionFailureType could reuse the current failureType in SCG failure message.

FFS on enhancements.

The condition “failureType is set to synchReconfigFailureSCG” for including RA information.

FFS: Introduce one bit flag to indicate whether T304 is running or not in SCG failure message.

The SCGFailureInformation is transmitted by the UE immediately after declaring failure on the SCG. Thus, it is fair to assume that the UE context is still available at both MN as well as the SN. Further, upon receiving the SCGFailureInformation, the MN forwards part of the contents of this message, scgFailureInfo, to the SN.

CG-ConfigInfo-IEs ::=           SEQUENCE {

ue-CapabilityInfo               OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)          OPTIONAL,-- Cond SN-AddMod

candidateCellInfoListMN         MeasResultList2NR          OPTIONAL,

candidateCellInfoListSN         OCTET STRING (CONTAINING MeasResultList2NR)   OPTIONAL,

measResultCellListSFTD-NR       MeasResultCellListSFTD-NR                     OPTIONAL,

scgFailureInfo                  SEQUENCE {

failureType                     ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, synchReconfigFailure-SCG,

scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG                   OCTET STRING (CONTAINING MeasResultSCG-Failure)

}    OPTIONAL,

configRestrictInfo              ConfigRestrictInfoSCG       OPTIONAL,

drx-InfoMCG                     DRX-Info                    OPTIONAL,

measConfigMN                    MeasConfigMN                OPTIONAL,

sourceConfigSCG                 OCTET STRING (CONTAINING RRCReconfiguration)  OPTIONAL,

scg-RB-Config                   OCTET STRING (CONTAINING RadioBearerConfig)   OPTIONAL,

mcg-RB-Config                   OCTET STRING (CONTAINING RadioBearerConfig)   OPTIONAL,

mrdc-AssistanceInfo             MRDC-AssistanceInfo                           OPTIONAL,

nonCriticalExtension            CG-ConfigInfo-v1540-IEs                       OPTIONAL

}

Many companies have shown interest to include the RA-Information associated to the RA procedure for PSCell change in SCGFailureInformation message at failure time. On the other hand, the size of the SCGFailureInformation, a mandatory message, is increased upon declaring SCG failure which is not desired.

The RA related failure information can be included selectively instead of all types of SCG failure scenarios associated to random access procedures i.e., failureType “beamFailureRecoveryFailure” could be due to other reasons than MRO and it happens more often. Hence, we propose that the RA-InformationCommon associated to the RA procedure that led to SCG failure are included in the RA report when the SCG failure type is set “randomAccessProblem” while T304 is running or when the failureType is set to “synchReconfigFailure-SCG”. By doing so, we reduce the instances when the RA related information is included in the SCGFailureInformation

The RA related information associated to the RA procedure that led to SCG failure are included only when:

  - the SCG failureType is set “randomAccessProblem” and T304 was running  at the time of declaring SCG failure

  - the SCG failureType is set to “synchReconfigFailure-SCG”.

Further, as the cell change procedures are always performed using the default BWP resources, the frequency information of the RA resources can be implicitly derived by the network. Thus, it is sufficient to include only perRAInfoList instead of the entire ra-InformationCommon. Similar size reduction is already performed for CEFReprot

The RA related information to be included in the SCGFailureInformation message is restricted to only perRAInfoList (instead of entire ra-InformationCommon) just like CEF report.

With this size reduction, the network can also implicitly derive that the UE has declared SCG failure due to SCG change related operation i.e., when the UE includes the RA related information (only perRAInfoList), then the network can implicitly derive that the UE is generating the SCGFailureInformation to indicate a failed SCG change procedure and thus there is no need to add one-bit flag.

No need to include a 1 bit flag in the SCGFailureInformation to indicate that the T304 was running when the UE declared the SCG failure due to random access problem indication in the SCG MAC. This information can be implicitly derived based on Proposal 3 and Proposal 4.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	Since the legacy MHI is an optional feature without capability indication, the network will become aware of the MHI stored by the UE via RRC messages only when  mobilityHistoryAvail is included.

Observation 2	In Rel.17, without further information in the specification, the network will not know whether the UE may transmit also the PSCell MHI in the MHI report.

Observation 3	With the proposal in Proposal 1, the network will know at least the UEs (legacy UEs) for which it is guaranteed that no PSCell MHI will be transmitted.

Observation 4	Existing contents of SCGFailureInformation are sufficient to perform MRO analysis at the time of declaring SCG failure during MN initiated SN change procedure. All the required information is available at Target_SN and upon receiving the SCGFailureInformation from the MN, the Target_SN can indicate whether this was a too early SN change (initiated by MN) or too early PSCell change (initiated by Target_SN after the completion of MN initiated SN change procedure).

Observation 5	If the MN receives the SCGFailureInformation message from the UE, after receiving the RRC(Connection)ReconfigurationComplete message in response to the RRC(Connection)Reconfiguration message instructing the UE to perform the SN change, then the MN can forward the SCGFailureInformation message to the target SN.

Observation 6	Existing contents of SCGFailureInformation are sufficient to perform MRO analysis at the time of declaring SCG failure during SN initiated SN change procedure. All the required information is available at Target_SN and upon receiving the SCGFailureInformation from the MN, the Target_SN can indicate whether this was a too early SN change (initiated by Source SN) or too early PSCell change (initiated by Target_SN after the completion of SN initiated SN change procedure).

Observation 7	If the MN receives the SCGFailureInformation message from the UE, after receiving the RRC(Connection)ReconfigurationComplete message in response to the RRC(Connection)Reconfiguration message instructing the UE to perform the SN change, then the MN can forward the SCGFailureInformation message to the target SN.

Observation 8	Existing contents of SCGFailureInformation are sufficient to perform MRO analysis at the time of declaring SCG failure during intra-SN PSCell change procedure involving SRB1. All the required information is available at the SN and upon receiving the SCGFailureInformation from the MN, the SN can realise whether this was a too early PSCell change or too late PSCell change.

Observation 9	If the MN receives the SCGFailureInformation message from the UE while not involving any SN change procedure, then the MN can forward the SCGFailureInformation message to the SN.

Observation 10	When the failureType is set to randomAccessProblem, the SN can use the measResultSCG-Failure included in the SCGFailureInformation to deduce if the failure occurred while performing RA in the source PSCell  or if the failure occurred while performing RA in the target PSCell  .

Observation 11	Existing contents of SCGFailureInformation are sufficient to perform MRO analysis at the time of declaring SCG failure during intra-SN PSCell change procedure involving SRB3. All the required information is available at the SN and upon receiving the SCGFailureInformation from the MN, the SN can realise whether this was a too early PSCell change or too late PSCell change.

Observation 12	If the MN receives the SCGFailureInformation message from the UE while not involving any SN change procedure, then the MN can forward the SCGFailureInformation message to the SN.

Observation 13	timeSCGFailure is nothing but the time of stay of a UE in a PSCell as recorded in the PSCell UHI (UE history information) as standardized by RAN3 wherein the time of stay related timer is started at the time of UE entering the PSCell and the timer is stopped at the time of reception of SCGFailureInformation that is forwarded by the MN.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Introduce an explicit capability indicator that indicates that the UE is capable of storing the PSCell related MHI.

Proposal 2	RAN2 to send an LS to RAN3 to indicate that failedPSCellID, sourcePSCellID and timeSCGFailure will not be introduced in SCGFailureInformation message.

Proposal 3	The RA related information associated to the RA procedure that led to SCG failure are included only when:

a.	the SCG failureType is set “randomAccessProblem” and T304 was running  at the time of declaring SCG failure

b.	the SCG failureType is set to “synchReconfigFailure-SCG”.

Proposal 4	The RA related information to be included in the SCGFailureInformation message is restricted to only perRAInfoList (instead of entire ra-InformationCommon) just like CEF report.

Proposal 5	No need to include a 1 bit flag in the SCGFailureInformation to indicate that the T304 was running when the UE declared the SCG failure due to random access problem indication in the SCG MAC. This information can be implicitly derived based on Proposal 3 and Proposal 4.

# 4	References

R2-2200005, [Post116-e][887.5][SONMDT] Leftover issues on SON (Ericsson)

Post116bis-e][833][SON/MDT] SON related open issue list (Ericsson)