3GPP TSG-RAN WG2 Meeting #117-e	R2-2203478

Electronic Meeting, February 21 – March 3, 2022

Agenda item:		8.9.3.2.1

Source:		Futurewei

Title:	On supporting PEI with eDRX

Document for:		Discussion and Decision

# Introduction

In RAN2 meeting #116bis-e [1], it was concluded that “RAN2 aims to Support PEI and subgrouping with eDRX”.

In this contribution, we propose some details regarding support of PEI with eDRX.

# Discussions

## Introducing dedicated subgroup for UEs configured with eDRX

UEs running on eDRX cycles tend to be paged rarely and power sensitive. When they share a same subgroup ID and PO with other UEs not configured with eDRX, their false paging alarm rate may still be too high due to (relatively) frequent paging for UEs not configured with eDRX but sharing the same subgroup ID and PO. Therefore, it may be desirable to separate UEs configured with eDRX cycles from UEs configured with legacy DRX cycles.

There are two approaches for separating UEs configured with eDRX cycles from UEs configured with legacy DRX cycles: CN-based and RAN-based.

## CN-based approach

In CN-based approach, the AMF may separate a UE configured with an eDRX cycle from UEs configured with legacy DRX cycles when assigning a CN-assigned subgroup ID to the UE configured with eDRX cycle. For example, the AMF may dedicate a CN-assigned subgroup for UEs configured with eDRX cycles. Such special subgrouping is transparent to RAN. However, this means that the AMF may not be able to assign a CN-assigned subgroup to the UE until the AMF knows whether the UE has been configured with an eDRX cycle, or means that the AMF, after the initial assignment, may need to re-assign the UE to the dedicated CN-assigned subgroup after the AMF knows that the UE has been configured with an eDRX cycle. In later case, another NAS signaling to the UE is needed.

## RAN-based approach

In RAN-based approach, the RAN, on a per-cell basis, may dedicate a subgroup (e.g., a subgroup taken from Nsg-UEID RAN-controlled groups but is not used for UEID-based subgrouping) for UEs configured with eDRX cycles. The RAN broadcasts the subgroup ID of this dedicated subgroup or simply broadcasts an indicator indicating that there is a dedicated RAN-controlled group for UEs configured with eDRX cycles. The subgroup ID of the dedicated subgroup is determined as the last RAN-controlled subgroup ID, which is also the last subgroup ID among all subgroups configured in the cell, and therefore can be determined directly from subgroupNumPerPO.

The presence (in the SI) of the subgroup ID of the dedicated subgroup or the presence of the indicator indicating that there is a dedicated RAN-controlled group also indicates to UEs, which will use the UEID-based subgrouping in the cell, a need to subtract Nsg-UEID broadcasted in the SI by one before using the subtracted Nsg-UEID in hashing its UEID.

With the RAN-based approach, any UE configured with an eDRX cycle and camping in such cell is automatically assigned to the dedicated subgroup no matter the UE supports CN-assigned subgrouping or not, supports UEID-based subgrouping or not, and/or has previously been assigned with a CN-assigned subgroup or not. With a new UE capability bit to be introduced for indicating whether a UE supports dedicated eDRX subgrouping or not, the UE and RAN are perfectly in synch with regards to whether to use the dedicated subgroup ID for the UE’s PEI or not.

## Within the dedicated subgroup

Only one dedicated subgroup is needed. Among the UEs configured with eDRX cycles, their PTWs can be offset from one another so that they don't falsely wake up each other.

Proposal 1: RAN2 consider introducing dedicated eDRX subgrouping for UEs configured with eDRX cycles for providing better protection to UEs configured with eDRX cycles.

# Conclusions

The following is proposed:

Proposal 1: RAN2 consider introducing dedicated eDRX subgrouping for UEs configured with eDRX cycles for providing better protection to UEs configured with eDRX cycles.

# References

R2-116bise Chair Notes Jan 28 EOM_rev2