3GPP TSG RAN WG2#117-e	R2-2203738

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG-RAN WG1 #108-e	R1-2202764

e-Meeting, February 21st – March 3rd, 2022

Title:	LS on updated Rel-16 RAN1 UE features lists for NR after RAN1#108-e

Response to:

Release:	Rel-16

Work Items:	NR_2step_RACH-Core, NR_unlic-Core, NR_IAB-Core, 5G_V2X_NRSL-Core, NR_L1enh_URLLC-Core, NR_IIOT-Core, NR_eMIMO-Core, NR_UE_pow_sav-Core, NR_pos-Core, NR_Mob_enh-Core, LTE_NR_DC_CA_enh-Core, TEI16, NR_CLI_RIM-Core

Source:	RAN WG1

To:	RAN WG2

CC:	RAN WG4

Contact Person:

#### Name:	Hiroki Harada

E-mail Address:	hiroki.harada@docomo-lab.com

Attachment:	             R1-2202763.zip (Updated version of RAN1 UE features list for Rel-16 NR after RAN1#108-e)

1. Overall Description:

Based on the agreements made in the RAN1#108-e meeting, RAN1 agreed to send the updated version of Rel-16 NR UE features list to RAN2 to facilitate their work on updating TR38.822. RAN1 kindly would like to ask RAN2 to take into account the updated Rel-16 RAN1 NR UE features list.

Following are some notes regarding the updates on the NR features list:

- As pointed by RAN2 LS in R1-2200856(R2-2111483), there were typos in FG13-2b/3b/4b and they are revised in the updated Rel-16 RAN1 UE features list.

- RAN1 discussed on whether pdcch-MonitoringSingleOccasion introduced for a certain case in DSS scenario with 15 kHz SCS needs to be revised or not. Currently it enables a UE to monitor PDCCH on a single span of three contiguous OFDM symbols that is not limited to the first three consecutive OFDM symbols in a slot. However, it is beyond what is necessary, and hence it was proposed that desired feature for the case in DSS scenario is, in reality, limited to PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot. Considering the potential NBC issue, RAN1 agreed to introduce the corresponding new UE capability (FG22-12) for Rel-16.

- As in another LS from RAN1 to RAN2/4 in R1-2202778, RAN1 agreed to introduce the new FG22-13 for cross-PUCCH group CSI reporting.

- RAN1 discussed UE feature for Multi-DCI Multi-TRP PUSCH operation to clarify how the multi-DCI multi-TRP PUSCH operation is supported by existing FGs. RAN1 agreed to make some clarification for FG16-2a as in the updated Rel-16 RAN1 UE features list.

RAN1 will share further updated version of the UE features list if any with RAN2/4 according to the further RAN1 discussion in next meeting.

2. Actions:

To RAN WG2

ACTION:

- RAN WG1 kindly would like to ask RAN WG2 to take into account the updated list of Rel-16 RAN1 NR UE features for updating TR38.822.

3. Date of Next RAN WG1 Meetings:

TSG-RAN WG1 Meeting #109-e			9th May – 20th May 2022		E-meeting.