3GPP TSG RAN WG2#117-e	R2-2204010

e-Meeting, 21st February - 3rd March, 2022

3GPP TSG-RAN WG1 Meeting #108-e 	R1-2202656

# e-Meeting, February 21st - March 3rd, 2022

Title:	Reply LS on the physical layer aspects of small data transmission

Response to: 	R1-2200881(R2-2201828)

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	RAN1

To:	RAN2

Contact Person:

#### Name:	Ziyang Li

E-mail Address:	li.ziyang1@zte.com.cn

Attachments:	None

1. Overall Description:

RAN1 would like to thank RAN2 for the reply LS in R1-2200881 and following question:

RAN2 would like RAN1 to confirm that the separate BWP in case of REDCAP may still be considered as the initial BWP and SDT resources can hence be configured on this BWP for REDCAP UEs.

RAN1 has discussed the question and made the following agreement:

RAN1 confirms that the separate BWP in case of RedCap may still be considered as the initial BWP and SDT resources(both CG-SDT and RA-SDT) can hence be configured on this BWP for RedCap UEs. - Note: details can be further studied to ensure proper functionality of RedCap UE performing SDT.

RAN1 has further discussed the remaining physical layer issues which have RRC impact, and the following agreements have been achieved:

Mapping ratio For CG-SDT, support mapping ratio {1/8, 1/4, 1/2} for SSB to CG PUSCH mapping. Repetition For CG-SDT, RAN1 cannot reach consensus on whether to support repetition or not, it’s up to RAN2 to decide on it. Association period - For CG-SDT, the starting time of association period is SFN0. - Regarding the candidate value set of association period, adopt the Table 2.4-1 - FFS CG period smaller than 5ms - Note: It does not mean to RAN2 the maximum CG period is 640ms. The table will be updated if RAN2 introduces other CG period values. - Note: The potential impact on PDCCH monitoring periodicity should be considered if larger CG period value is introduced in RAN2 Table 2.4-1: Mapping between CG period and SS/PBCH block to CG PUSCH resource association period CG period Tcg (msec); Association period (number of CG periods except when CG period is less than 5 ms). 5; {1, 2, 4, 8,16, 32, 64, 128}. 8; {1, 2, 4, 5, 8, 10, 16, 20, 40, 80}. 10; {1, 2, 4, 8,16, 32, 64}. 16; {1, 2, 4, 5, 8,10,20,40}. 20; {1, 2, 4, 8,16, 32}. 32; {1, 2, 4, 5, 10, 20}. 40; {1, 2, 4, 8, 16}. 64; {1, 2, 5, 10}. 80; {1, 2, 4, 8}. 128; {1, 5}. 160; {1, 2, 4}. 320; {1, 2}. 640; {1 }. DMRS configuration Introduce a new parameter e.g. sdt-DMRSports to configure the set of DMRS ports for SSB to PUSCH mapping. - Parameter antennaPort is not applicable to CG-SDT Support up to 2 DMRS sequences for CG-SDT for CP-OFDM, the generation mechanism and configuration can reuse that of msgA PUSCH. - Introduce a new parameter sdt-NrofDMRS-Sequences to configure 1 or 2 DMRS sequences. - The description of parameter dmrs-SeqInitialization can be revised as “ It’s present when single DMRS sequence is configured for CG-SDT”. Antenna port Only single antenna port for single layer transmission is supported for CG-SDT - srs-ResourceIndicator in ConfiguredGrantConfig is not applicable to CG-SDT. - precodingAndNumberOfLayers in ConfiguredGrantConfig is always 1 for CG-SDT. BWP level RRC configuration - UE specific parameter pucch-Config-r17 is not needed for SDT. - RAN1 cannot reach consensus on the support of UE specific CORESET, UE specific parameters pusch-Config-r17 and pdsch-Config-r17. Others - It’s up to RAN2 to decide on whether to support uci-OnPUSCH for CG-SDT. - phy-PriorityIndex-r16 in ConfiguredGrantConfig is not applicable to CG-SDT.

2. Actions:

To RAN2:

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account.

4. Date of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #109-e	16th – 27th May 2022	e-meeting