3GPP TSG-RAN WG2 Meeting #117-e	R2-22xxxxx

Online, 21 February-3 March 2022

Source: 	Session Chair (MediaTek)

Title:	Report from session on positioning and sidelink relay

# Status of At-Meeting Email Discussions

This subclause is not an Agenda Item. It contains a running summary of the email discussions assigned to take place during the meeting weeks.  This section will be moved to an appendix in the final version of the report.

[AT117-e][600][POS][Relay] Organisational Nathan – Positioning/Relay (MediaTek)

Scope: Organisational discussions and announcements, as needed throughout the meeting weeks.

Intended outcome: Well-informed participants

Deadline:  Thursday 2022-03-03 1000 UTC

Positioning running CRs:

[AT117-e][601][POS] BDS running CRs (CATT)

Scope: Review the following CRs, collect comments, and update if necessary:

- R2-2202402 (BDS introduction to 37.355)

- R2-2202403 (BDS introduction to 36.305)

- R2-2202404 (BDS introduction to 38.305)

Intended outcome: Endorsable CRs

Deadline:  Friday 2022-02-25 1000 UTC

[AT117-e][602][POS] NavIC running CRs (Ericsson/Huawei)

Scope: Review the following CRs, collect comments, and update if necessary:

- R2-2202607 (NavIC introduction to 38.305)

- R2-2203710 (NavIC introduction to 38.331)

Intended outcome: Endorsable CRs

Deadline:  Friday 2022-02-25 1000 UTC

[AT117-e][603][POS] Integrity stage 2 CRs (InterDigital)

Scope: Review and update the following CRs:

- R2-2202861 (integrity introduction to 36.305)

- R2-2202862 (integrity introduction to 38.305)

Intended outcome: Endorsable CRs

Deadline:  Friday 2022-02-25 1000 UTC

[AT117-e][604][POS] RAT-dependent positioning running CR to 38.305 (Intel)

Scope: Review and update the CR in R2-2202490.

Intended outcome: Endorsable CR

Deadline:  Friday 2022-02-25 1000 UTC

[AT117-e][605][POS] Capability running CRs (Intel)

Scope: Review and update the following CRs:

- R2-2202495 (capability running CR to 38.331)

- R2-2202496 (capability running CR to 38.306)

Intended outcome: Endorsable CRs

Deadline:  Friday 2022-02-25 1000 UTC

[AT117-e][606][POS] LPP running CR (Qualcomm)

Scope: Review and update the CR in R2-2203310.

Intended outcome: Endorsable CR

Deadline:  Friday 2022-02-25 1000 UTC

[AT117-e][607][POS] Positioning running CR to 38.331 (Ericsson)

Scope: Review and update the CR in R2-2203364, including merge of the draft CRs in R2-2203362 and R2-2203445.

Intended outcome: Endorsable CR

Deadline:  Friday 2022-02-25 1000 UTC

[AT117-e][608][POS] Positioning running CR to 38.321 (Huawei)

Scope: Review and update the CR in R2-2202605.

Intended outcome: Endorsable CR

Deadline:  Friday 2022-02-25 1000 UTC

[AT117-e][609][POS] Positioning running CR to 36.331 (Huawei)

Scope: Review and update the CR in R2-2202606.

Intended outcome: Endorsable CR

Deadline:  Friday 2022-02-25 1000 UTC

Relay running CRs:

[AT117-e][610][Relay] Relay running CR to 38.300 (MediaTek)

Scope: Review and update the CR in R2-2202343.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2022-03-01 1200 UTC

[AT117-e][611][Relay] Relay running CR to 38.304 (Ericsson)

Scope: Review and update the CR in R2-2203324.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2022-03-01 1200 UTC

[AT117-e][612][Relay] Relay running CR to 38.306 (Qualcomm)

Scope: Review and update the CR in R2-2203519.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2022-03-01 1200 UTC

[AT117-e][613][Relay] Relay running CR to 38.321 (Apple)

Scope: Review and update the CR in R2-2202543.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2022-03-01 1200 UTC

[AT117-e][614][Relay] Relay running CRs to 38.322/38.323 (Samsung)

Scope: Review and update the CRs in R2-2202950 and R2-2202951.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2022-03-01 1200 UTC

[AT117-e][615][Relay] Relay running CR to 38.331 (Huawei)

Scope: Review and update the CR in R2-2202819.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2022-03-01 1200 UTC

[AT117-e][616][Relay] Relay running CR to 38.351 (OPPO)

Scope: Review and update the CR in R2-2202276.

Intended outcome: Agreeable CR

Deadline:  Tuesday 2022-03-01 1200 UTC

Other discussions:

[AT117-e][617][POS] LS to RAN1 on positioning issues needing further input (Intel)

Scope: Draft an LS to RAN1 based on the outcome of [Pre117-e][614], taking into account other issues identified in the pre-meeting discussions where guidance from RAN1 is needed.

Intended outcome: Approvable LS

Deadline:  Wednesday 2022-02-23 0200 UTC

[AT117-e][618][POS] Beam and antenna information for DL-AoD accuracy enhancements (CATT)

Scope: Treat P10/P11/P12/P13/P15 of R2-2202410 and attempt to converge.

Intended outcome: Report to Monday online session

Deadline:  Friday 2022-02-25 1000 UTC

[AT117-e][619][Relay] Flow control and pre-emptive BSR mechanisms (Samsung)

Scope: Discuss P1-P3 of R2-2202955 and determine if agreeable mechanisms can be developed.  The features can be considered independently of each other.

Intended outcome: Endorsable TPs to affected specifications

Deadline:  Thursday 2022-02-24 1200 UTC

[AT117-e][620][Relay] Reply LS to RAN3 on mapping configuration (Samsung)

Scope: Draft a reply to the LS in R2-2202136.

Intended outcome: Approved LS (preferably without CB)

Deadline:  Thursday 2022-02-24 1200 UTC

[AT117-e][621][Relay] Additional issues on service continuity (OPPO)

Scope: Filter the issues raised in company tdocs under agenda item 8.7.2.2, determine if any critical issues need resolution, and attempt to converge on any critical issues.

Intended outcome: Report to Friday online session

Deadline:  Thursday 2022-02-24 1200 UTC

[AT117-e][622][Relay] Remaining issues on discovery and (re)selection (ZTE)

Scope:

- Discuss the “for discussion” proposals from R2-2202378 and attempt to converge.

- Filter the issues raised in company tdocs under agenda item 8.7.2.5, determine if any critical issues need resolution, and attempt to converge on any critical issues.

Intended outcome: Report to Friday online session

Deadline:  Thursday 2022-02-24 1200 UTC

[AT117-e][623][POS] Early discussion of integrity issues (ESA)

Scope: Discuss the need for signalling cross-covariance terms in the integrity assistance data, and identify if there are other critical issues that need treatment outside the running CR discussions.

Intended outcome: Report to Wednesday online session

Deadline:  Wednesday 2022-02-23 0200 UTC

[AT117-e][624][POS] Agenda item 5.5 (Huawei)

Scope: Treat documents R2-2202597, R2-2202598, and R2-2202599 and conclude on the CRs.

Intended outcome: Agreed CRs (without CB)

Deadline:  Wednesday 2022-03-02 1000 UTC

[AT117-e][625][POS] Agenda item 6.3.2 (CATT)

Scope: Treat documents R2-2202407 and R2-2202596 and conclude on the CRs.

Intended outcome: Agreed CRs (without CB)

Deadline:  Wednesday 2022-03-02 1000 UTC

[AT117-e][626][POS] Agenda item 6.3.3 (Ericsson)

Scope: Treat documents R2-2202224, R2-2203275, R2-2203277, R2-2203531, and R2-2203368 and conclude on the CRs.

Intended outcome: Agreed CRs (without CB)

Deadline:  Wednesday 2022-03-02 1000 UTC

# 4	EUTRA corrections Rel-15 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.4	Positioning corrections Rel-15 and earlier

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

# 5	Rel-15 WI: New Radio (NR) Access Technology

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

Only essential corrections. Please submit CRs marked “NR_newRAT-Core, TEI16” under one of the below clauses.

Tdoc limitation: AI5 + AI6: 14

## 5.5	Positioning corrections

Corrections to both the stage 2 and stage 3 aspects related to positioning. Stage 2 CRs shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission.  Stage 2 CRs not discussed with the specification rapporteur will not be treated.

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

R2-2202597	Corretion on the object identifier of LPP ASN.1 for R15	Huawei, HiSilicon	CR	Rel-15	37.355	15.2.0	0328	-	F	NR_newRAT-Core

R2-2202598	Corretion on the object identifier of LPP ASN.1 for R16	Huawei, HiSilicon	CR	Rel-16	37.355	16.7.0	0329	-	A	NR_newRAT-Core

R2-2202599	Discussion on the object identifier of LPP ASN.1	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

# 6	Rel-16 NR Work Items

Essential corrections only.

Tdoc Limitation: See common tdoc limitation with AI 5

## 6.3	NR Positioning Support

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item, and non-urgent documents may be postponed to next meeting.

Tdoc Limitation: See tdoc limitation for Agenda Item 6

### 6.3.1	General and Stage 2 corrections

Including incoming LSs, Including impact to 36.305 and 38.305. Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

This agenda item may use a summary document (decision to be made based on submitted tdocs).

R2-2202119	Reply LS to RAN2 on the misalignment in SRS configuration (R3-216009; contact: Samsung)	RAN3	LS in	Rel-16	To:RAN2	Cc:SA2

R2-2202406	Miscellaneous corrections in TS 38.305	CATT	CR	Rel-16	38.305	16.7.0	0085	-	F	NR_pos-Core

### 6.3.2	RRC corrections

Including impact to 36.331, 38.331, and 38.306.

This agenda item may use a summary document (decision to be made based on submitted tdocs).

R2-2202407	Corrections on the description of maxNrofSRS-PosResources-1-r16	CATT	CR	Rel-16	38.331	16.7.0	2890	-	F	NR_pos-Core

R2-2202596	Correction on srs-PosResourceIdList in RRC	Huawei, HiSilicon	CR	Rel-16	38.331	16.7.0	2897	-	F	NR_pos-Core

### 6.3.3	LPP corrections

This agenda item may use a summary document (decision to be made based on submitted tdocs).

R2-2202224	Addition of missing need code for the BDS TGD2 parameter	Lenovo, Motorola Mobility	CR	Rel-16	37.355	16.7.0	0326	-	F	TEI16

R2-2203275	Correction of reference TRP for DL-AoD and Multi-RTT measurement report	Qualcomm Incorporated	CR	Rel-16	37.355	16.7.0	0330	-	F	NR_pos-Core

R2-2203277	Correction to NR-DL-PRS-ResourcesCapability field description	Qualcomm Incorporated	CR	Rel-16	37.355	16.7.0	0331	-	F	NR_pos-Core

R2-2203367	Introducing new high accuracy GAD shape with scalable uncertainty	Ericsson, T-Mobile USA	CR	Rel-16	37.355	16.7.0	0333	-	B	TEI16

- Revised in R2-2203531

R2-2203531	Introducing new high accuracy GAD shape with scalable uncertainty	Ericsson, T-Mobile USA, Qualcomm Incorporated	CR	Rel-16	37.355	16.7.0	0333	1	F	TEI16

R2-2203368	Clarification on LPP segmentation	Ericsson	CR	Rel-16	37.355	16.7.0	0334	-	F	NR_pos-Core

### 6.3.4	MAC corrections

# 7	Rel-16 EUTRA Work Items

Only essential corrections. No documents should be submitted to 7. Please submit to 7.x

## 7.5	LTE Positioning

(NavIC, LTE TEI16 Positioning)

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

# 8	Rel-17 NR Work Items

## 8.7	NR Sidelink relay

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.7.1	Organizational

Incoming LSs, TS updates, rapporteur inputs.  This AI is reserved for rapporteur and organizational inputs.  Documents in this AI do not count towards the tdoc limitation.  For LSes that need action or have impact beyond taking into account by CR rapporteurs: One tdoc by contact company (one company) to address the LS and potential reply is considered Rapporteur Input and may be provided.  Related documents and proposed responses from companies other than the contact company should be submitted to the corresponding technical agenda item (and do count towards the tdoc limitation).

Incoming LSs and related documents

R2-2202127	Reply LS for authorization information for 5G ProSe Layer-3 Remote UE (R3-221202; contact: CATT)	RAN3	LS in	Rel-17	To:SA2, RAN2

R2-2202136	LS on mapping configuration of sidelink relay (R3-221411; contact: Samsung)	RAN3	LS in	Rel-17	To:RAN2

R2-2202952	Discussion on RAN3 LS on mapping configuration of sidelink relay	Samsung	discussion	Rel-17	NR_SL_relay-Core

Work plan and open issues

R2-2202201	Work planning for R17 SL relay	OPPO, CMCC	Work Plan	Rel-17	NR_SL_relay-Core

R2-2202202	Remaining open issues for R17 SL relay	OPPO	discussion	Rel-17	NR_SL_relay-Core

Running CRs and related documents

R2-2202343	Stage 2 CR on Introduction of R17 SL Relay	MediaTek Inc.	CR	Rel-17	38.300	16.8.0	0403	-	B	NR_SL_relay-Core

R2-2203324	38.304 CR for SL relay	Ericsson	CR	Rel-17	38.304	16.7.0	0232	-	B	NR_SL_relay-Core

R2-2203325	Way forward on open issues in 38.304 for SL relay	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2202543	Introduction of Sidelink Relay	Apple	CR	Rel-17	38.321	16.7.0	1194	-	B	NR_SL_relay-Core

R2-2202544	Discussion on remaining issues of MAC CR	Apple	discussion	Rel-17	NR_SL_relay-Core

R2-2202950	Introduction of SL Relay in 38.322	Samsung	CR	Rel-17	38.322	16.2.0	0046	-	B	NR_SL_relay-Core

R2-2202951	Introduction of SL Relay in 38.323	Samsung	CR	Rel-17	38.323	16.6.0	0086	-	B	NR_SL_relay-Core

R2-2202819	Introduction of SL relay	Huawei, HiSilicon	CR	Rel-17	38.331	16.7.0	2910	-	B	NR_SL_relay-Core

R2-2202820	Stage3 open issues handling for RRC CR	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2202276	Running CR for TS 38.351	OPPO	draft TS	Rel-17	38.351	0.4.0	NR_SL_relay-Core

R2-2203519	Draft 38.306 CR for sidelink relay UE capabilities	Qualcomm Incorporated	CR	Rel-17	38.306	16.7.0	0696	-	B	NR_SL_relay-Core

Withdrawn/Not available

R2-2202781	Stage 2 Running CR on Introduction of R17 SL Relay	MediaTek Inc.	CR	Rel-17	38.300	16.8.0	0410	-	B	NR_SL_relay-Core	Withdrawn

### 8.7.2	Open issues

No documents should be submitted to 8.7.2.  Please submit to 8.7.2.x.

#### 8.7.2.1	Control plane procedures

Including connection management, SI delivery, paging, access control for remote UE.

Including report of [Pre117-e][605][Relay] Open issues on relay control plane procedures (Huawei).

Email discussion report

R2-2202822	Summary of [Pre117-e][605][Relay] Open issues on relay control plane procedures	Huawei, HiSilicon	report	Rel-17	NR_SL_relay-Core	Late

Proposals for agreements:

[Easy][23/23] Proposal 3: intraFreqReselection in MIB is not forwarded by relay UE.

[Easy] [23/23] Proposal 4: useT312 cannot be configured to event X1 and X2.

[Easy] [23/23] Proposal 5: useT312 cannot be configured to event Y1 and Y2.

[Easy] [23/23] Proposal 6: PCI is included in suspendConfig (together with C-RNTI).

[Easy] [22/23] Proposal 7: SRAP configuration is not stored in UE Inactive AS context when relay UE/remote UE enters RRC_INACTIVE state.

[Easy] [18/23] Proposal 8: New RLC configuration is introduced to configure Uu/PC5 RLC channel.

[Easy] [20/23] Proposal 9: Regarding how to allocate LCID for PC5 RLC channel of remote UE Uu RBs including SRB2 and DRBs, RAN2 confirms Rel-16 SL method is reused, i.e. LCID is allocated by UE.

Proposals for discussion:

Cause value:

Proposal 1a: On how to set the cause value in msg3 by relay UE when remote UE’s first RRC message triggers relay UE entering RRC_CONNECTED state, RAN2 to down select the following solutions: (A new cause value specific to relay case is to be added in RRCSetupRequest/RRCResumeRequest. No new PC5 signalling. No NAS involvement.)

‐	Solution 2.1: The relay UE should set identical cause value as the one received in remote UE’s msg3 except for remote UE’s path switch or remote UE’s RNAU or remote UE’s RRC reestablishment in which cases the relay UE should use a new value.

‐	Solution 3.2: The relay UE should use a new value irrespective of remote UE’s access cause.

Discussion:

Ericsson think there would be NAS involvement if we have a new failure cause.

CATT support solution 3.2.

Intel wonder how the network will determine what priority to assign the connection request based on the new cause value in solution 3.2.

Apple have a concern with solution 3.2 and think it has more inter-layer impact than other solutions.

Nokia have a concern with the new cause value, since there are limited values available.  If all the requests from the remote UE have the same cause value, they are concerned that the network cannot differentiate e.g. emergency sessions.  They can accept 2.1 but not 3.2.  Ericsson agree with Nokia.

MediaTek support solution 3.2 or an implementation solution but not 2.1.

CATT think both options can work but prefer solution 3.2 for layering; they would like to avoid that the relay UE decodes the remote UE’s message.

OPPO cannot accept 2.1; they understand both solutions require a new cause value, but 2.1 involves more effort for the relay UE, and does not allow the gNB to differentiate between the remote UE’s access and the relay UE’s own access.

Intel wonder about solution 2.1: They understand there was a baseline of no new PC5-RRC signalling to pass the cause value, but wonder if there is any problem with using e.g. RRCReconfigurationSidelink for this purpose; they think we could use existing signalling on PC5 in a way that does not trigger the relay’s connection.  Apple could accept this suggestion.

LG prefer solution 3.2 or implementation, because in solution 2.1 the relay UE needs to decode the remote UE’s message.

Ericsson are OK with Intel’s suggestion, but think solution 3.2 is not acceptable.

Intel clarify that their suggestion differs from solution 2.1 in that the relay UE is not required to decode the message and it does not require a new cause value on Uu; they think the relay UE could choose an existing cause value for the exceptional cases.

OPPO and MediaTek cannot accept Intel’s suggestion.  OPPO see this as not in the spirit of having no new PC5 signalling.  They do not see additional benefit of this idea over solution 2.1.  MediaTek do not accept the PC5-RRC impact and extra use of PC5 resources.

Ericsson think solution 2.1 can be done without new signalling if the remote UE uses the new cause value in its own Msg3 also.  They think the important thing is that the remote and relay UEs use the same cause value.

Xiaomi wonder if we leave it to relay UE implementation, the relay UE would have freedom to set any cause value (e.g. emergency).  They do not think it is acceptable if the relay UE can set the emergency value by implementation.  Ericsson have the same concern.  Apple have the same concern.

OPPO think we relied on implementation in IAB and P1b is a workable compromise.

Show of hands between companies who can accept solutions 2.1 and 3.2:

2.1: 9

3.2: 7

Proposal 1b: If no consensus can be achieved on proposal 1a, it is left to relay UE’s implementation on how to set cause value in its own msg3 when remote UE’s first RRC message triggers relay UE entering RRC_CONNECTED state. (No new signalling. No RAN2 spec impact.)

Cell barring:

Proposal 2: RAN2 to further discuss how to handle the case when the cellBarred in the MIB is set to barred:

‐	[2/22] Option 1: Relay UE forwards cellBar in the discovery message together with cellAccessRelatedInfo.

‐	[8/22] Option 2: Relay UE does not accept new remote UE’s DCR except the UEs accessing for path switch, and release the PC5 connections with other idle/inactive remote UEs.

‐	Option 3: Leave to network and/or relay UE implementation. RAN2 does not pursue specified solutions in Rel-17.

Discussion:

Ericsson would prefer option 3 with no spec impact.  Nokia have a similar view and think the majority was for not forwarding cell barring.

Huawei think only two companies suggested option 3 in the email discussion; they prefer option 1 and think options 2 and 3 both add complexity, but they can accept option 3.

Agreements:

[Easy][23/23] Proposal 3: intraFreqReselection in MIB is not forwarded by relay UE.

[Easy] [23/23] Proposal 4: useT312 cannot be configured to event X1 and X2.

[Easy] [23/23] Proposal 5: useT312 cannot be configured to event Y1 and Y2.

[Easy] [23/23] Proposal 6: PCI is included in suspendConfig (together with C-RNTI).

[Easy] [22/23] Proposal 7: SRAP configuration is not stored in UE Inactive AS context when relay UE/remote UE enters RRC_INACTIVE state.

[Easy] [18/23] Proposal 8: New RLC configuration is introduced to configure Uu/PC5 RLC channel.

[Easy] [20/23] Proposal 9: Regarding how to allocate LCID for PC5 RLC channel of remote UE Uu RBs including SRB2 and DRBs, RAN2 confirms Rel-16 SL method is reused, i.e. LCID is allocated by UE.

A new cause value specific to relay case is to be added in RRCSetupRequest/RRCResumeRequest. No new PC5 signalling. No NAS impact.

It is left to relay UE’s implementation on how to set cause value in its own msg3 when remote UE’s first RRC message triggers relay UE entering RRC_CONNECTED state, with the possible exception of the emergency case (to be discussed offline).

Leave the handling of barred cell to network and relay UE implementation. RAN2 does not pursue specified solutions in Rel-17.

[AT117-e][627][Relay] Remaining issues on control plane (Huawei)

Scope:

- Discuss emergency case for relay UE setting cause value

Intended outcome: Report to CB session

Deadline:  Tuesday 2022-03-01 1200 UTC

Agenda item summary

R2-2203591	Summary of [Pre117-e][609][Relay] Summary of AI 8.7.2.1 Control plane procedures (InterDigital)	InterDigital	discussion	Rel-17	NR_SL_relay-Core

Potentially agreeable:

Recommendation 8:	Update the running CR to disable relay UE sending SI update to the remote UE when the remote UE enters RRC_CONNECTED.

Recommendation 9:	Discuss observations 1-3, 6 from R2-2202471 in the running CR discussion.

Recommendation 10:	Update the running CR to capture that relay reselection can occur following transmission of the RRCSetupRequest and before the connection is established.

Recommendation 12:	Update the running CR to include the PC5-RLC channel configuration and SRAP configuration of the remote UE SRB1 in the RRCSetup message.

Discussion:

OPPO think these recommendations are not needed as agreements and can be directly discussed under the running CR.

Nokia also think it should be handled in the running CR.

ZTE think R12 is separate from the running CR and should be discussed online.

Xiaomi wonder if R8 requires the relay UE to be aware of the remote UE’s RRC state.  InterDigital indicate we had agreed last meeting that the RRC state would not be explicitly informed, but the SI forwarding would be disabled when the remote UE is in RRC_CONNECTED; the proposal is just to capture the existing agreement.  OPPO understand this is already in the running CR.

Agreement:

Include the PC5-RLC channel configuration and SRAP configuration of the remote UE SRB1 in the RRCSetup message.

For further discussion:

Recommendation 1:	RAN2 discuss whether the remote UE provides the relay UE an indication whether to use the same i_s to determine the PO in RRC_INACTIVE as in RRC_IDLE.

Recommendation 3:	A remote UE in RRC_IDLE/RRC_INACTIVE receiving NotificationMessageSidelink message with indicationType as relayUE-CellReselection or relayUE-HO and deciding to keep the PC5-RRC connection assumes that a cell reselection occurs.  RAN2 discusses how to capture the cell ID acquisition at the remote UE in the running CR if the cell change occurs to the relay.

Recommendation 4:	RAN2 discuss whether the relay UE sends notification message to the remote UE upon CHO triggered at the relay UE.

Recommendation 5:	RAN2 discuss whether the relay UE sends notification message to the remote UE upon failed re-establishment.

Recommendation 11:	RAN2 discuss whether the AS layer sends an indication to upper layer for service request upon reception of a message via SL-RLC0

Recommendation 16:	RAN2 discuss whether new triggers for reporting SidelinkUEInformationNR (in addition to legacy triggers) are needed for reporting the source L2 ID by a relay UE.

Discussion:

Ericsson think R16 should be discussed because the existing triggers do not consider relaying.  Qualcomm agree.

InterDigital think R3 is also important, but it can be discussed in the running CR.

OPPO think R16 overlaps with R3.1c in the report of [Pre117-e][604].  For R3, they think the first sentence is unavoidable and the remote UE has no other option.

Xiaomi think R5 is important.

ZTE think R11 can be left to UE implementation.

Kyocera agree with Xiaomi about R5.

InterDigital think we could assign these to the running CR discussion explicitly.

Agreement:

The recommendations above (apart from R12) can be raised in the RRC running CR discussion.

The following documents will not be individually treated

R2-2202184	Remaining issues on control plane procedure of L2 U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2202340	Left issue on NR sidelink relay control plane procedure	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2202344	Discussion on notification of cell reselection and HO of a relay UE	SHARP Corporation	discussion	NR_SL_relay-Core

R2-2202345	Discussion on SRAP config	SHARP Corporation	discussion	NR_SL_relay-Core

R2-2202357	Indication to Upper Layer to Trigger Service Request of L2 Relay	CATT	discussion	Rel-17	NR_SL_relay-Core

R2-2202358	Impacts on RAN of AN Release of Relay UE	CATT	discussion	Rel-17	NR_SL_relay-Core

R2-2202379	Further discussion on RRC connection establishment of remote UE	ZTE, Sanechips	discussion	Rel-17

R2-2202411	Remaining open issues on control plane procedures for L2 U2N relay	Spreadtrum Communications	discussion	Rel-17

R2-2202471	On Capturing the Agreements Related to SI in the RRC CR	InterDigital	discussion	Rel-17	NR_SL_relay-Core

R2-2202472	Cause Value Setting for Connection Establishment for UE to NW Relays	InterDigital	discussion	Rel-17	NR_SL_relay-Core

R2-2202473	Handling the Sidelink Notification Message	InterDigital	discussion	Rel-17	NR_SL_relay-Core

R2-2202567	Further Discussion on L2 CP Issue O6.03	vivo	discussion

R2-2202569	Draft reply LS on establishment/resume cause value on L2 SL Relay	vivo	LS out	To:CT1	Cc:SA2, RAN3

R2-2202847	Reflecting agreement on sidelink resource allocation mode configuration for L2 U2N remote UE in RRC running CR	ASUSTeK	discussion	Rel-17	38.331	NR_SL_relay-Core

R2-2202953	Open issue on SI request over PC5	Samsung	discussion	Rel-17	NR_SL_relay-Core

R2-2203135	Considerations on cause codes	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_SL_relay_enh-Core

R2-2203148	Discussion on connection control open issues	Xiaomi	discussion

R2-2203178	Remaining issues on CP	Lenovo, Motorola Mobility	discussion	NR_SL_relay-Core

R2-2203272	Support of relay UE in RRC_IDLE/INACTIVE state during direct to indirect path switching	Nokia, Nokia Shanghai Bell	discussion	NR_SL_relay_enh-Core	Late

R2-2203306	Setting cause value for Relay UE access	Intel Corporation	discussion	Rel-17	NR_SL_relay-Core

R2-2203308	Discussion on added latency for paging forwarding	Nokia, Nokia Shanghai Bell	discussion	NR_SL_relay-Core

R2-2203326	Remaining issues on control plane for L2 sidelink relay	Ericsson	discussion	Rel-17	NR_SL_relay-Core

#### 8.7.2.2	Service continuity

Service continuity between Uu and relay paths, limited to intra-gNB cases.

Including report of [Pre117-e][603][Relay] Open issues on relay service continuity (CATT)

Email discussion report

R2-2202356	Report of [Pre117-e][603][Relay] Open Issues on Relay Service Continuity (CATT)	CATT	report	Rel-17	NR_SL_relay-Core	Late

Potentially easily agreeable:

Proposal 1: [22/23] RAN2 confirm the working assumption of “The gNB can select a relay UE in any RRC state i.e., RRC_IDLE/INACTIVE/CONNECTED as a target Relay UE when triggering the direct to indirect path switch procedure for the Remote UE by the Remote UE oriented solution, i.e. after receiving the path switch command, Remote UE establishes PC5 link with the Relay UE and sends HO complete message via the Relay UE which will trigger the Relay UE to enter CONNECTED state. ”

Proposal 2: [22/23] For the delivery of RRCReconfigurationComplete message by the Remote UE, default configuration of SL-RLC1 is used for PC5 RLC channel configuration to support RRC_IDLE/INACTIVE target Relay UE for direct to indirect path switch procedure.

Proposal 4: [18/23]The stop condition of the new T304-like timer in Remote UE is upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledge is received from target relay).

Proposal 5: [19/23] When SL-RSRP of the serving relay is not available, it is up to remote UE’s implementation to measure SD-RSRP.

Agreements:

Proposal 1: [22/23] RAN2 confirm the working assumption of “The gNB can select a relay UE in any RRC state i.e., RRC_IDLE/INACTIVE/CONNECTED as a target Relay UE when triggering the direct to indirect path switch procedure for the Remote UE by the Remote UE oriented solution, i.e. after receiving the path switch command, Remote UE establishes PC5 link with the Relay UE and sends HO complete message via the Relay UE which will trigger the Relay UE to enter CONNECTED state. ”

Proposal 2: [22/23] For the delivery of RRCReconfigurationComplete message by the Remote UE, default configuration of SL-RLC1 is used for PC5 RLC channel configuration to support RRC_IDLE/INACTIVE target Relay UE for direct to indirect path switch procedure.

Proposal 4: [18/23]The stop condition of the new T304-like timer in Remote UE is upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledge is received from target relay).

Proposal 5: [19/23] When SL-RSRP of the serving relay is not available, it is up to remote UE’s implementation to measure SD-RSRP.

For further discussion:

Proposal 3: RAN2 to further discuss whether to confirm the working assumption of “UE capability for support by the remote UE of handover to idle/inactive UE.”[13/23] or not [10/23]

Discussion:

OPPO think we should support this as part of the compromise for handover to idle/inactive relay UEs.

Ericsson wonder what the impact for the remote UE to support this would be; they see no need for the capability.  Qualcomm indicate that some issues, e.g. P7 below, occur in the idle/inactive case only and would require new implementation by the remote UE.

Lenovo agree with Ericsson.  Intel agree with Qualcomm.

Ericsson think the remote UE just needs to accept the path switch command.

Proposal 7: If remote UE identifies the target relay UE has changed its serving cell before path switch, remote UE triggers RRC reestablishment as legacy behavior upon expiry of T304 timer. FFS on how the remote UE identifies that the target relay UE has changed.

Discussion:

Qualcomm think this is a failure case where almost all companies agreed.

Lenovo wonder if a UE capability will resolve P7.  Qualcomm understand that the capability can prevent the failure case from occurring for remote UEs that cannot support it.

Xiaomi think P7 is essential to support the path switch to idle/inactive UE, and it is a new UE behaviour, so a capability makes sense.

Ericsson understand that P7 could also happen when the relay UE is connected.  Chair understood the network would not trigger the relay UE handover during a path switch.  Nokia agree with Ericsson.

Xiaomi think we can remove the FFS in P7 because the cell ID is in the discovery message.  Apple think we are not sure how frequently the discovery message is sent; InterDigital have the same concern.  Intel think the UE may enter RRC_CONNECTED after sending the discovery message.

Lenovo think the remote UE has to handle T304 expiry in any case.

Agreements:

Proposal 7 (modified): If remote UE identifies the target relay UE has changed its serving cell before path switch, remote UE triggers RRC reestablishment as legacy behavior upon expiry of T304 timer, at least for the case of relay UE in RRC_IDLE/RRC_INACTIVE. To be determined in [AT117-e][621] how the remote UE identifies that the target relay UE has changed cell and if this can occur in RRC_CONNECTED.

If RRC_CONNECTED and RRC_IDLE/RRC_INACTIVE cases are differentiated, confirm the working assumption of “UE capability for support by the remote UE of handover to idle/inactive UE.” This refers to a capability of the remote UE itself.  If they are not differentiated, check the need for a capability in [AT117-e][621].

Proposal 6: RAN2 to further discuss that whether separate threshold is needed for SD-RSRP measurement for the case that when SL RSRP of the serving relay is not available [9/23]or not [14/23].

Discussion:

OPPO think a single threshold is enough.  Qualcomm agree.

Apple think if there is only a single threshold, the UE will have to treat the SD-RSRP measurement with the same threshold as SL-RSRP.

LG think if power imbalance does not occur, one threshold is enough.

Agreement:

No separate threshold is needed for SD-RSRP measurement for the case that when SL RSRP of the serving relay is not available (UE treats the SD-RSRP measurement with the same threshold as SL-RSRP).

Proposal 8: When the new T304-like timer is stopped in remote UE but the direct to indirect path switch fails due to IDLE/INACTIVE relay UE fails to establish the connection on Uu hop of indirect path, a similar handling as relay UE’s HO/Uu RLF, i.e.:

-Upon relay UE receives RRCReject or experiences other connection establishment/resume failure, it either triggers PC5-S release or sends notification message indicating Uu RRC connection failure to remote UE.

-PC5-S release or notification message shall trigger remote UE’s RRC reestablishment. But in case of notification, remote UE can choose to keep the current PC5 connection with this target relay, or release the PC5 connection and reselect to other relay.

Agreement:

Proposal 8 above will be handled in [AT117-e][621].

Other documents

R2-2202185	 Remaining issues on service continuity of L2 U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2202341	Left issue on NR sidelink relay service continuity	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2202380	Remaining issues on service continuity	ZTE, Sanechips	discussion	Rel-17

R2-2202545	Discussion on remaining issues for direct-to-indirect path switch	Apple	discussion	Rel-17	NR_SL_relay-Core

R2-2202584	Path switching in L2 U2N relay case	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2202738	RRC corrections on path switch	NEC Corporation	discussion	Rel-17	NR_SL_relay_enh-Core

R2-2202821	Stage3 issue on NCGI reporting in measurement result	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2202848	Potential issues on multiple PDU sessions handling during U2N direct to indirect path switching	ASUSTeK	discussion	Rel-17	NR_SL_relay-Core

R2-2203202	Service continuity open issues in L2 NR sidelink relay	Sony	discussion	Rel-17	NR_SL_relay-Core

#### 8.7.2.3	Adaptation layer design

Including bearer mapping, remote UE identification, security aspects if any.

Including report of [Pre117-e][604][Relay] Open issues on relay adaptation layer (OPPO)

Email discussion report

R2-2202200	Summary of [Pre117-e][604][Relay] Open issues on relay adaptation layer (OPPO)	OPPO	report	Rel-17	NR_SL_relay-Core	Late

Unanimous:

Recommendation 1 [19/19]: RAN2 confirm the working assumption of ” Remote local UE ID is 8 bits.”

Recommendation 2 [19/19]: RAN2 confirm the working assumption of ” Remote UE ID is always present in PC5 adaptation layer header.”

Recommendation 3-1a-1 [19/19]: L2 relay UE report source L2 ID of relay-related discovery transmission to gNB.

Recommendation 4 [19/19]: When a SRAP Data PDU that contains a UE ID or BEARER ID which is not included in sl-SRAP-Config-Remote (for Remote UE) or sl-SRAP-Config-Relay (for Relay UE) is received, the SRAP entity shall discard the received SRAP Data PDU.

With clear majority (>80% support)

Recommendation 3-2a [18/19]: L2-remote, L2-relay, L3-remote and L3-relay UE report destination L2 ID for discovery transmission. L2-relay-UE, L3-remote-UE and L3-relay-UE report (i.e., except L2-remote-UE) destination L2 ID for established PC5 link for relaying.

Recommendation 3-2b [17/19]: In SUI, when reporting a particular destination L2 ID associated with discovery, RAN2 not pursue explicit relay type indication to differentiate between relay-discovery and non-relay-discovery.

Recommendation 3-2c [16/19]: For the destination L2 ID reporting for discovery and for established PC5 link for relay, add a new IE (i.e., instead of reusing the existing field sl-DestinationIdentity).

Recommendation 3-2e [17/19]: L2 relay-UE not report the updated ID of L2-remote UE of the established PC5 link.

Recommendation 5 [18/19]: For RRC_INACTIVE / RRC_IDLE L2-Relay UE, it gets local ID configuration for L2-remote UE during direct-to-indirect switching from network configuration on sl-LocalIdentity-r17.

Recommendation 6 [17/19]: In order for L2-relay UE to differentiate between SRAP data PDU for SRB and DRB if the BEARER ID is 0/1/2/3, for a SRAP Data PDU received from PC5 (or Uu) via sl-Egress-RLC-Channel-Uu (or via sl-Egress-RLC-Channel-PC5), L2-relay UE can know whether it is SRB or DRB based on the associated sl-RemoteUE-RB-Identity.

Discussion:

Huawei think on recommendation 3-2b, if we do not have this differentiation, the network cannot know how to configure the threshold.  OPPO understand this was discussed by email and the network has to blindly provide both the relay- and non-relay-related parameters.

ZTE think the wording of R2 should be ”local UE ID”.

Apple wonder on R3-2e, if it results in the remote UE having two IDs after the change, potentially confusing the gNB.  ZTE agree with Apple. OPPO indicate that the relay UE anyway has to maintain the mapping for both IDs during the transition period, so from the gNB perspective there is no effort.

Apple think we agreed that the remote UE reports the source L2ID to the gNB directly, and R3-2e seems not consistent with that.  OPPO think whether the source ID is reported for the PC5 link is a separate proposal.  Ericsson agree with OPPO.

Xiaomi think the WA is not correctly copied in R2.  We said ”RAN2 does not pursue procedural impact for handling it beyond P6 of” a document from the previous meeting.

Huawei wonder in R3-2b, how the gNB can know which kind of authorisation to do (relay or non-relay).  OPPO understand the authorisation has separate IEs for the two cases, and the gNB can determine on this basis.

Agreements:

Recommendation 1 [19/19]: RAN2 confirm the working assumption of ”Remote local UE ID is 8 bits.”

Recommendation 2 [19/19] (modified): RAN2 confirm the working assumption of ”Remote UE ID is always present in PC5 adaptation layer header.”  This refers to the remote local UE ID.  No impact to RRC signalling (as indicated in the original WA).

Recommendation 3-1a-1 [19/19]: L2 relay UE report source L2 ID of relay-related discovery transmission to gNB.

Recommendation 4 [19/19]: When a SRAP Data PDU that contains a UE ID or BEARER ID which is not included in sl-SRAP-Config-Remote (for Remote UE) or sl-SRAP-Config-Relay (for Relay UE) is received, the SRAP entity shall discard the received SRAP Data PDU.

Recommendation 3-2a [18/19]: L2-remote, L2-relay, L3-remote and L3-relay UE report destination L2 ID for discovery transmission. L2-relay-UE, L3-remote-UE and L3-relay-UE report (i.e., except L2-remote-UE) destination L2 ID for established PC5 link for relaying.

Recommendation 3-2c [16/19]: For the destination L2 ID reporting for discovery and for established PC5 link for relay, add a new IE (i.e., instead of reusing the existing field sl-DestinationIdentity).

Recommendation 3-2e [17/19]: L2 relay-UE not report the updated ID of L2-remote UE of the established PC5 link.

Recommendation 5 [18/19]: For RRC_INACTIVE / RRC_IDLE L2-Relay UE, it gets local ID configuration for L2-remote UE during direct-to-indirect switching from network configuration on sl-LocalIdentity-r17.

Recommendation 6 [17/19]: In order for L2-relay UE to differentiate between SRAP data PDU for SRB and DRB if the BEARER ID is 0/1/2/3, for a SRAP Data PDU received from PC5 (or Uu) via sl-Egress-RLC-Channel-Uu (or via sl-Egress-RLC-Channel-PC5), L2-relay UE can know whether

it is SRB or DRB based on the associated sl-RemoteUE-RB-Identity.

Recommendation 3-2b above to be discussed in the RRC running CR discussion.

For further discussion

Recommendation 3-1a-2 [?/19]: RAN2 discuss whether to report 1) source L2 ID to be used to establish PC5 link with L2 relay UE (i.e., used to send DCR message) or 2) source L2 ID of relay-related discovery transmission to gNB (by assuming it is also the source L2 ID used to send DCR message if model-B discovery is used). And if the latter one is adopted, RAN2 discuss how to handle the case where model-A discovery is used by relay UE.

Recommendation 3-1c [?/19]: Relying RRC running-CR discussion on how to specify the initiation condition for source L2 ID reporting, at least including when source L2 ID is updated.

Recommendation  3-2d: When L2-relay UE report destination L2 ID of peer UE (i.e., ID of L2-remote UE), RAN2 discuss whether to report an indicator on whether local ID allocation is required [2/6] or not [4/6].

Agreement:

Recommendations 3-1a-2, and 3-2d above to be discussed in the RRC running CR discussion.

Other documents

R2-2202392	Discussion on SRAP for L2 U2N relay	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2202429	Remaining issues of the adaptation layer	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2202675	Remaining issue on sidelink adaptation layer 	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2202897	Discussion on UE's L2 ID	Sharp	discussion

R2-2203172	SRAP - miscellaneous issues	Samsung Electronics GmbH	discussion

Withdrawn/Not available

R2-2202854	SRAP header format design	CMCC	discussion	Rel-17	NR_SL_relay-Core	Withdrawn

#### 8.7.2.4	QoS

Mechanisms for E2E QoS management.

Including report of [Pre117-e][602][Relay] Open issues on relay QoS (Samsung)

Email discussion report

R2-2202955	Summary of [Pre117-e][602][Relay] Open issues on relay QoS (Samsung)	Samsung	discussion	Rel-17	NR_SL_relay-Core	Late

Other documents

R2-2202339	Left issue on QoS for layer 2 relay	OPPO	discussion	Rel-17	NR_SL_relay-Core

R2-2202381	Miscellaneous issues on bearer mapping and QoS	ZTE, Sanechips	discussion	Rel-17

R2-2202428	Aspects for QoS management with SL relay	Ericsson	discussion	Rel-17	NR_SL_relay-Core

R2-2202954	Open issue on new code-point for address resolution protocol (ARP) in PDCP SDU type	Samsung	discussion	Rel-17	NR_SL_relay-Core

#### 8.7.2.5	Discovery and re/selection

Including 5G ProSe Direct Discovery for the non-relaying case.  Re-using LTE discovery and re/selection as baseline.

Including report of [Pre117-e][601][Relay] Discovery and relay re/selection (ZTE)

Email discussion report

R2-2202378	Summary of [Pre117-e][601][Relay] Discovery and relay re-selection (ZTE)	ZTE, Sanechips	discussion	Rel-17	Late

Other documents

R2-2202186	Remaining issues on discovery and relay (re)selection	Qualcomm Incorporated	discussion	NR_SL_relay-Core

R2-2202412	Remaining issues on NotificationMessageSidelink message	Spreadtrum Communications	discussion	Rel-17

R2-2202568	Remaining issues on Discovery and Relay (re)selection	vivo	discussion

R2-2202585	Discovery and Relay (re)selection in L2 and L3 relay case	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2202849	Issues on priority between PC5 signalling and SL discovery	ASUSTeK	discussion	Rel-17	38.321	NR_SL_relay-Core

R2-2203233	Discussion on relay re-selection and discovery	Huawei, HiSilicon	discussion	Rel-17	NR_SL_relay-Core

R2-2203506	Sidelink discovery support as indicated within SIB12	Beijing Xiaomi Mobile Software	discussion	Rel-17

#### 8.7.2.6	UE capabilities

Including report of [Pre117-e][606][Relay] Open issues on relay UE capabilities (Qualcomm)

Email discussion report

R2-2202676	Summary report of offline606 - Open issues on relay UE capabilities (Qualcomm)	Qualcomm Incorporated	discussion	NR_SL_relay-Core	Late

Other documents

R2-2202359	Further Discussion on UE Capability	CATT	discussion	Rel-17	NR_SL_relay-Core

### 8.7.3	Other

Any other topics on NR sidelink relay.

## 8.11	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

Time budget: 2 TU

Tdoc Limitation: 3 tdocs

### 8.11.1	Organizational

Rapporteur input. Incoming LS etc. This AI is reserved for rapporteur and organizational inputs; documents in this AI do not count towards the tdoc limitation.  For LSes that need action or have impact beyond taking into account by CR rapporteurs: One tdoc by contact company (one company) to address the LS and potential reply is considered Rapporteur Input and may be provided.  Related documents and proposed responses from companies other than the contact company should be submitted to the corresponding technical agenda item (and do count towards the tdoc limitation).

Including report of [Pre117-e][613][POS] RAN1 parameter list impact to RRC running CR (Ericsson)

Including report of [Pre117-e][614][POS] Issues requiring RAN1 input (Intel)

Incoming LSs

R2-2202164	LS on SRS for multi-RTT positioning (R4-2202680; contact: Huawei)	RAN4	LS in	Rel-17	To:RAN1	Cc:RAN2, RAN3

R2-2202165	Reply LS on reporting of the Tx TEG association information (R4-2202685; contact: Huawei)	RAN4	LS in	Rel-17	To:RAN1, RAN2	Cc:RAN3

R2-2202166	LS on DRX cycle used in PRS measurement in RRC_INACTIVE state (R4-2202686; contact: Qualcomm)	RAN4	LS in	Rel-17	To:RAN2, RAN3	Cc:RAN1

R2-2202169	Reply LS on reporting and definition of DL PRS path RSRP (R4-2202780; contact: Nokia)	RAN4	LS in	Rel-17	To:RAN1, RAN2

Open issues list

R2-2202488	Open issues list on Rel-17 positioning WI	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core

Email discussion reports and related documents

R2-2202492	Report of [Pre117-e][614][POS] Issues requiring RAN1 input (Intel)	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core	Late

R2-2202493	Draft LS on issues requiring RAN1 input	Intel Corporation	LS out	Rel-17	NR_pos_enh-Core	To:RAN1	Late

R2-2203363	Report on RAN1 parameter list impact to RRC running CR	Ericsson	discussion	Rel-17	Late

Running CRs and related documents

R2-2202489	Open issues on stage 2 running CR	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core

R2-2202490	Running 38.305 CR for Positioning WI on RAT dependent positioning methods	Intel Corporation	draftCR	Rel-17	38.305	16.7.0	B	NR_pos_enh-Core

R2-2202861	Running CR of 36.305 for GNSS Positioning Integrity	InterDigital, Inc.	draftCR	Rel-17	36.305	16.4.0	B	NR_pos_enh-Core

R2-2202862	Running CR of 38.305 for GNSS Positioning Integrity	InterDigital, Inc.	draftCR	Rel-17	38.305	16.7.0	B	NR_pos_enh-Core

R2-2203310	Running LPP CR for NR positioning enhancements	Qualcomm Incorporated	draftCR	Rel-17	37.355	16.7.0	B	NR_pos_enh-Core

R2-2203362	RAN1 parameter list impact to RRC running CR	Ericsson	draftCR	Rel-17	38.331	16.7.0	B	NR_pos_enh-Core	Late

Merged CRs

R2-2202405	Introduction of B2a and B3I signal in BDS system and GNSS Positioning Integrity	CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, HiSilicon, Intel Corporation, ZTE Corporation, CBN, vivo, OPPO, Lenovo, MediaTek Inc, Spreadtrum Communications, Xiaomi.	CR	Rel-17	36.305	16.4.0	0107	-	B	NR_pos_enh-Core

R2-2202491	38.305 CR for Positioning WI	Intel Corporation	CR	Rel-17	38.305	16.7.0	0086	-	B	NR_pos_enh-Core	Late

R2-2203315	Introduction of R17 Positioning Enhancements in LPP	Qualcomm Incorporated	CR	Rel-17	37.355	16.7.0	0332	-	B	NR_pos_enh-Core	Late

R2-2203364	Introduction of Enhanced Positioning feature	Ericsson	CR	Rel-17	38.331	16.7.0	2952	-	B	NR_pos_enh-Core

R2-2202605	Introduction of R17 PositioningEnh in MAC spec	Huawei, HiSilicon	CR	Rel-17	38.321	16.7.0	1197	-	B	NR_pos_enh-Core

R2-2202606	Introduction of R17 PositioningEnh in LTE RRC spec	Huawei, HiSilicon	CR	Rel-17	36.331	16.7.0	4762	-	B	NR_pos_enh-Core

### 8.11.2	Open issues

No documents should be submitted to 8.11.2.  Please submit to 8.11.2.x.

#### 8.11.2.1	Latency enhancements

Enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods.

Including report of [Pre117-e][607][POS] Open issues on positioning latency enhancements (Huawei)

Email report

R2-2202604	Summary of [Pre117-e][607][POS] Open issues on positioning latency enhancements (Huawei)	Huawei, HiSilicon	discussion	Rel-17	NR_pos_enh-Core	Late

Agenda item summary

R2-2203592	Summary of AI 8.11.2.1	Apple	discussion

The following documents will not be individually treated

R2-2202408	Discussion and TP on areaID for Latency enhancements	CATT	discussion	Rel-17	NR_pos_enh-Core

R2-2202487	On Latency Reduction open issues	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core

R2-2202592	On remaining issues for latency improvements	Apple	discussion

R2-2202603	Remaining issues on latency and accuracy enhacnement	Huawei, HiSilicon	discussion	Rel-17	NR_pos_enh-Core

R2-2202858	Remaining Issues on Latency Reduction	InterDigital, Inc.	discussion	Rel-17	NR_pos_enh-Core

R2-2202922	MAC CE for pre-MG (de)activation request	Samsung	discussion	Rel-17	NR_pos_enh-Core

R2-2202930	Remaining issue on positioning latency reduction	Xiaomi	discussion

R2-2203042	Way forward for preconfigured assistance data	Fraunhofer IIS; Fraunhofer HHI; Ericsson;	discussion

R2-2203088	Discussion on latency enhancement	vivo	discussion	Rel-17	NR_pos_enh-Core

R2-2203181	Discussion on open issues of positioning latency enhancements	ZTE	discussion

R2-2203204	Considerations on positioning measurement report latency	Sony	discussion	Rel-17	NR_pos_enh-Core

R2-2203211	Discussion of positioning latency enhancement open issues	OPPO	discussion	Rel-17	NR_ENDC_SON_MDT_enh-Core

R2-2203360	TP on RRC Impacts and MAC CE design	Ericsson	discussion	Rel-17

#### 8.11.2.2	RRC_INACTIVE

Methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions.  UL and DL+UL NR positioning methods and gNB positioning measurements for UEs in RRC_INACTIVE are treated at lower priority.

Including report of [Pre117-e][609][POS] Open issues on positioning in RRC_INACTIVE (InterDigital)

Email report

R2-2203524	Email discussion Report on [Pre117-e][609][POS] Open issues on positioning in RRC_INACTIVE (InterDigital)	InterDigital, Inc.	discussion	Rel-17	NR_pos_enh-Core

Other documents

R2-2202338	Discussion on remaining issues for Positioning in RRC_INACTIVE state	OPPO	discussion	Rel-17	NR_pos_enh-Core

R2-2202601	Remaining Issues on RRC_INACTIVE Positioning	Huawei, HiSilicon	discussion	Rel-17	NR_pos_enh-Core

R2-2202602	Draft LS on Positioning in RRC_INACTIVE State	Huawei, HiSilicon	LS out	Rel-17	NR_pos_enh-Core	To:SA2	Cc:RAN3

R2-2203089	Discussion on positioning in RRC_INACTIVE	vivo	discussion	Rel-17	NR_pos_enh-Core

R2-2203091	Consideration on the configuration of UL positioning in RRC_INACTIVE	CATT	discussion	Rel-17	NR_pos_enh-Core

R2-2203180	Discussion on UL positioning configuration in RRC_INACTIVE	ZTE	discussion

R2-2203443	Remaining issues for positioning of UEs in RRC_INACTIVE State	Qualcomm Incorporated	discussion

R2-2203444	[draft] LS on Positioning in RRC_INACTIVE State	Qualcomm Incorporated	LS out	Rel-17	NR_pos_enh	R2-2200961	To:SA2	Cc:RAN3

R2-2203445	Capturing RRC impacts for RAT dependent Positioning	Ericsson	draftCR	Rel-17	38.331	16.7.0	B	NR_pos_enh-Core	R2-2202048

#### 8.11.2.3	On-demand PRS

Specify UE-initiated and LMF-initiated on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions.

Including report of [Pre117-e][608][POS] Open issues on on-demand PRS (Lenovo)

Email report

R2-2202236	Report of [Pre117-e][608][POS] Open issues on on-demand PRS 	Lenovo, Motorola Mobility	discussion	Rel-17	NR_pos_enh-Core	Late

Other documents

R2-2202337	Discussion on remaining issues for on-demand DL-PRS	OPPO	discussion	Rel-17	NR_pos_enh-Core

R2-2202409	Discussion on the remaining issues of on-demand PRS	CATT	discussion

R2-2202859	Remaining Issues for On-demand PRS 	InterDigital, Inc.	discussion	Rel-17	NR_pos_enh-Core

R2-2203169	Remaining issues for the On demand DL PRS	Samsung R&D Institute UK	discussion

R2-2203463	On-demand PRS Open Issues	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_pos_enh-Core

#### 8.11.2.4	GNSS positioning integrity

Signalling and procedures to support GNSS positioning integrity determination.

Including report of [Pre117-e][610][POS] Open issues on GNSS positioning integrity (ESA)

Email report

R2-2203525	[Pre117-e][610][POS] Open issues on GNSS positioning integrity (ESA)	ESA	discussion	Rel-17	NR_pos_enh-Core

Other documents

R2-2203034	UE-aided detection of threat to GNSS systems and assistance data signaling	Fraunhofer IIS; Fraunhofer HHI; Ericsson; ESA	discussion	R2-2200955

R2-2203090	Discussion on GNSS positioning integrity	vivo	discussion	Rel-17	NR_pos_enh-Core

R2-2203199	Reporting of GNSS Positioning Integrity Result	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_pos_enh

R2-2203359	On remaining GNSS Integrity open issues	Ericsson	discussion	Rel-17

#### 8.11.2.5	A-GNSS enhancements

Including support of BDS B2a and B3I signals and support of NavIC.

Running CRs

R2-2202402	Introduction of B2a and B3I signal in BDS system in A-GNSS	CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, HiSilicon, Intel Corporation, ZTE Corporation, CBN, vivo, OPPO, Lenovo, MediaTek Inc, Spreadtrum Communications, Xiaomi.	CR	Rel-17	37.355	16.7.0	0327	-	B	NR_pos_enh-Core	R2-2200298

R2-2202403	Introduction of B2a and B3I signal in BDS system in A-GNSS	CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, HiSilicon, Intel Corporation, ZTE Corporation, CBN, vivo, OPPO, Lenovo, MediaTek Inc, Spreadtrum Communications, Xiaomi.	CR	Rel-17	36.305	16.4.0	0106	-	B	NR_pos_enh-Core	R2-2109485

R2-2202404	Introduction of B2a and B3I signal in BDS system in A-GNSS	CATT, CAICT, CMCC, China Telecom, China Unicom, Huawei, HiSilicon, Intel Corporation, ZTE Corporation, CBN, vivo, OPPO, Lenovo, MediaTek Inc, Spreadtrum Communications, Xiaomi.	CR	Rel-17	38.305	16.7.0	0084	-	B	NR_pos_enh-Core	R2-2109485

R2-2202607	Draft running CR for stage2 spec for NAVIC in R17 positioning	Huawei, HiSilicon	draftCR	Rel-17	38.305	16.7.0	B	NR_pos_enh-Core

R2-2203710	Introduction of NavIC for broadcast support	Ericsson	draftCR	Rel-17	38.331	16.7.0	B	NR_pos_enh-Core

#### 8.11.2.6	Accuracy enhancements

Input on the accuracy enhancement objectives led by RAN1.

Including report of [Pre117-e][611][POS] Open issues on positioning accuracy enhancements (CATT)

Email report

R2-2202410	Report of [Pre117-e][611][POS] Open issues on positioning accuracy enhancements (CATT)	CATT	discussion	Late

Other documents

R2-2202593	On UE Tx TEG association for UL-TDOA via RRC	Apple	discussion

R2-2202860	Remaining Issues for Accuracy Enhancements 	InterDigital, Inc.	discussion	Rel-17	NR_pos_enh-Core

R2-2203205	Considerations on Timing Error aspects	Sony	discussion	Rel-17	NR_pos_enh-Core

R2-2203361	LPP Remaining Issues on Accuracy enhancements and On-Demand PRS	Ericsson	discussion	Rel-17

R2-2203462	Timing Error Group (TEG) definition	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_pos_enh-Core

#### 8.11.2.7	UE capabilities

Including report of [Pre117-e][612][POS] Open issues on positioning UE capabilities (Intel)

Email report

R2-2202494	Report of [Pre117-e][612][POS] Open issues on positioning UE capabilities (Intel)	Intel Corporation	discussion	Rel-17	NR_pos_enh-Core	Late

Running CRs

R2-2202495	Running 331 CR for Positioning UE capabilities	Intel Corporation	draftCR	Rel-17	38.331	16.7.0	B	NR_pos_enh-Core	Late

R2-2202496	Running 306 CR for Positioning UE capabilities	Intel Corporation	draftCR	Rel-17	38.306	16.7.0	B	NR_pos_enh-Core	Late

### 8.11.3	Other

Any other topics on NR positioning enhancements.