3GPP TSG-RAN WG2 Meeting #117-e					R2-220xxxx

Electronic Meeting, February 21 – March 3, 2022

Agenda item:	5.4.1	NR RRC

Source: 	Intel Corporation (Rapporteur)

Title: 	[AT117-e][028][NR15] RRC misc II (Intel)

Document for:		Report

Introduction

This document captures the discussion and report on the following offline discussion:

[AT117-e][028][NR15] RRC misc II (Intel)

Scope: Treat R2-2202637, R2-2202638, R2-2202639, R2-2203327, R2-2203328

Ph1 Determine agreeable parts, Ph2 For agreeable parts, progress CRs

Intended outcome: Report, Agreed CRs.

Deadline: Schedule 1

A first round with Deadline for comments W1 Thur Feb 24th 1200 UTC to settle scope what is agreeable etc

A Final round with Final deadline W2 Wed March 2nd 1200 UTC to settle details / agree CRs etc.

Please provide the contact information in the following Table:

| Company | Point of contact | Email address |
| --- | --- | --- |
| Qualcomm | Mouaffac | mambriss@qti.qualcomm.com |
| Ericsson | Antonino Orsino | antonino.orsino@ericsson.com |
| Huawei, HiSilicon | Tong Sha | shatong3@hisilicon.com |
| CATT | Rui Zhou | Zhourui@catt.cn |
| Intel | Sudeep Palat | Sudeep.k.palat@intelcom |
| Nokia | Amaanat Ali | amaanat.ali@nokiacom |
| ZTE | Yu Liu | liu.yu3@zte.com.cn |
| Apple | Naveen Palle | naveen.palle@apple.com |
| Samsung | Sangbum Kim | sb07.kim@samsung.com |

Discussion

NCC handling for re-establishment and Resume

- Scope: Treat R2-2202637, R2-2202638, R2-2202639

R2-2202637	Issues with use of NCC for KgNB derivation during re-establishment and Resume procedure	Intel Corporation	discussion	Rel-15	38.331	NR_newRAT-Core

R2-2202638	Correction of NCC storage during re-establishment and Resume	Intel Corporation	CR	Rel-15	38.331	15.16.0	2899	-	F	NR_newRAT-Core

R2-2202639	Correction of NCC storage during re-establishment and Resume	Intel Corporation	CR	Rel-16	38.331	16.7.0	2900	-	A	NR_newRAT-Core

These documents/CRs observe that:

The current procedural text for NCC storage and key derivation in re-establishment procedure is incorrect and result in wrong KgNBs.

The current specification text related to the storage and usage of NCC during Resume procedure is inconsistent and incorrect and can result in wrong KgNB during Handover, Reestablishment or Resume procedure and failure of these procedures.

And proposes:

Proposal #1: Correct 38.331 procedural text for the re-establishment with the TP shown above (i.e. storing the NCC received in the RRCReestablishment message after updating the KgNB key with the received NCC).

Proposal #2: Discuss if the above specification corrections regarding handling of NCC for Resume procedure as captured on corresponding CR R2-2202638 are essential and if so for which release.

The CRs proposes to correct the re-establishment and Resume procedures as summarised in the cover page:

- The storage of NCC is moved to after key generation in the procedural text [for re-establishment]

- nextHopChainingCount received in RRC Release message is stored in UE Inactive context.  The value of nextHopChainingCount used for the current keys is stored on receipt of Resume message and also on receipt of RRC Release in response to a ResumeRequest.  It is clarified that the value of nextHopChainingCount received in RRCRelease message and stored in UE Inactive context is used for key derivation during ResumeRequest procedure.

Q1: Please provide your company views on the proposed corrections – whether the corrections are useful/needed/Not essential and if needed, for which release.

| Company | Correction to re-establishment useful/needed/Not essential | Corrections to Resume useful/needed/Not essential | Comments (including, if needed, how to capture/which release to capture) |
| --- | --- | --- | --- |
| QCOM | Not needed | Not needed | Already devices are in the field with no interoperability issue. besides how UE stores NH and how to derive horizontal and vertical keys in reestablishment and resume are clearly defined in 33.501 |
| Ericsson | Useful | Needed | For the reestablishment case, we think that for consistency this change makes things clear in the spec but also for the UE implementation. Also, if all the UEs already have implemented the procedure correctly, this change should not be very critical. For the resume case, if a UE implements the specification line by line, it is evident that is not clear how UE stores NH and how to derive horizontal and vertical keys. In 33.501 it is described how the UE should perform horizontal and vertical key derivation but not how the signalling should be modelled. All in all, we think that there is a hole in the current RRC specification and is better to fix it. |
| Huawei, HiSilicon | Not essential | Not essential | Agree with QCOM. The handle of NCC is a basic operation, and it is unnecessary to clarify since there is no interoperability issues so far. |
| CATT | Not essential | Not essential | Agree with QCOM. We believe a correct UE implementation can store the new NCC correctly, without flushing the old NCC. |
| Intel | Useful | Needed | Apart from the justifications provided in the discussion document, it is also useful to get the basic framework specifications correct to help with future specification work involving Resume and security handling. The gaps and errors in the current specs make evaluation and update of the specs for future features difficult. |
| Nokia | Useful | Needed | We also note that checking from LTE side it really seems that this is an issue but hopefully as UE vendors above mention that they have realized this and implemented correctly. In the cover page of the CR, we should ideally not have an interoperability issue with R15, R16 as all UE vendors would have implemented this correctly. If this is the case, we would be okay for the change but mentioning that there is no interop issue. |
| ZTE | Not essential | Not essential | Agree with QCOM and CATT. |
| Apple | Not essential. | Not essential. | Same views are CATT and Qualcomm |
| Samsung | Needed | Needed | Ambiguity exists for both cases. It is reasonable to fix it, even though we see no critical problem in real networks, e.g. due to smart UE implementation. |

Summary:

Q2: Please provide comments, if any, on the technical details of the proposed corrections.

| Company | Comments, if any, on the technical details of the corrections |
| --- | --- |

Summary:

Correction on Full Configuration regarding reconfigWithSync

- Scope: Treat R2-2203327, R2-2203328

R2-2203327	Correction on Full configuration	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.16.0	2941	-	F	NR_newRAT-Core

R2-2203328	Correction on Full configuration(R16)	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.7.0	2942	-	A	NR_newRAT-Core

These CRs propose that the current text on 5.3.5.11 Full configuration:

is incorrect, because the fullConfig is applicable to all cases of  reconfiguration with sync

So we suggest to delete the words ‘(i.e., SpCell change)’ above.

And proposes the following correction:

1>	if the spCellConfig in the masterCellGroup includes the reconfigurationWithSync:

Q3: Please provide company views on the proposed correction - whether the correction is useful/needed/Not essential and if needed, for which release.

| Company | Correction is useful/needed/Not essential | Comments (including, if needed, how to capture/which release to capture) |
| --- | --- | --- |
| QCOM | - | The change is correct … will go with majority |
| Ericsson | Not essential | This change is not essential. If majority wants to go for it we can have it in the Rapporteur’s CR. |
| Huawei, HiSilicon | Not needed | According the field description below, fullconfiguration only applied to handover scenario (including resume and re-establishment which is like handover). fullConfig Indicates that the full configuration option is applicable for the RRCReconfiguration message for intra-system intra-RAT HO. For inter-RAT HO from E-UTRA to NR, fullConfig indicates whether or not delta signalling of SDAP/PDCP from source RAT is applicable. This field is absent if any DAPS bearer is configured or when the RRCReconfiguration message is transmitted on SRB3, and in an RRCReconfiguration message for SCG contained in another RRCReconfiguration message (or RRCConnectionReconfiguration message, see TS 36.331 [10]) transmitted on SRB1. |
| CATT | useful | The change seems OK as indeed there is case that IE “reconfigurationWithSync” is included but not for SpCell change. |
| Intel | Useful | No strong view and OK to go with majority. We had previously considered reconfigWithSync as a HO (intra or inter) and so the current text though could be a bit misleading is not incorrect. |
| Nokia | Useful, see comments | We agree with the case described here and would support this as the i.e., seems to indeed exclude other use cases. Usually anything in parentheses is not requirement thus deleting does not change anything. Case is purely editorial so we propose rapporteur CR only rather than such individual one. |
| ZTE | Useful | @Huawei: The presence condition of the field fullConfig is “The field is mandatory present in case of inter-system handover from E-UTRA/EPC to NR. It is optionally present, Need N, during reconfiguration with sync and also in first reconfiguration after reestablishment; or for intra-system handover from E-UTRA/5GC to NR. It is absent otherwise”, i.e. the fullConfig is applicable to all cases of reconfiguration with sync, so we think the CRs are needed. |
| Apple | Useful, we are not very strong on having this and we can go with majority. |  |
| Samsung | Useful | It’s minor and useful. We agree to have it in the Rapporteur’s CR |

Summary:

Q4: Please provide comments, if any, on the technical details of the proposed correction.

| Company | Comments, if any, on the technical details of the correction |
| --- | --- |

Summary:

Summary and proposals

[TBD]