3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-220xxxx

Online, 21 February – 03 March 2022

Agenda item:	5.4.1

Source:	Nokia (Rapporteur)

Title:	Offline [AT117-e][029][NR15] RRC Inter-Node Signalling (Nokia)

WID/SID:	NR_newRAT-Core

Document for:	Discussion and Decision

1	Introduction

This document is the report of the following email discussion:

[AT117-e][029][NR15] RRC Inter-Node Signalling (Nokia)

Scope: Treat R2-2202121, R2-2203500, R2-2203501, R2-2202806, R2-2202807, R2-2202808, R2-2202123, R2-2203321, R2-2203322. Ph1 Determine agreeable parts, Ph2 For agreeable parts, progress CRs, (reply LSes out only if needed).

Intended outcome: Report, Agreed CRs

Deadline: Schedule 1

Inter-MN HO without SN change

[1] R2-2202121	Reply LS on inter-MN handover without SN change (R3-216165; contact: Huawei)	RAN3	LS in	Rel-15	To:RAN2

Moved from 5.1

[2] R2-2203500	Clarification on inter-MN handover without SN change (R15)	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, Samsung	CR	Rel-15	37.340	15.15.0	0299	-	F	NR_newRAT-Core

[3] R2-2203501	Clarification on inter-MN handover without SN change (R16)	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Ericsson, ZTE Corporation, Samsung	CR	Rel-16	37.340	16.8.0	0300	-	A	NR_newRAT-Core

[4] R2-2202806	Signalling in inter-MN HO without SN change	NEC	discussion	Rel-15	NR_newRAT-Core

[5] R2-2202807	Clarification on inter-MN handover without SN change	NEC	CR	Rel-15	38.331	15.16.0	2907	-	F	NR_newRAT-Core

[6] R2-2202808	Clarification on inter-MN handover without SN change	NEC	CR	Rel-16	38.331	16.7.0	2908	-	A	NR_newRAT-Core

SN initiated release of SCG

[7] R2-2202123	Reply LS on signalling SN initiated release of SCG (R3-216236; contact: Ericsson)	RAN3	LS in	Rel-15	To:RAN2

Moved from 5.1

[8] R2-2203320	Clarification on SN initiated release of an SCG	Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-15	38.331	15.16.0	2938	-	F	NR_newRAT-Core

[9] R2-2203321	Clarification on SN initiated release of an SCG	Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.7.0	2939	-	A	NR_newRAT-Core

2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Nokia (Rapporteur) | Amaanat Ali | amaanat.ali@nokia.com |
| ZTE | LiuJing | liu.jing30@zte.com.cn |
| Huawei, HiSilicon | Lili Zheng | zhenglili4@huawei.com |
| Ericsson | Antonino Orsino | antonino.orsino@ericsson.com |
| NEC | Hisashi Futaki | hisashi.futaki @ nec.com |

3	Discussion

Topic 1: Inter-MN HO without SN change

In RAN2#113bis-e and #114e, the interpretation of the current specifications for inter-MN handover without SN change was deeply discussed, but there was no clear consensus. The LS to RAN3 was sent for asking the clarification on the presence of the SN UE X2/XnAP ID in the SN Addition Request from the target MN to the target (= source) SN. Now RAN2 has received the reply LS from RAN3 in [1] but unfortunately RAN3 did not address the key points which would have clearly helped RAN2 to conclude. Especially “No consensus on if the absence of SN UE X2/XnAP ID is applicable in case the source SN and target SN are the same SN” and “In case the SN UE X2/XnAP ID is provided alone, the SN is able to retrieve the stored UE context, there is no description in RAN3 specifications on whether the SN may perform delta configuration or not.” Seem to require RAN2 to make the final decision on the consequence of including or not including SN UE X2/XnAP ID and the influence of SN UE X2/XnAP ID on the full/delta configuration.

There is a TDOC in [4] with two proposals (the discussion will be on these to help companies understand the changes in [2], [3] which are Stage-2 focussed and [5], [6] which are Stage-3 focussed.

It is understood based on [4], that the proposal is to keep the SN UE X2/XnAP ID always PRESENT and switch the full/delta configuration behavior based on the presence or absence of the field set {sourceConfigSCG/ scg-RB-Config}. The rapporteur observes that [4] would like to keep the same “intention” of the current Stage-2 specification (labelled as Option 2 below)

- Option 2: IE sourceConfigSCG and scg-RB-Config as full or delta configuration flag [3] - Inter-MN HO without SN change (delta config is allowed in SN) - SN UE X2AP ID present - sourceConfigSCG present - scg-RB-Config present - Inter-MN HO without SN change (SN must apply full config) - SN UE X2AP ID present - sourceConfigSCG not present - scg-RB-Config not present

However, the CRs in [2], [3] which are supported by all the network vendors seem to align to a different objective which is to allow both the network implementation options labelled as Case 1 and Case 2 below.

Case 1: - Inter-MN HO without SN change (delta config is allowed in SN) - SN UE X2AP ID present - sourceConfigSCG not present - scg-RB-Config not present Case 2: - Inter-MN HO without SN change (delta config is allowed in SN) - SN UE X2AP ID not present - sourceConfigSCG present - scg-RB-Config present

Rapporteur notes that Case 1 is the same as Option 2. Also, Case 2 is different from Case 1 as can be seen above.

Proposal 1: RAN2 to confirm that current Stage 2 spec is aligned with the confirmation by RAN3. No change is necessary for Stage 2.

Proposal 2: RAN2 to confirm that the current specification intends the Option 2, where SN UE X2/Xn AP ID is “present” in SN Addition Request from target MN to target SN in inter-MN HO without SN change, regardless of delta config or full config for target SCG configuration.

Question 1: Do companies agree that both the network implementation possibilities (i.e., Case 1 and Case 2) need to be covered (which implies Stage-2 alignment)?

| Answers to Question 1 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Nokia | Yes | We understand both the network implementation options need to be supported based on what’s already there out on the field. So, we think only supporting behavior based on only P1/P2 may not be appropriate. Hence we support moving ahead with the changes in [2], [3] |
| ZTE | Yes | Same view as Nokia. |
| Huawei, HiSilicon | Yes | Same view as Nokia. |
| Ericsson | Yes | Same view as Nokia |
| NEC | Yes with comment | We understand majority wants to support this (or understands this is already supported), then we are fine with this direction. On the other hand, we have a concern on the cover sheet of [2][3]. It is the fact that the source SN and target SN are the same in Case 2. However, from procedure point of view, it is just like inter-MN HO “with” SN change, while it is the special case where source SN is same as target SN. As this is important to understand the real intention/meaning of inter-MN HO without SN change procedure, we would like to suggest changing the cover sheet by considering that aspect (e.g. to add text below in the last part of Reason for change), while are fine with the changes, as it seems majority is fine. “ Note that in the Case 2, target SN is the same as source SN, while the handover procedure looks like a special case of inter-MN HO with SN change, where the source SN is selected as target SN. “ We would also like to ask a confirmation for our CRs [5][6] to be agreed with possible clarifications in the cover page by considering the changes by [2][3] ? |

Summary 1: TBD.

Proposal 1: TBD.

Topic 2: SN initiated release of SCG

In [7] RAN3 has agreed a batch of CRs as attached. The changes include:

- X2AP: reuse the existing IE named EN-DC Resource Configuration IE in the SGNB MODIFICATION REQUIRED message with clarification on the usage of the IE, to support the SN initiated SCG release request.

- XnAP, F1AP and W1AP: enhance the signalling with introduction of a new “SCG Indicator” IE to support the SN initiated SCG release request.

In particular, If the SCG Indicator IE is contained in the S-NODE MODIFICATION REQUIRED message and it is set to “released”, the M-NG-RAN node shall, if supported, deduce that an SCG is removed.

As an indirect impact to RAN2 specification due to the above behavior accepted in RAN3, when the SCG indicator is included, the SN should not include the scg-CellGroupConfig and scg-CellGroupConfigEUTRA fields in the inter-node RRC messages and this should be clarified in the current specification.

Question 2: Do companies support the intent of the CRs in R2-2203320 and R2-2203321 to align the field descriptions in scg-CellGroupConfig and scg-CellGroupConfigEUTRA in accordance with the RAN3 LS?

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Nokia | Yes | [Proponent] If the behavior is not aligned in RAN2 this may cause an unclear behavior at the target node since the target node does not know whether the SCG should be released or reconfigured. |
| ZTE | Yes | We think the changes are aligned with RAN3 LS, to avoid inter-operability issue, it is better to make it clear in our spec. |
| Huawei, HiSilicon | No | The changes are not needed, as they are covered by the existing wording. The field is absent if neither SCG (re)configuration nor SCG configuration query nor SN triggered SN change is performed, e.g. at inter-node capability/configuration coordination which does not result in SCG (re)configuration towards the UE. The field is also absent upon an SCG release triggered by the SN. This field is not applicable in NE-DC. The current text indicates that, the field is absent if neither A nor B nor C takes place. And the added scenario D (SN triggered SCG release) belongs to neither A nor B nor C, so naturally the field is absent for D. We don’t see a need to add an extra sentence as above. |
| Ericsson | Yes | [Proponent] Basically same view as Nokia. Also, good to align to the RAN3 LS and avoid any inter-operability issue in current specification. |
| NEC | Yes | We agree to clarify this in the spec. One very minor comment for “Consequence if not approved”. Probably, we can say “If the CR is not approved, the target node may receive an SCG configuration during the SN-initiated SCG modification, even if the procedure is to release the SCG. …” (i.e. change will to may) |

Summary 2: TBD.

Proposal 2: TBD.

4	Conclusion

TBD.

5	Annex-1

In RAN2#113bis-e and #114e, the interpretation of the current specifications for inter-MN handover without SN change was deeply discussed, but there was no clear consensus. The LS to RAN3 was sent for asking the clarification on the presence of the SN UE X2/XnAP ID in the SN Addition Request from the target MN to the target (= source) SN. Now RAN2 has received the reply LS from RAN3 indicating the following answers [1]:

Question 1: In the inter-MN handover without SN change scenario, is the SN UE X2/XnAP ID always required to be present when target MN sends SN Addition Request to SN? RAN3 answer: - If the target MN decides to keep the SN, the target MN sends SN Addition Request to the SN including the SN UE X2/XnAP ID as a reference to the UE context in the SN that was established by the source MN. - No consensus on if the absence of SN UE X2/XnAP ID is applicable in case the source SN and target SN are the same SN. Question 2: For the same scenario, RAN2 would like to confirm with RAN3 if the receipt of SN UE X2/XnAP ID alone may be interpreted by SN to retrieve the SCG configuration to provide delta configuration? RAN3 answer: - In case the SN UE X2/XnAP ID is provided alone, the SN is able to retrieve the stored UE context, there is no description in RAN3 specifications on whether the SN may perform delta configuration or not.