3GPP TSG-RAN WG2 Meeting #117-e		         		       	  	            R2-220xxxx

Online, 21th Feb -3rd March, 2022

Title:	[Draft] Reply LS on interruption for PUCCH SCell activation in invalid TA case

Response to:	R2-2202149 (R4-2120420) LS on interruption for PUCCH SCell activation in invalid TA case

Release:	Rel-17

Work Item:	NR_RRM_enh2-Core

Source:	RAN2

To:	RAN WG4

Cc:	RAN WG1

Contact Person:

Name:                   Jie Shi

E-mail Address:   shijie@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 thanks RAN4 for the LS on interruption for PUCCH SCell activation in invalid TA case. For RAN4’s questions Q1/Q2:

- Q1: Can UE capability diffNumerologyAcrossPUCCH-Group defined in TS 38.306 be used to allow indication of UE support of parallel transmission of PRACH on PUCCH SCell and physical channels/signals with different SCS on other activated serving cells?

  - If YES, RAN4 would like to respectfully ask RAN2 to clarify that diffNumerologyAcrossPUCCH-Group applies to PRACH in the corresponding “Definitions for parameters” in TS 38.306.

  - If NO, are there any other existing capabilities that can serve this purpose? If so, RAN4 would like RAN1 to indicate the capabilities.

- Q2: If the answer in Q1 is NO, and there is no capabilities that can serve the purpose, what would be the expected UE behavior when a target PUCCH SCell PRACH has a different SCS from SpCell/SCell physical channels/signals according to RAN1/2 spec? e.g. Shall UE be able to simultaneously transmit PRACH on PUCCH SCell and physical channels/signals on other activated serving cells irrespective of SCS configurations?

RAN2 has taken RAN1 reply LS(R2-2203885/ R1-2202599) into account and discussed the technical aspects from RAN2 scope, RAN2 reply is as follows for Q1/Q2:

- A1:

- The answer is “NO”, according to the capability description in 38.306, the UE capability diffNumerologyAcrossPUCCH-Group is only related to data/control (data refers to PUSCH and control refers to PUCCH), but not to PRACH transmission

- The relevant capability in 38.306 which serves RAN4’s designated purpose is parallelTxPRACH-SRS-PUCCH-PUSCH for inter-band CA.

- The following spec texts quoted from 38.306 are for RAN4’s reference

diffNumerologyAcrossPUCCH-Group Indicates whether different numerology across two NR PUCCH groups for data and control channel at a given time in NR CA and (NG)EN-DC/NE-DC is supported by the UE.

parallelTxPRACH-SRS-PUCCH-PUSCH Indicates whether the UE supports parallel transmission of PRACH and SRS/PUCCH/PUSCH across CCs in an inter-band CA band combination.

- A2:

- For inter-band CA capability of simultaneous PRACH transmission on PUCCH SCell and SRS/PUCCH/PUSCH transmission on other activated serving cells, it is provided by parallelTxPRACH-SRS-PUCCH-PUSCH as from 38.306, regardless of the SCS combination, there is no restriction to the PUCCH SCell for PRACH and the other activated serving cells for physical channels/signals should be in the same PUCCH group or different PUCCH groups.

- For intra-band CA, RAN2 agree to the understanding of RAN1 reply LS(R2-2203885/ R1-2202599), i.e., according to 38.213 8.1 as in below text table, UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when the gap between PRACH transmission and PUSCH/PUCCH/SRS transmission is less than N symbols; hence, UE does not transmit PRACH and PUSCH/PUCCH/SRS simultaneously for intra-band CA.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not transmit PRACH and PUSCH/PUCCH/SRS in a same slot or when a gap between the first or last symbol of a PRACH transmission in a first slot is separated by less than N symbols from the last or first symbol, respectively, of a PUSCH/PUCCH/SRS transmission in a second slot where N=2 for μ=0 or μ=1, N=4 for μ=2 or μ=3, and μ is the SCS configuration for the active UL BWP. For a PUSCH transmission with repetition Type B, this applies to each actual repetition for PUSCH transmission [6, TS 38.214].

2. Actions:

To RAN WG4

ACTION: 	RAN2 kindly asks RAN4 to take the above information into account.

3. Date of Next TSG-RAN2 Meetings:

RAN2#118-e          2022-05-16            2022-05-20     Online Meeting

RAN2#119             2022-08-22            2022-08-22     Toulouse, FR