3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-220xxxx

Online, 21 February – 03 March 2022

Agenda item:	8.24.1

Source:	Nokia (Rapporteur)

Title:	Offline [AT117-e][059][NR17] FR2 CA BW Classes and CBM (Nokia)

WID/SID:	NR_RF_FR2_req_enh2-Core

Document for:	Discussion and Decision

# 1	Introduction

This document is the report of the following email discussion:

[AT117-e][059][NR17] FR2 CA BW Classes and CBM (Nokia)

Scope: Treat R2-2202377, R2-2202904, R2-2203122, R2-2203024, R2-2202905, R2-2202389, R2-2202390, R2-2202910, R2-2202911, R2-2202912, R2-2202913, R2-2203493, R2-2203494, R2-2202365, R2-2202366. Ph1 Determine agreeable parts and converge on discussion points if any, Ph2 agree CRs and Reply LS out.

Intended outcome: Report, Agreed CRs (CRs with certain early impl. character need to be separate CRs), Approved LS out

Deadline: Schedule 1

RF FR2 - CA BW Classes and CBM

Offline

[AT117-e][059][NR17] FR2 CA BW Classes and CBM (Nokia)

Scope: Treat R2-2202377, R2-2202904, R2-2203122, R2-2203024, R2-2202905, R2-2202389, R2-2202390, R2-2202910, R2-2202911, R2-2202912, R2-2202913, R2-2203493, R2-2203494, R2-2202365, R2-2202366. Ph1 Determine agreeable parts and converge on discussion points if any, Ph2 agree CRs and Reply LS out.

Intended outcome: Report, Agreed CRs (CRs with certain early impl. character need to be separate CRs), Approved LS out

Deadline: Schedule 1

Topic 1: FR2 CA BW Classes

[1] R2-2202377	Reply LS on release independence aspects of newly introduced FR2 CA BW Classes and CBM/IBM UE capability	Nokia, Nokia Shanghai Bell	LS out	Rel-17	NR_RF_FR2_req_enh2-Core	R2-2200843	To:RAN4

[2] R2-2202904	Consideration on the FR2 CA bandwidth classes	ZTE Corporation, Sanechips	discussion	Rel-17	NR_RF_FR2_req_enh2-Core

[3] R2-2203122	Introduction of new FR2 CA bandwidth classes	Xiaomi Communications	discussion	Rel-17	NR_RF_FR2_req_enh2-Core	R2-2201385

[4] R2-2203024	Discussion on FR2 new bandwidth class	Huawei, HiSilicon	discussion	Rel-17	NR_RF_FR2_req_enh2-Core

[6] R2-2202389	Introduction of FR2 FBG2 CA BW classes	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	16.7.0	2867	1	B	NR_RF_FR2_req_enh2-Core	R2-2200839

[7] R2-2202390	Introduction of FR2 FBG2 CA BW classes	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.306	16.7.0	0678	-	B	NR_RF_FR2_req_enh2-Core

[8] R2-2202910	CR on the FR2 CA bandwidth classes-38331	ZTE Corporation, Sanechips	CR	Rel-17	38.331	16.7.0	2915	-	B	NR_RF_FR2_req_enh2-Core

[9] R2-2202911	CR on the FR2 CA bandwidth classes-38306	ZTE Corporation, Sanechips	CR	Rel-17	38.306	16.7.0	0689	-	B	NR_RF_FR2_req_enh2-Core

[12] R2-2203493	Introduction of new FR2 CA bandwidth classes	Huawei, HiSilicon	draftCR	Rel-17	38.331	16.7.0	B	NR_RF_FR2_req_enh2-Core

[13] R2-2203494	Introduction of new FR2 CA bandwidth classes	Huawei, HiSilicon	draftCR	Rel-17	38.306	16.7.0	B	NR_RF_FR2_req_enh2-Core

Topic 2: CBM/IBM reporting

[5] R2-2202905	Consideration on the CBM/IBM reporting	ZTE Corporation, Sanechips	discussion	Rel-17	NR_RF_FR2_req_enh2-Core

[10] R2-2202912	CR on the CBM/IBM reporting-38331	ZTE Corporation, Sanechips	CR	Rel-17	38.331	16.7.0	2916	-	B	NR_RF_FR2_req_enh2-Core

[11] R2-2202913	CR on the CBM/IBM reporting-38306	ZTE Corporation, Sanechips	CR	Rel-17	38.306	16.7.0	0690	-	B	NR_RF_FR2_req_enh2-Core

[14] R2-2202365	Introduction of CBM capability	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.331	16.7.0	2868	1	B	NR_RF_FR2_req_enh2-Core	R2-2200840

[15] R2-2202366	Introduction of CBM capability	Nokia, Nokia Shanghai Bell	CR	Rel-17	38.306	16.7.0	0668	1	B	NR_RF_FR2_req_enh2-Core	R2-2200841

# 2	Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Nokia (Rapporteur) | Amaanat Ali | amaanat.ali@nokia.com |
| Huawei, HiSilicon | Tong Sha | shatong3@hisilicon.com |
| Qualcomm Incorporated | Masato Kitazoe | mkitazoe@qti.qualcomm.com |
| Samsung | Seungri Jin | Seungri.jin@samsung.com |

# 3	Discussion

The rapporteur proposes to continue the discussion from the previous meeting. To summarize the way forward from last meeting.

R2-2201928 Offline 037 on FR2 CA BW class Nokia [037] Noted, reflected below [037] Continue discussion for solution options for introducing the extended bandwidth class for FR2 CA bandwidth class in FBG2 (early implementation target as Rel-15) [037] FFS if RAN2 aims to harmonize solution to also include “dual bandwidth class across FBG” which is under discussion in RAN4 [037] Introduce CBM-only capability from Rel-17 (allowing early implementation from Rel-16) and dummify CBM enumeration from Rel-16 capability [037] FFS if IBM/CBM capability apply to DL and/or UL

Topic 1: FR2 CA BW Classes

First some views from different companies:

Listing the proposals from [2]

Proposal 1: Take solution direction 2 that a UE reports one of the new bandwidth classes and also reports the older one for a BC as baseline.

Proposal 2: The similar method to R/S/T/U reporting can be adopted for the “dual bandwidth class across FBG”.

Proposal 3: Do not extend the Aggregatedbandwidth (maxBandwidthRequestedDL/UL), the network can set the maxBandwidthRequestedDL/UL as absent and meanwhile limit the maxCarriersRequestedDL/ maxCarriersRequestedUL to achieve the similar result.

Listing the proposals from [3]

Proposal 1: When the UE indicates a new bandwidth class (i.e. R, S, T, U), the UE shall also indicate bandwidth class F.

Proposal 2: The indication of the new bandwidth classes (i.e. R, S, T, U) is via new capability signalling of ca-BandwidthClassDL-NR-v17xy/ ca-BandwidthClassUL-NR-v17xy.

Proposal 3: The indication of the new bandwidth classes (i.e. R, S, T, U) is allowed for early implementation from Rel-15.

Listing the proposals from [4]

Proposal 1: To ensure backward compatibility, it is suggested to consider select one of the solution below:

Solution 1: Introduce separate capability signalling to indicate support of new bandwidth classes (e.g. R, S, T, U).

Solution 2: Introduce a capability filter from the network side indicating new introduced bandwidth classes.

Proposal 2: Considering the future compatibility and signalling overhead, it is recommended to adopt solution 1.

From a check of also the CRs on the FR2 CA BW topic, there is clear consensus that the introduction of these new BW classes should take into account that the new UEs supporting these changes must also be able to signal the legacy BW class for those networks that may not be upgraded. This implies the following:

- UE reports one of the new bandwidth classes and also reports the older one for a BC as baseline i.e. UE indicates a new bandwidth class (i.e. R, S, T, U), the UE shall also indicate bandwidth class F.

- Introduce separate capability signalling to indicate support of new bandwidth classes (e.g. R, S, T, U) via Rel-17 specific extension using capability signalling of ca-BandwidthClassDL-NR-v17xy/ ca-BandwidthClassUL-NR-v17xy

Question 1: Do companies agree to the following principle: “A UE that indicates a new bandwidth class (i.e. R, S, T, U), the UE shall also indicate bandwidth class F” or more generically “UE reports one of the new bandwidth classes and also reports the older one for a BC as baseline”

| Answers to Question 1 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Nokia | Yes | Yes, to avoid the regression issue that the legacy network will not be able to understand the new bandwidth classes and may not be able to configure the UE and that results in reconfiguration failure it is good to have the principle agreed. |
| Huawei, HiSilicon | Yes | We understand the non-backward compatibility should be considered when introduction of new FR2 CA bandwidth classes. The solution above can avoid repeated BC reporting. |
| Qualcomm Incorporated | Yes | It should be clarified that the UE shall include at least as many FeatureSetDownlinkPerCC in a feature set according to the new ca-BandwidthClassDL-r17. See ZTE document in R2-2202904. |
| ZTE | Yes (and prefer the more general way) | Similar view as Nokia, and considering that more new bandwidth classes would be added in the future, to keep forward compatibility, we prefer the more general description that “UE reports one of the new bandwidth classes and also reports the older one for a BC as baseline” |
| Samsung | Yes | This approach seems the best solution with small specification impact. |

Summary 1: TBD.

Proposal 1: TBD.

On the need to update the Aggregated bandwidth capability filter, company in [2] proposes not to update the filter as the number of CC’s range is sufficient for the network to elicit the UE to report the new bandwidth classes. Company in [4] lists it as possible solution option but does not recommend to update so essentially aligned to Proposal 3 in [2]. Rapporteur proposes to check within the companies if there is a network vendor that would rely on the aggregated bandwidth capability filter only for asking UE to report one of the new bandwidth classes.

Question 2: From a network perspective is it sufficient to just use the maxCarriersRequestedDL/ maxCarriersRequestedUL to enable UE to report the new bandwidth class (i.e. no further updates to Aggregatedbandwidth (maxBandwidthRequestedDL/UL))?

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Nokia | Tend to No | - We would also prefer that the network has the option of using the Aggregatedbandwidth part of the filter as this is one possible network implementation and we would propose not to rule this out. - So we request RAN2 to update the maximum bandwidth in accordance to the RAN4 table so as to support the new bandwidth classes. - From a specification impact perspective this is not intensive change, so we request RAN2 to do it |
| Huawei, HiSilicon | No | We agree with Nokia that AggregatedBandwidth filter should be extended to support the new bandwidth classes, and how to use the filter is up to network. |
| Qualcomm Incorporated | No strong view | maxCarriersRequestedDL/UL has the benefit that it is comprehensible by legacy UE. Extension of Aggregatedbandwidth is not visible to legacy UEs. So we assume the network indicates both legacy and new field. It should be clear how the network populates legacy field and new field, and how the UE supporting both should handle the case, e.g. ignore the legacy field if new field is signalled. |
| ZTE | Yes(proponent) | We think the key issue is whether we need to extend the Aggregatedbandwidth in the UE capability filter or just use the maxCarriersRequestedDL/ maxCarriersRequestedUL to achieve the similar result. In the last meeting, we raised the question about whether to extend the Aggregatedbandwidth in the UE capability filter for that the legacy maximum Aggregatedbandwidth is 800M (and absence of Aggregatedbandwidth means 1200M ) , while the Aggregatedbandwidth of R/S/T/U are larger than it (e.g. 1000M/1200M/1400M/1600M). However after further thinking, we also find that there are several issues to be solved if we extend the Aggregatedbandwidth in the UE capability filter: - Modification to the UE capability reporting procedure to include newly added aggregated bandwidth - For the legacy filter, absent means the UE can report 1200Mhz, then we need to clarify how to set the legacy field when the the newly extended AggregatedBandwidth-r17 was set to 1000Mhz. - Option 1: Set the legacy field to be 800mhz, then the problem is that when handover to the old gNB, the old gNB will take it as 800M (Though the UE has also reported the bandwidth that support 1000M) Maybe this option can work for that 1000M was not supported in the Rel15/16 - Option 2: Set the legacy field to be absent-> then the problem is that When handover to old gNB, the old gNB will take it as 1200Mhz (which is the maximum aggregated bandwidth in Rel-15), however the UE that support RSTU may not report the BC with aggregated bandwidth 1200M for that the AggregatedBandwidth-r17 was set to 1000Mhz - The extend aggregated bandwidth shall also be included in the UE capability information as the legacy FreqBandlist (as below) has done appliedFreqBandListFilter FreqBandList OPTIONAL, Based on the above 3 issues, and also considering that maxCarriersRequestedDL/ maxCarriersRequestedUL can be used to achieve the similar result ( as analyzed in [2] that when the aggregated bandwidth was increased, the number of carriers are also increased), we prefer not to extend the legacy aggregated bandwidth. |
| Samsung | No strong view | Same view with Qualcomm that the handling of Extension of Aggregatedbandwidth from legacy UE should be clear if this approach is used. If this is not clear enough in the specification, the approach of maxCarriersRequestedDL/UL method is better than Extension of Aggregatedbandwidth. |

Summary 2: TBD.

Proposal 2: TBD.

Topic 2: CBM/IBM reporting

There was single FFS from last meeting.

- FFS if IBM/CBM capability apply to DL and/or UL

Based on the contributions in [5] and the rapporteur company’s understanding as well there seems to be no need to distinguish DL from UL and also CRs in [10], [11] and [14], [15] implement the decision from the last meeting.

- Introduce CBM-only capability from Rel-17 (allowing early implementation from Rel-16) and dummify CBM enumeration from Rel-16 capability

Question 3: Do companies agree that for IBM/CBM capability there is no need to distinguish DL from UL?

| Answers to Question 3 |  |  |
| --- | --- | --- |
| Company | Yes/No | Technical Arguments |
| Qualcomm Incorporated | Yes |  |
| ZTE | Yes (no need to distinguish) | After internal discussion, our understanding is that the network determine the DL/UL CBM/IBM supporting based on the beam correspondence as defined below Beam correspondence: the ability of the UE to select a suitable beam for UL transmission based on DL measurements with or without relying on UL beam sweeping.. According to the RAN4’s discussion, in Rel-17 the beam correspondence can be simply summarized as below: DL: CBM --->UL: only support single carrier DL: IBM ----> UL: IBM for the CA or single carrier. From beam correspondence aspect, the network can determine the UL beam management based on the DL, thus there is no need to distinguish DL from UL. Otherwise, some restriction to the UL and DL capabilities shall be added, e.g. when the UE support DL IBM the UE shall also support UL IBM. |
| Samsung | Yes |  |

Summary 3: TBD.

Proposal 3: TBD.

# 4	Conclusion

TBD.