3GPP TSG-RAN WG2#117-e	Draft R2-2203639

Online, 21 February - 3 March 2022

Source: 	Huawei, HiSilicon

Title: 	[AT117-e][222][DCCA] Actions at SCG activation and deactivation (Huawei)

Agenda Item:	8.2.2.2

Document for:	Discussion and decision

1	Introduction

This document is a summary of:

[AT117-e][222][DCCA] Actions at SCG activation and deactivation (Huawei)

Scope: Discuss remaining critical open issues (MAC aspects, SCG deactivation UE preference) for actions at SCG de/activation that were not yet handled as part of [Pre117-e][220].

Intended outcome: Discussion report in R2-2203639.

Deadline: Deadline 2

Company contact persons for this discussion are invited to fill one entry in the table below:

| Company | Contact details |
| --- | --- |
| Samsung | Jaehyuk Jang (jack.jang@samsung.com) |
| ZTE | LiuJing (liu.jing30@zte.com.cn) |
| Nokia | Jarkko Koskela (jarkko.t.koskela@nokia.com) |
| Lenovo, Motorola Mobility | Congchi Zhang (zhangcc16@lenovo.com) |
| OPPO | wangshukun@oppo.com |

2	Discussion

2.1	MAC actions at SCG deactivation / activation

2.1.1	Handling of Bj

This is about:

FFS if UE initializes Bj for each logical channel to zero upon SCG deactivation as a part of partial MAC reset. Should consider e.g. what to do with possible Bj increase while SCG is deactivated.

[23] [32][33] propose the Bj are initialized to zero upon SCG deactivation as part of partial MAC reset and are not increased while the SCG is deactivated.

[29] thinks LCP is not done when the SCG is deactivated, so the Bj are never incremented and if set to 0 at SCG deactivation they will remain to zero, while [39][40] think Bj will increase.

[39][40] propose initializing the Bj to zero upon SCG activation. [40] further considers that it is better not to initialize Bj to zero at SCG deactivation even if they are not initialized to zero at SCG activation.

So the possible options are:

1)	Bj are initialized to zero and remain to zero while the SCG is deactivated

2)	Bj are initialized to zero at SCG activation

3)	Bj are not initialized to zero, neither at SCG deactivation nor at SCG deactivation

Q1: Which option(s) do companies prefer?

| Company | Preference(s) | Comments |
| --- | --- | --- |
| Samsung | 2) or 1) | We are also fine to initialize Bj at SCG deactivation to avoid any possible impacts. |
| ZTE | 1) |  |
| Nokia | 2) and no need to change current MAC reset | Easiest is to keep MAC reset as is and just reset at activation – easy to capture in LCP section (see [39] TP). Trying to capture „remain zero“ while deactivated seems bit of a hack solution. |
| Lenovo, Motorola Mobility | 1) | In our understanding, the problem is essential about the operation about Bj value update should be stopped when SCG is deactivated. If UE keeps updating Bj value when SCG is in deactivated, it is purely a waste of processing and does not add any value, no matter if Bj value is initialized upon SCG deactivation or SCG reactivation. If UE stops any operation about Bj value update when SCG is deactivated and stops tracking the elapsed time since last increment, there is no real difference if initializing Bj upon SCG deactivation or SCG reactivation, since Bj will start from 0 at the moment of SCG activation. |
| OPPO | 2) | I wonder whether the Bj will also increase when UE enters RRC_INACTIVE state after MAC is reset? If yes, I think we can follow legacy behaviour as RRC_INACTIVE, i.e. the BJ is set to 0 in patial MAC reset. |

2.1.2	Explicitly signalled contention-free Random Access Resources

This is about

FFS if UE discards explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any, upon SCG deactivation as a part of partial MAC reset.

[39] thinks the “explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type” is rach-ConfigBFR in BeamFailureRecoveryConfig.

Several other companies think that this refers to resources configured in rach-ConfigDedicated in reconfigurationWithSync.

In Rel-16, rach-ConfigDedicated is optional “Need N”, i.e. this field is not stored and upon reception of a subsequent RRC message in which reconfigurationWithSync is included but this field is absent, the UE performs contention-based RACH.

The rapporteur would like to remind that RAN2 discussed the possibility to store, while the SCG is deactivated, dedicated RACH resources to be used for random access (when needed) at SCG activation and this was not agreed, so the rapporteur assumes this is not supported.

What is perhaps not sufficiently clear is that, if RACH is triggered by an SCG activation command that does not include an SN RRC message with reconfigurationWithSync, the UE shall perform contention-based random access.

This could be clarified e.g.

1)	in the initiation of the random access procedure due to SCG activation while reconfigurationWithSync is not included (could be in 38.321 or in 38.331)

2)	in 38.331 by indicating to discard the contents of rach-ConfigDedicated, if it was included, in the completion of a reconfiguration procedure in which the SCG is deactivated

The rapporteur thinks 2) might be simpler from specification perspective.

Q2: Would companies like to clarify that if RACH is triggered by an SCG activation command that does not include an SN RRC message with reconfigurationWithSync, the UE shall perform contention-based random access? If yes, please indicate preference between 1) and 2) or other.

| Company | Yes (and choice) or No | Comments |
| --- | --- | --- |
| Samsung | Yes – 2) | It can be done as a part of partial MAC reset. |
| ZTE | No | Based on RAN2 agreements in RAN2#116bis-e, if RACH is triggered, then SCG activation command will include SN RRC message together with reconfigurationWithSync. If reconfigurationWithSync is not included, then the UE is requested to perform RACH-less SCG activation. So it is unclear to us why SCG activation command without SN reconfigurationWithSync results in CBRA? Regarding whether network can provide dedicated RACH resource prior to SCG activation, it is a FFS in RAN2 agreements, so no conclusion has been made so far. Based on previous agreement, the network is allowed to reconfigure any RRC parameters during SCG deactivation, so asking UE to must discard dedicated RACH resources seems do not bring any benefit? Like BFR, dedicated RACH resource can also be pre-configured for SCG activation. We understand the rapporteur has concern on “Need N“ , but we think it is easy to clarify in spec that UE can store the configuration if SCG is deactivated. (like the way proposed in 2), just opposite view) |
| Nokia | No | We agree with ZTE |
| Lenovo, Motorola Mobility | No | Agree with ZTE and Nokia |
| OPPO | Yes -2) |  |

Q3: Assuming a clarification such as 1) and 2) is captured, do companies see the need to indicate in the partial MAC reset at SCG deactivation that the UE “discards explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any”?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Samsung | Yes | It would be the simplest approach. |
| ZTE | No | See our response to Q2. |
| Nokia | No |  |
| Lenovo, Motorola Mobility | no |  |
| OPPO | Yes |  |

2.2	SCG deactivation UE preference

RAN2 agreed previously to support an indication that the UE prefers the SCG is deactivated but did not agree any detail. There are a few proposals in this meeting for this, but they are not aligned.

[10] proposes something reusing the principles of UE preferences indicated in UE assistance information:

-	the network configures the UE to provide a preference for SCG deactivation

-	if this is configured, the UE can report a "preference for the SCG to be deactivated" or no such preference

-	the UE can only report a preference if it has changed (i.e. change from "preference for the SCG to be deactivated" to no such preference, or vice-versa)

-	the UE cannot report a preference before a certain time after its last report

-	the UE preference is valid as long as the RRC connection is not release

-	the reporting does not depend whether an SCG is configured or not, and, if configured, it is activated or deactivated

[40] proposes the same except that:

-	the preference is either "SCG activated" or "SCG deactivated"

-	there is a cause value

-	"SCG activated" with cause "uplinkData" is used instead of current uplink data indication for SCG bearer while the SCG is deactivated

However, that proposal does not consider the following scenario:

-	the UE indicates a preference of "SCG activation" because there are uplink data for an SCG DRB

-	the network activates the SCG and uplink data are transmitted

-	as there is no more activity on SCG DRBs, the network deactivates the SCG

-	there are uplink data but, since the UE preference is already "SCG activation", the UE cannot report it again

So this proposal is not feasible as it is.

[35] proposes:

-		the network configures the UE to provide a preference for SCG deactivation

-	if this is configured, the UE can report a preference "deactivated", "activated" or "released"

-	the UE can report the preference even if it has not changed

-	no prohibit timer

Since there is no time for any further study, the rapporteur suggests that the choice is limited between [10] and [35].

Q4: Do companies want to specify in Rel-17 a new UE assistance information as summarized above for [10] or [35]?

| Company | Yes/No Preference between [10] and [35] | Comments |
| --- | --- | --- |
| Samsung | Yes – [35] | It is simpler and has less impact. |
| ZTE | Yes – [10] with comments | [35] without prohibit timer should not be considerred. For only SCG deactivation preference, we prefer [10]. But we prefer to discuss the details during CR reviewing, the listed bullets are a bit vague. However, we don’t think the absence of SCG deactivation preference indication can be used to indicate “SCG activation request due to UL data arrival on SCG DRB”. In our view, we should treat “SCG deactivation preference” and “SCG activation request” separately. For “SCG activation request“ triggered by UL data on SCG bearer, it should not be restricted by the prohibit timer set for SCG deactivation reference indication. |
| Nokia | No – we have already agreed UL data arrival report on SCG to the NW which is the UE preference indication. no need for further indications. | If we have no rule why would UE indicate preference then it is impossible for NW to use this information. NW would not know why SCG should be activated based on the UE internal preference. And in fact we already agreed to introduce UE preference in a way by UE indicating UL data arrival on SCG via RRC (details are being discussed in this meeting). |
| Lenovo, Motorola Mobility | Yes [10] with comment | We assume this is only about SCG deactivation preference, since we agreed on the indication of UL data arrival for SCG activation as Nokia commented. A prohibit timer could be benefitial. |
| OPPO | No | UE can only request SCG activation due to UL data arrival. For SCG deactivation decision, the network can make the decision based on BSR. |

3	Conclusion

4	References

[1] R2-2202248, How to model the PSCell in SCG deactivation?, OPPO

[2] R2-2202250, SCG deactivation indication when resuming from RRC_INACTIVE due to MO data, OPPO

[3] R2-2202280	QoS flow remapping during SCG deactivation	Fujitsu

[4] R2-2202575	Discussion on UE behavior with SCG deactivated	Lenovo, Motorola Mobility

[5] R2-2202649	Discussion on UE behaviour when SCG is deactivated	ZTE Corporation, Sanechips

[6] R2-2202679	Views on several issues	Samsung Electronics

[7] R2-2202680	DC power sharing for deactivated SCG	Samsung Electronics

[8] R2-2202705	UE behaviour while SCG is deactivated	Qualcomm Incorporated

[9] R2-2202756	UE behavior while the SCG is deactivated	InterDigital, Inc.

[10] R2-2202767	Deactivation of SCG	LG Electronics Finland

[11] R2-2202795	Discussion on UE behaviour while SCG is deactivated	vivo

[12] R2-2202919	TA timer and RLM/BFD while the SCG is deactivated	MediaTek Inc.

[13] R2-2203097	Discussions on UE Behavior in Deactivated SCG	CATT

[14] R2-2203176	Open Issues on UE Behavior	NTT DOCOMO INC.

[15] R2-2203184	UE behaviour while SCG is deactivated	Nokia, Nokia Shanghai Bell

[16] R2-2203375	Open issues on UE behaviours while the SCG is deactivated	Huawei, HiSilicon

[17] R2-2203390	UE behaviour while SCG is deactivated	Ericsson

[18] R2-2202247	L2 based SCG activation and SCG RRM	OPPO

[19] R2-2202281	Proposal for releasing statusReportRequired for SCG bearers at SCG deactivation	Fujitsu

[20] R2-2202282	Remaining issues on UL data arrival for SCG	Fujitsu

[21] R2-2202351	Futher discussion on actions at SCG activation or deactivation	Transsion Holdings

[22] R2-2202413	Discussion on activation and deactivation of SCG	Spreadtrum Communications

[23] R2-2202576	MAC related issues upon SCG activation and deactivation	Lenovo, Motorola Mobility

[24] R2-2202650	Activation of deactivated SCG	ZTE Corporation, Sanechips

[25] R2-2202701	Actions at SCG activation and deactivation	Qualcomm Incorporated

[26] R2-2202757	Deactivation of SCG	InterDigital, Inc.

[27] R2-2202758	Activation of SCG	InterDigital, Inc.

[28] R2-2202796	Discussion on actions at SCG activation and deactivation	vivo

[29] R2-2202809	Remaining issues on SCG deactivation	NEC

[30] R2-2203039	Remaining issues for MAC procedure in deactivated SCG	SHARP Corporation

[31] R2-2203061	split bearer handling upon SCG deactivation	Sharp

[32] R2-2203087	Open issues on SCG deactivation	DENSO CORPORATION

[33] R2-2203092	Discussion on partial MAC reset upon SCG deactivation	LG Electronics Inc.

[34] R2-2203098	Remaining Issues on Actions at SCG Activation and Deactivation	CATT

[35] R2-2203099	Discussion on RRC Aspects of SCG Deactivation	CATT

[36] R2-2203166	Discussion on data transmission to MN for split bearer	LG Electronics Inc.

[37] R2-2203177	Open Issues on SCG Activation and Deactivation	NTT DOCOMO INC.

[38] R2-2203185	UL data handling at SCG deactivation	Nokia, Nokia Shanghai Bell

[39] R2-2203186	Actions at SCG activation and deactivation	Nokia, Nokia Shanghai Bell

[40] R2-2203391	Actions at SCG activation and deactivation	Ericsson

[41] R2-2203376	Handling of uplink split bearers and BWP when the SCG deactivated	Huawei, HiSilicon