3GPP TSG-RAN WG2 Meeting #117-e	R2-220xxxx

# Electronic meeting, 21 February – 3 March 2022

Agenda item:		8.5.1

Source:	Intel Corporation

Title:	Report of email discussion [AT117-e][508][IIoT] UE Capabilities (Intel)

Document for:		Discussion and Decision

# Introduction

The contribution is the report of following email discussion:

[AT117-e][508][IIoT] UE Capabilities (Intel)

UE capabilities CR (38.306/308.331)

Deadline: final approval by March 2nd

# Contact Information

| Company | Name | Email |
| --- | --- | --- |
| Intel | Yujian Zhang | yujian.zhang@intel.com |

# Discussion

## UE capability for RTT based PDC

In email discussion “[POST116bis-e][513][IIoT] CP open issues (Ericsson)” R2-2203302 [1], UE capability regarding UE-side and gNB-side PDC was discussed. Following is the summary and proposal by the rapporteur. In RAN2#117-e online session, the issue was not handled due to lack of time and will be discussed in current email discussion.

Summary: A UE supporting FG 25-19/25-19a shall also support: - 3 companies support UE-side PDC - 8 companies support both UE-side and gNB-side PDC (if agreed) - 4 companies support at least UE-side or gNB-side PDC (if agreed) There seems to have some misunderstandings. Rapporteur intends to sort-out the capability dependency, in light of the RAN1 introduced FG 25-19/25-19a. Some more clarification below (assuming the gNB-side PDC is agreed): - If UE supporting of FG 25-19/25-19a also supports UE-side PDC, then there is a need for a separate optional UE capability for gNB-side PDC; - If UE supporting of FG 25-19/25-19a supports both UE-side and gNB-side PDC, then there is no need for any more optional UE capabilities in RAN2; - If UE supporting of FG 25-19/25-19a also at least supports UE-side PDC or gNB-side PDC, then there is a need for two separate optional UE capabilities, one for gNB-side PDC and another for UE-side PDC. The proposal below is a majority view - Proposal 8 UE supporting of FG 25-19/25-19a also supports both UE-side and gNB-side PDC (if agreed). (8/15)

In current email discussion, we will focus on UE capability aspect. RAN1 has defined two separate RTT based PDC capabilities in feature list R1-2200780 [2]:

| 25-19 | RTT-based Propagation delay compensation based on CSI-RS for tracking and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS | 2-51, 2-53 |
| --- | --- | --- | --- |
| 25-19a | RTT-based Propagation delay compensation based on DL PRS and SRS | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS | 25-19, 13-1, 2-53 |

As highlighted by rapporteur of “[POST116bis-e][513][IIoT] CP open issues (Ericsson)”, if RAN2 agreed that UE supporting of FG 25-19/25-19a shall support both UE-side and gNB-side PDC, then there is no need for any additional UE capabilities in RAN2. Otherwise, additional UE capabilities should be defined.

Question 1: Please provide your preferred option on UE capability regarding UE-side and gNB-side RTT based PDC.

- Option a: A UE supporting FG 25-19/25-19a shall support both UE-side and gNB-side PDC (if agreed). In this option, there is no need for any additional optional UE capabilities in RAN2.

- Option b: A UE supporting FG 25-19/25-19a shall support UE-side PDC. The UE supports gNB-side PDC (if agreed) optionally. In this option, separate UE capability for gNB side PDC should be introduced.

- Option c: A UE supporting FG 25-19/25-19a shall support either UE-side or gNB-side PDC (if agreed). In this option, two separate UE capabilities for UE-side and gNB side PDC should be introduced.

| Company | Preferred option (a/b/c) | Comments |
| --- | --- | --- |
| Intel | a | Our understanding is that if UE supports one of the UE-side or gNB-side RTT PDC method, the additional efforts to support the other is marginal. Therefore it is reasonable to go with option a to avoid market segmentation as well as to minimize the introduction of additional UE capabilities due to option b/c. |

If RAN2 agrees on option b in Q1, separate UE capability for gNB side PDC should be introduced, and the dependency, capability type, and FRx/xDD differentiation of the capability need to be discussed. Since RAN1 has separate UE capabilities FG 25-19/25-19a, there could be various options regarding the dependency between UE capability for gNB side PDC and FG 25-19/25-19a. Currently the dependency between FG 25-19 and 25-19a is still not finalized (highlighted in yellow above) in R1-2200780 [2]. For simplicity, it is proposed that if option b in Q1 is supported, a UE supporting gNB side RTT based PDC shall support either 25-19 or 25-19a. If RAN1 confirms that 25-19 is one of the prerequisite FGs of 25-19a, then RAN2 dependency can be updated so that a UE supporting gNB side RTT based PDC shall support 25-19. In addition, given that gNB side RTT based PDC is mainly a RAN2 feature related to RAN1 features, it is proposed that the capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

Question 2: If Option b in Q1 is agreed, do you agree with the following:

An optional UE capability signalling is introduced for gNB side RTT based PDC (if agreed). A UE supporting this feature shall also support FG 25-19 or 25-19a (the dependency can be further updated based on RAN1 progress). The capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

| Company | Agree / Disagree | Comments |
| --- | --- | --- |
| Intel | Agree |  |

Similar reasoning regarding option b can be applicable to option c.

Question 3: If Option c in Q1 is agreed, do you agree with the following:

An optional UE capability signalling is introduced for gNB side RTT based PDC (if agreed). A UE supporting this feature shall also support FG 25-19 or 25-19a (the dependency can be further updated based on RAN1 progress). The capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

An optional UE capability signalling is introduced for UE side RTT based PDC. A UE supporting this feature shall also support FG 25-19 or 25-19a (the dependency can be further updated based on RAN1 progress). The capability is per UE, not FDD-TDD DIFF, not FR1-FR2 DIFF.

| Company | Agree / Disagree | Comments |
| --- | --- | --- |
| Intel | Agree |  |

## Other disucssion for UE capability CRs

Draft UE capabilities CRs based on [3][4] are provided for review. In addition to use comments for the draft CRs, additional comments can be provided below.

Question 4: Additional comments for draft UE capabilities CRs can be provided below.

| Company | Comments |
| --- | --- |

# Conclusion

To be updated.

# References

[1] R2-2203302, Ericsson, "Summary of [POST116bis-e][513][IIoT] CP open issues (Ericsson)"

[2] R1-2200780, Moderators (AT&T, NTT DOCOMO, INC.), "Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e"

[3] R2-2202464, Intel, "Draft 38.306 CR for Rel-17 NR IIoT URLLC UE capabilities"

[4] R2-2202465, Intel, "Draft 38.331 CR for Rel-17 NR IIoT URLLC UE capabilities "