3GPP TSG-RAN WG2 Meeting #117 Electronic	R2-2203951

Online, Feb 21st – Mar 3rd, 2022

Agenda Item:	8.5.1

Source:	Ericsson

Title:	Summary of [AT117-e][513][IIoT] CR 38.331(Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

This contribution collects companies’ views on some aspects of the RRC CR, per the below email discussion instruction:

[AT117-e][513][IIoT] CR 38.331 (Ericsson)

Review and agree to final CR 38.331

Deadline:

Contact person(s) for each participating company:

| Company | Name | Email |
| --- | --- | --- |
| Ericsson | Zhenhua Zou | zhenhua.zou@ericsson.com |

# 2	Discussion

## 2.1	Periodicity of UE Rx-Tx time difference

RAN2 agrees the below

1	RAN2 confirms that gNB-side RTT Propagation Delay Compensation is supported.

2	UE Rx-Tx time difference measurement report is triggered by an explicit one-shot RRC request.

3	Periodic measurement reporting is supported

4	The periodicity of UE Rx-Tx time difference measurement is part of the RRC configuration.

5	The periodicity value is selected by the gNB as part of periodic reporting configuration. Range for required periodicities can be decided by RAN2 and further confirmed with RAN1/RAN4 later, if needed.

In the paper R2-2202728 [1], it is proposed that two options can be used to determine the periodicity.

- The same order of SIB9, which is used to periodically deliver RTI. But, after receiving the UE Rx-Tx time difference, gNB can only transmit the pre-compensated RTI in a unicast message, per the latest agreement. It does not seem to be necessary to align with SIB9.

The supported periodicity is shown below. One “rf” (radio frame) equals to 10 milliseconds.

SchedulingInfo ::=                  SEQUENCE {

si-BroadcastStatus                  ENUMERATED {broadcasting, notBroadcasting},

si-Periodicity                      ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo                     SIB-Mapping

}

- The regular RRM measurement periodicity. NR supports periodical reporting, and the period is configured as reportInterval. However, it is unclear if this can meet the requirement of 100 nanoseconds accuracy of reference time delivery on the Uu interface and so the same PDC accuracy requirement. The channel could change quite a lot in the lowest periodicity, 120 milliseconds, that impact the propagation delay, while it would be okay for mobility and RRM.

–	ReportInterval

The IE ReportInterval indicates the interval between periodical reports. The ReportInterval is applicable if the UE performs periodical reporting (i.e. when reportAmount exceeds 1), for triggerTypeevent as well as for triggerTypeperiodical. Value ms120 corresponds to 120 ms, value ms240 corresponds to 240 ms and so on, while value min1 corresponds to 1 min, min6 corresponds to 6 min and so on.

ReportInterval information element

-- ASN1START

-- TAG-REPORTINTERVAL-START

ReportInterval ::=  ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960,min1,min6, min12, min30}

-- TAG-REPORTINTERVAL-STOP

-- ASN1STOP

Since this is the last meeting, rapporteur proposes to first adopt a set of values that seem to be okay for most companies and discuss case-by-case the minimum periodicity and the maximum periodicity. The baseline value is the values supported by both reportInterval and si-Periodicity and up-to 5120 ms.

Q1. Do companies agree to use the below value as the baseline for the supported periodicity? Please also provide any inputs on if some periodicity values are not needed.

{80ms, 120ms, 160ms, 240ms, 320ms, 480ms, 640ms, 1024ms, 1280ms, 2048ms, 2560ms, 5120ms}

| Company | Yes, No? | Comments |
| --- | --- | --- |

For larger periodicity value, it can be implemented by the network with a periodic one-shot request and thus the benefit beyond signalling overhead reduction needs to be discussed.

Q2. Do companies see any benefits to have periodicity larger than, e.g., 5120 ms? If yes, what is the maximum value?

| Company | Yes, No? | Comments |
| --- | --- | --- |

On the other hand, the smallest periodicity of the PRS is 4 slots at 15 kHz SCS, 8 slots at 30 kHz SCS, 16 slots at 60 kHz SCS, 32 slots at 120 kHz. The smallest periodicity of the CSI-RS resources is 4 slots and not clear if it would be different for different SCS. Both PRS and CSI-RS can be configured for measurements. The smallest periodicity of the SRS is 1 slot and not clear if it would be different for different SCS. See annex 5 for details. On one extreme, the UE reports for every measurement and so the smallest reporting periodicity can be four slots, i.e., 4 milliseconds.

The next question is to ask if companies see any benefits to have periodicity smaller than 80 milliseconds. RAN2 can also leave this question to RAN1/4. In any case, there will be reserved code points in ASN.1 to incorporate any smaller periodicity if found useful in deployment.

Q3. On the smallest periodicity, what are the companies’ preference?

Alt1: Periodicity can be smaller than 80 milliseconds, e.g., 10 ms, 20ms, 40ms

Alt2: The smallest periodicity is 80 milliseconds

Alt3: RAN2 ask RAN1/4 for further inputs.

| Company | Alt1, Alt2, Alt3? | Comments |
| --- | --- | --- |

## 2.2 One-shot explicit request

There were comment online that the one-shot explicit request and periodic request can re-use the same RRC signalling structure. It seems that the one-shot explicit request can be implemented with reportAmount=1. See below RRC spec excerpt

### 5.5.5 Measurement reporting 1> increment the numberOfReportsSent as defined within the VarMeasReportList for this measId by 1; 1> stop the periodical reporting timer, if running; 1> if the numberOfReportsSent as defined within the VarMeasReportList for this measId is less than the reportAmount as defined within the corresponding reportConfig for this measId: 2> start the periodical reporting timer with the value of reportInterval as defined within the corresponding reportConfig for this measId;

Q4. Do companies agree that the explicit one-shot request can be implemented with reportAmount configured with value one?

| Company | Yes, No? | Comments |
| --- | --- | --- |

## 2.3 Explicit indication to fallback to SIB9

RAN2 has agreed to the below

6	The network tells the UE whether to fallback to SIB9 via explicit signalling, at least in the RRC reconfiguration with synch and reconfiguration after re-establishment.

There are a couple of options for which the explicit signalling can be added. The explicit signalling can be, for example, an RRC field with value “fallback” in the below.

Option 1: the RRC message DLInformationTransfer. This is aligned with the existing RTI delivery procedure. The network can indicate this, if needed, after handover (RRC reconfiguration with synch), RLF recovery (reconfiguration after re-establishment) and etc.

Option 2: The RRC message RRCReconfiguration.

Since the explicit signalling is agreed, from rapporteur’s point of view, option 1 is preferred. It is easier to capture and has less impacts from/to other features/functionalities in the RRC message RRCReconfiguration.

Q5. Do companies agree to add the explicit indication to fallback to SIB9 in the RRC message DLInformaionTrasnfer? If not, please indicate an alternative.

| Company | Yes, No? | Comments |
| --- | --- | --- |

# 3	Conclusion

TBD

# 4	References

- R2-2202728, Remaining Issues on PDC Enhancement, CMCC

# 5	Annex

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r17 ::= CHOICE {

scs15-r17		CHOICE {

n4-r17					INTEGER (0..3),

n5-r17					INTEGER (0..4),

n8-r17					INTEGER (0..7),

n10-r17					INTEGER (0..9),

n16-r17					INTEGER (0..15),

n20-r17					INTEGER (0..19),

n32-r17					INTEGER (0..31),

n40-r17					INTEGER (0..39),

n64-r17					INTEGER (0..63),

n80-r17					INTEGER (0..79),

n160-r17				INTEGER (0..159),

n320-r17				INTEGER (0..319),

n640-r17				INTEGER (0..639),

n1280-r17				INTEGER (0..1279),

n2560-r17				INTEGER (0..2559),

n5120-r17				INTEGER (0..5119),

n10240-r17				INTEGER (0..10239),

...

},

scs30-r17		CHOICE {

n8-r17					INTEGER (0..7),

n10-r17					INTEGER (0..9),

n16-r17					INTEGER (0..15),

n20-r17					INTEGER (0..19),

n32-r17					INTEGER (0..31),

n40-r17					INTEGER (0..39),

n64-r17					INTEGER (0..63),

n80-r17					INTEGER (0..79),

n128-r17				INTEGER (0..127),

n160-r17				INTEGER (0..159),

n320-r17				INTEGER (0..319),

n640-r17				INTEGER (0..639),

n1280-r17				INTEGER (0..1279),

n2560-r17				INTEGER (0..2559),

n5120-r17				INTEGER (0..5119),

n10240-r17				INTEGER (0..10239),

n20480-r17				INTEGER (0..20479),

...

},

scs60-r17		CHOICE {

n16-r17					INTEGER (0..15),

n20-r17					INTEGER (0..19),

n32-r17					INTEGER (0..31),

n40-r17					INTEGER (0..39),

n64-r17					INTEGER (0..63),

n80-r17					INTEGER (0..79),

n128-r17				INTEGER (0..127),

n160-r17				INTEGER (0..159),

n256-r17				INTEGER (0..255),

n320-r17				INTEGER (0..319),

n640-r17				INTEGER (0..639),

n1280-r17				INTEGER (0..1279),

n2560-r17				INTEGER (0..2559),

n5120-r17				INTEGER (0..5119),

n10240-r17				INTEGER (0..10239),

n20480-r17				INTEGER (0..20479),

n40960-r17				INTEGER (0..40959),

...

},

scs120-r17		CHOICE {

n32-r17					INTEGER (0..31),

n40-r17					INTEGER (0..39),

n64-r17					INTEGER (0..63),

n80-r17					INTEGER (0..79),

n128-r17				INTEGER (0..127),

n160-r17				INTEGER (0..159),

n256-r17				INTEGER (0..255),

n320-r17				INTEGER (0..319),

n512-r17				INTEGER (0..511),

n640-r17				INTEGER (0..639),

n1280-r17				INTEGER (0..1279),

n2560-r17				INTEGER (0..2559),

n5120-r17				INTEGER (0..5119),

n10240-r17				INTEGER (0..10239),

n20480-r17				INTEGER (0..20479),

n40960-r17				INTEGER (0..40959),

n81920-r17				INTEGER (0..81919),

...

},

...

}

periodicityAndOffset

This field specifies the periodicity of DL-PRS allocation in slots and the slot offset with respect to SFN #0 slot #0 in the PCell where the DL-PRS-PDC Resource Set is configured (i.e., slot where the first DL-PRS Resource of DL-PRS-PDC Resource Set occurs).

#### –	CSI-ResourcePeriodicityAndOffset

The IE CSI-ResourcePeriodicityAndOffset is used to configure a periodicity and a corresponding offset for periodic and semi-persistent CSI resources, and for periodic and semi-persistent reporting on PUCCH. both, the periodicity and the offset are given in number of slots. The periodicity value slots4 corresponds to 4 slots, value slots5 corresponds to 5 slots, and so on.

CSI-ResourcePeriodicityAndOffset information element

-- ASN1START

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-START

CSI-ResourcePeriodicityAndOffset ::=    CHOICE {

slots4                                  INTEGER (0..3),

slots5                                  INTEGER (0..4),

slots8                                  INTEGER (0..7),

slots10                                 INTEGER (0..9),

slots16                                 INTEGER (0..15),

slots20                                 INTEGER (0..19),

slots32                                 INTEGER (0..31),

slots40                                 INTEGER (0..39),

slots64                                 INTEGER (0..63),

slots80                                 INTEGER (0..79),

slots160                                INTEGER (0..159),

slots320                                INTEGER (0..319),

slots640                                INTEGER (0..639)

}

-- TAG-CSI-RESOURCEPERIODICITYANDOFFSET-STOP

-- ASN1STOP

SRS-PeriodicityAndOffset-r16 ::=        CHOICE {

sl1                                     NULL,

sl2                                     INTEGER(0..1),

sl4                                     INTEGER(0..3),

sl5                                     INTEGER(0..4),

sl8                                     INTEGER(0..7),

sl10                                    INTEGER(0..9),

sl16                                    INTEGER(0..15),

sl20                                    INTEGER(0..19),

sl32                                    INTEGER(0..31),

sl40                                    INTEGER(0..39),

sl64                                    INTEGER(0..63),

sl80                                    INTEGER(0..79),

sl160                                   INTEGER(0..159),

sl320                                   INTEGER(0..319),

sl640                                   INTEGER(0..639),

sl1280                                  INTEGER(0..1279),

sl2560                                  INTEGER(0..2559),

sl5120                                  INTEGER(0..5119),

sl10240                                 INTEGER(0..10239),

sl40960                                 INTEGER(0..40959),

sl81920                                 INTEGER(0..81919),

...

}