3GPP TSG-RAN WG2 #117-e	Tdoc R2-22xxxxx

Electronic meeting, 2022-02-21 - 2022-03-03

Agenda Item:	8.11.1

Source:	Ericsson

Title:	 [AT117-e][607][POS] Positioning running CR to 38.331 (Ericsson)

Document for:	Discussion, Decision

# 1	Introduction

# 1	Introduction

This document is to collect comments for the RRC CRs:

- [AT117-e][607][POS] Positioning running CR to 38.331 (Ericsson)

Scope: Review and update the CR in R2-2203364, including merge of the draft CRs in R2-2203362 and R2-2203445.

Intended outcome: Endorsable CR

Deadline:  Friday 2022-02-25 1000 UTC

The intention is to endorse below RRC CRs and capture it is R2-2203364

R2-2203362 RAN1 parameter list impact to RRC running CR Ericsson draftCR Rel-17 38.331 16.7.0 B NR_pos_enh-Core

R2-2203445 Capturing RRC impacts for RAT dependent Positioning Ericsson draftCR Rel-17 38.331 16.7.0 B NR_pos_enh-Core

# 2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| CATT | Jianxiang Li (lijianxiang@catt.cn) |
| Intel | Yi.guo@intel.com |
| Qualcomm | sfischer@qti.qualcomm.com |

# 3	3	Discussion

## 3.1	RAN1 Parameter list and Related RAN2 Agreements CR

Please provide your comments on the CR.

| Company | Comments |
| --- | --- |
| CATT | #### 5.7.XX.2 Initiation 1> if ue-TxTEG-RequestUL-TDOA-Config in RRCReconfiguration message to requestthe association between UL SRS Resource for positioning and Tx TEG: |
| Intel | 1 MG, so far the preconfigured MG cannot be released. MeasGapPreConfigList-r17 ::= SEQUENCE (SIZE (1..maxGapConfig)) OF MeasGapPreConfig-r17 Would be good to use SetupRelease structure. 2 Similar to MGE WI (R2-2202868), would be good to add MGID MeasGapConfig ?, e.g. GapConfig ::= SEQUENCE { gapOffset INTEGER (0..159), mgl ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6}, mgrp ENUMERATED {ms20, ms40, ms80, ms160}, mgta ENUMERATED {ms0, ms0dot25, ms0dot5}, ..., [[ refServCellIndicator ENUMERATED {pCell, pSCell, mcg-FR2} OPTIONAL -- Cond NEDCorNRDC ]], [[ refFR2ServCellAsyncCA-r16 ServCellIndex OPTIONAL, -- Cond AsyncCA mgl-r16 ENUMERATED {ms10, ms20} OPTIONAL -- Cond PRS ]], [[ measGapId-r17 MeasGapId-r17 OPTIONAL, preConfigInd-r17 ENUMERATED {true} OPTIONAL -- Need R ]] } |

## 3.2	RAT dependent Positioning

Please provide your views on RAT dependent positioning CR.

| Company | Comments |
| --- | --- |
| Qualcomm | (According to RAN1 agreements/LS (e.g., R2-2200083), SRS for positioning may be transmitted in RRC_INACTIVE State, and therefore: - RAN1 assumes that - SRS for positioning for UEs in RRC_INACTIVE state is configured using the SRS-PosResourceSet IE However, in the current running CR, SRS for positioning in SuspendConfig is configured via SRS-Config. A possible implementation according to RAN1 agreement was proposed by e.g., CATT in R2-2203091: SRS-PosRRC-InactiveConfig-r17 ::= SEQUENCE { srs-PosConfig-r17 SetupRelease {SRS-PosConfig-r17} OPTIONAL, -- Need M bwp-r17 BWP OPTIONAL, -- Need S srs-TimeAlignmentTimer-r17 ENUMERARED {FFS align with SDT} OPTIONAL, -- Need R changeThresh-r17 RSRP-ChangeThresh-r17, OPTIONAL, -- Need R } RSRP-ChangeThresh-r17 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1} SRS-PosConfig-r17 ::= EQUENCE { srs-PosResourceSetToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16 OPTIONAL, -- Need N srs-PosResourceSetToAddModList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16 OPTIONAL,-- Need N srs-PosResourceToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16 OPTIONAL,-- Need N srs-PosResourceToAddModList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16 OPTIONAL -- Need N } This would be a better implementation of the RAN1 agreements. Therefore, the SRS-Config should be replaced with SRSPosResourceSet |

## 3.3	Any other comments

Question 3: please provide any additional comment; e.g. any additional impacts foreseen

| Company | Comments |
| --- | --- |

# 4	Conclusion