3GPP TSG RAN WG2 Meeting #117-e                                           R2-220xxxx

# Electronic meeting, 21 Feb- 3 March, 2022

Agenda item:			8.11.2.7

Source:			Intel Corporation

Title:		Report of [Pre117-e][612][POS] Open issues on positioning UE capabilities (Intel)

Document for:	 	Discussion and decision

- Introduction

This is the report of offline discussion [Pre117-e][612][POS] Open issues on positioning UE capabilities (Intel).

Feb 9th 	Start of Pre-discussions that collects structured company Input.

Feb 14th, 2359 UTC.	General Tdoc Submission Deadline. Tdoc number allocation deadline. Kick off, summaries. Stop of Pre-discussions that collects structured company Input (rapporteurs to provide report at earliest convenient time, within 24h if possible).

Feb 17th 1800 UTC	Tdocs submission deadline for Summaries

Companies please provide your comments by Feb 14th, 2359 UTC.

Annex: companies’ point of contact

| Company | Point of contact | Email address |
| --- | --- | --- |
| Intel Corporation | Yi Guo | Yi.guo@intel.com |
| Huawei, HiSIlicon | Yinghao Guo | Yinghaoguo@huawei.com |

Discussion

Based on R2-2202005 and R2-2201722, positioning UE capabilities related open issues list for are captured in section 5 and 6. Only the issues not covered by other pre-meeting discussions will be discussed in Pre-117-e612.

3.2 RAN2 led items

3.2.1 Finer granularity of response time

LPP running CR rapporteur captured it as

ResponseTime --> unit-r15 --> ten-milli-seconds-r17 OTDOA-ProvideCapabilities --> ten-ms-unit-ResponseTime-r17 A-GNSS-ProvideCapabilities --> ten-ms-unit-ResponseTime-r17 ECID-ProvideCapabilities --> ten-ms-unit-ResponseTime-r17 TBS-ProvideCapabilities-r13 --> ten-ms-unit-ResponseTime-r17 Sensor-ProvideCapabilities-r13 --> ten-ms-unit-ResponseTime-r17 WLAN-ProvideCapabilities-r13 --> ten-ms-unit-ResponseTime-r17 BT-ProvideCapabilities-r13 --> ten-ms-unit-ResponseTime-r17 NR-ECID-ProvideCapabilities-r16 --> ten-ms-unit-ResponseTime-r17 NR-DL-TDOA-ProvideCapabilities-r16 --> ten-ms-unit-ResponseTime-r17 NR-DL-AoD-ProvideCapabilities-r16 --> ten-ms-unit-ResponseTime-r17 NR-Multi-RTT-ProvideCapabilities-r16 --> ten-ms-unit-ResponseTime-r17

The discussion was:

Huawei: Not clear why the capability is affecting OTDOA/E-CID and RAT-independent response time capability. We assume that 10-ms granularity only applies for NR positioning methods (DL and DL+UL as in the WID). Nokia: 1. Whether the LTE positioning shall be enhanced to support finer granularity? 2. Although we support to introduce ten-milli-seconds, we would like to know if this has been concluded? QC: Issue #R1-7 Because it is in CommonIEsRequestLocationInformation. However, if we don't want this potentially being applicable to all methods, it can also be solved via field description.

Discussion point 3.2.1-1: For the finer granularity, which option do you prefer?

Option 1 Finer granularity is only applied for NR RAT dependent positioning methods;

Option 2 Finer granularity is only applied for NR RAT dependent positioning methods and RAT independent positioning methods;

Option 3 Finer granularity is applied for LTE and NR RAT dependent positioning methods and RAT independent positioning methods;

| Company’s name | Option 1 or Option 2 or Option 3 | Comments, if any |
| --- | --- | --- |
| Huawei,HiSilicon | Option1 | No strong view on the supporting for the others though |

Summary:

3.2.2 GNSS integrity

The capability on GNSS integrity was discussion in RAN2#116bis. Companies have different view on how to handle it. LPP running CR rapporteur captured it as

GNSS-SSR-CodeBiasSupport-r15-->ssr-IntegrityCodeBiasBoundsSup-r17 GNSS-SSR-PhaseBiasSupport-r16-->ssr-IntegrityPhaseBiasBoundsSup-r17 GNSS-SSR-STEC-CorrectionSupport-r16-->stec-IntegritySup-r17 GNSS-SSR-GriddedCorrectionSupport-r16 -->griddedCorrectionIntegritySup-r17 GNSS-CommonAssistanceDataSupport-->GNSS-Integrity-ServiceAlertSupport-r17 GNSS-CommonAssistanceDataSupport-->GNSS-Integrity-ServiceParametersSupport-r17

Discussion point 3.2.2-1: For GNSS integrity capability, do you agree capabilities captured in the running LPP CR R2-2201723?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

Summary: .

3.3 RAN1 feature lists

Note: For RAN1 feature lists, Rapporteur will only provide TP to show how to capture RAN1 features. FFS should be resolved by RAN1.

3.3.1 27-1 TEG

| 27. NR_pos_enh | 27-1-1 | UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning | 1. Support of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning 2. The maximum number of UE-RxTEG, which is supported and reported by UE for UE assisted DL TDOA and/or Multi-RTT positioning | 13-1, one or more of {13-3, 13-4} | No |  | UE-RxTEG reporting is not supported and no assumption can be made on the UE Rx timing errors for the measurements | per band | n/a | n/a | n/a | Component 1 candidate values: [One or more of] {UE-assisted DL TDOA, Multi-RTT positioning, UE-assisted DL TDOA and Multi-RTT positioning} Component 2 candidate values: {1, 2, 3, 4, 6, 8} Note: a single value is reported when both multi-RTT and DL-TDOA are supported Need for location server to know if the feature is supported If the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the UE Rx timing errors for this measurement Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-2 | Support of UE-TxTEGs for UL TDOA | The maximum number of UE-TxTEG for SRS resource for positioning, which is supported and reported by UE for UL TDOA | 13-8 | Yes |  | UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the [mitigation of] UE Tx timing error for the SRS resource for positioning | per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA Note: If the UE does not include TxTEG-ID associated with a SRS resource for positioning, no assumption can be made on the UE Tx timing error for this SRS resource for positioning. | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported and no assumption can be made on the [mitigation of] UE Tx timing error for the SRS resource for positioning | per band | n/a | n/a | n/a | The candidate values are {1,2,3,4,6,8} Need for location server to know if the feature is supported If the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the [mitigation of] UE Tx timing errors for this SRS resource for positioning Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-3 | Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4 and 13-8 | No |  | UE RxTx for Multi-RTT is not supported and no assumption can be made on the UE RxTx timing [error/delays] for the measurement | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 36, 48, 64} Need for location server to know if the feature is supported If the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the UE RxTx timing [errors/delays] for this measurement Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to 255 | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4 | Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP | 27-1-1 | No |  | Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | per band | n/a | n/a | n/a | The candidate values are {2, 3, 4, 6, 8} Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously | 27-1-4 | No |  |  | Per band | n/a | n/a | n/a | The candidate values are {1,2,34,6,8} Need for location server to know if the feature is supported. | Optional with capability signaling |

In LPP running CR R2-2201723, 27-1 was captured as per UE capability

DL TDOA

nr-UE-Rx-TEG-ID-Support-r17				INTEGER (1..maxNumOfRxTEGs-r17)				OPTIONAL,

measureSameDL-PRS-ResourceWithDifferentRxTEGsFR1-r17													INTEGER (1..maxNumOfRxTEGs-r17)		OPTIONAL,

measureSameDL-PRS-ResourceWithDifferentRxTEGsFR2-r17													INTEGER (1..maxNumOfRxTEGs-r17)		OPTIONAL,

Multi-RTT

nr-UE-RxTx-TEG-ID-Support-r17			NR-UE-RxTx-TEG-ID-Support-r17				OPTIONAL,

NR-UE-RxTx-TEG-ID-Support-r17 ::= SEQUENCE {

case1-Sup-r17	SEQUENCE {

nr-UE-RxTx-TEG-Sup-r17	INTEGER (1..maxNumOfRxTxTEGs-r17)

}															OPTIONAL,

case2-Sup-r17	SEQUENCE {

nr-UE-RxTx-TEG-Sup-r17	INTEGER (1..maxNumOfRxTxTEGs-r17),

nr-UE-Tx-TEG-Sup-r17	INTEGER (1..maxNumOfTxTEGs-r17)

}																OPTIONAL,

case3-Sup-r17	SEQUENCE {

nr-UE-Rx-TEG-Sup-r17	INTEGER (1..maxNumOfRxTEGs-r17),

nr-UE-Tx-TEG-Sup-r17	INTEGER (1..maxNumOfTxTEGs-r17)

}																OPTIONAL,

case4-Sup-r17	SEQUENCE {

nr-UE-RxTx-TEG-Sup-r17	INTEGER (1..maxNumOfRxTxTEGs-r17),

nr-UE-Tx-TEG-Sup-r17	INTEGER (1..maxNumOfTxTEGs-r17),

nr-UE-Rx-TEG-Sup-r17	INTEGER (1..maxNumOfRxTEGs-r17)

}																OPTIONAL,

...

}

measureSameDL-PRS-ResourceWithDifferentRxTxTEGsFR1-r17	INTEGER (1..maxNumOfRxTxTEGs-r17)

OPTIONAL,

measureSameDL-PRS-ResourceWithDifferentRxTxTEGsFR2-r17	INTEGER (1..maxNumOfRxTxTEGs-r17)

OPTIONAL,

measureSameDL-PRS-ResourceWithDifferentRxTEGsFR1-r17	INTEGER (1..maxNumOfRxTEGs-r17)

OPTIONAL,

measureSameDL-PRS-ResourceWithDifferentRxTEGsFR2-r17	INTEGER (1..maxNumOfRxTEGs-r17)

OPTIONAL,

However it should be per band capability, and some of features are not captured. Therefore it should be captured as

Suggested LPP TP:

DL TDOA/Multi-RTT, we may introduce common IE, and clarify 27-1-2a, 27-1-3 are only for multi-RTT;

nr-UE-TEG-ID-CapabilityBandList-r17	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

NR-UE-TEG-ID-CapabilityPerBand-r17,

NR-UE-TEG-ID-CapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

nr-UE-RxTEG-ID-Support-r17			INTEGER (1..maxNumOfRxTEGs-r17)				OPTIONAL, -- 27-1-1 for both DL TDOA and multi-RTT

nr-UE-RxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL,	-- 27-1-1 for both DL TDOA and multi-RTT

nr-UE-TxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL,	-- 27-1-2a for multi-RTT

nr-UE-RxTxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8, n12, n16,

n24, n32, n36, n48, n64}			OPTIONAL,	-- 27-1-3 for multi-RTT

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17		ENUMERATED {n2, n3, n4, n6, n8}			OPTIONAL,	-- 27-1-4 for both DL TDOA and multi-RTT

measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17		ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL,	-- 27-1-4a for both DL TDOA and multi-RTT

}

UL TDOA

-- ASN1START

NR-UL-ProvideCapabilities-r16 ::= SEQUENCE {

nr-UL-SRS-Capability-r16		NR-UL-SRS-Capability-r16,

...

[[

nr-UE-TEG-ID-CapabilityBandList-r17	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

NR-UE-TEG-ID-CapabilityPerBand-r17

]]

}

NR-UE-TEG-ID-CapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

nr-UE-TxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL	-- 27-1-2 for UL TDOA

}

Suggested TS38.331 TP (27-1-2):

BandNR ::=                          SEQUENCE {

--Skip unrelated parts;--

[[

enhancedSkipUplinkTxConfigured-v1660      ENUMERATED {supported}                       OPTIONAL,

enhancedSkipUplinkTxDynamic-v1660         ENUMERATED {supported}                       OPTIONAL

]],

[[

maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16    ENUMERATED {n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}   OPTIONAL,

txDiversity-r16                           ENUMERATED {supported}                       OPTIONAL

]],

[[

nr-UE-TxTEG-ID-MaxSupport-r17             ENUMERATED {n1, n2, n3, n4, n6, n8}          OPTIONAL -- 27-1-2 for UL TDOA

]]

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

Suggested TS38.306 TP (27-1-2):

| nonGroupSINR-reporting-r16 Indicates N_max L1-SINR values reported when UE supports non-group based L1-SINR reporting. UE indicates support of this feature shall indicate support of ssb-csirs-SINR-measurement-r16. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| nr-UE-TxTEG-ID-MaxSupport-r17 Indicates the maximum number of UE-TxTEG for SRS resource for positioning, which is supported and reported by UE for UL TDOA. | Band | No | N/A | N/A |

Discussion point 3.3.1-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Yes in general, but | We suggest to remove the following field from NR-UE-TEG-ID-CapabilityPerBand-r17 for DL-TDOA and Multi-RTT. This field seems duplicated with nr-UE-RxTEG-ID-MaxSupport-r17 nr-UE-RxTEG-ID-Support-r17 INTEGER (1..maxNumOfRxTEGs-r17) OPTIONAL, -- 27-1-1 for both DL TDOA and multi-RTT |

3.3.2 27-2, 27-13, 27-13a, 27-14, 27-14a

| 27. NR_pos_enh | 27-2-1 | DL PRS RSRP measurement report of the first path for UE-assisted DL-AoD | 1.) Support of measuring and reporting the PRS RSRPP of the first path for DL-AoD positioning method 2.) The maximum number of first path PRS RSRPP per TRP | 13-5 or 27-2-2 | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: 2,4,8,16,24 Need for location server to know if the feature is supported The maximum number of first path PRS RSRP per TRP should be less than or equal to the maximum number of PRS RSRP (27-2-2) [Note: Having FG 13-5 as the prerequisite FG does not imply that in a measurement report, reporting PRS-RSRP of a PRS resource should be the prerequisite of reporting PRS-RSRPP for the first path of the PRS resource] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. | 13-5 | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | Per UE | n/a | Yes | n/a | The candidate values are {16, 24} Need for location server to know if the feature is supported The maximum number of reported DL PRS RSRP in the capability signaling should be no less than the maximum number of reported DL PRS RSRPP of the first path in the capability signaling | Optional with capability signaling |

| 27. NR_pos_enh | 27-13 | Additional path reporting for UE-assisted DL-TDOA | 1. Support of additional detected path timing reporting for K>2 additional paths for UE-assisted DL-TDOA 2. Support of RSRPP reporting for additional paths | 13-13a | No |  |  | Per UE | No | No | No | Component 1 candidate values: [{4, 6, 8}] Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-13a | First path reporting for UE-assisted DL-TDOA | 1. Support of RSRPP reporting for first path | 13-1 | No |  |  | FFS: Per UE or per band | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-14 | Additional path reporting for Multi-RTT | 1. Support of additional detected path timing reporting for K>2 additional paths for Multi-RTT 2. Support of RSRPP reporting for additional paths | 13-14a | No |  |  | Per UE | No | No | No | Component 1 candidate values: [{4, 6, 8}] Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-14a | First path reporting for Multi-RTT | 1. Support of RSRPP reporting for first path | 13-1 | No |  |  | FFS: Per UE or per band | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |

In LPP running CR R2-2201723, it was captured as per UE capability

DL AoD, DL-TDOA, Multi-RTT

...,

[[

maxDL-PRS-RSRP-MeasurementFR1-r17			INTEGER (9..24)							OPTIONAL,

maxDL-PRS-RSRP-MeasurementFR2-r17			INTEGER (9..24)							OPTIONAL,

supportOfDL-PRS-FirstPathRSRP-MeasFR1-r17	ENUMERATED { supported }				OPTIONAL,

supportOfDL-PRS-FirstPathRSRP-MeasFR2-r17	ENUMERATED { supported }				OPTIONAL,

supportedDL-PRS-ProcessingSamplesFR1-r17	BIT STRING {

m1 (0)

}	(SIZE(1..8))						OPTIONAL,

supportedDL-PRS-ProcessingSamplesFR2-r17	BIT STRING {

m1 (0)

}	(SIZE(1..8))						OPTIONAL

]]

}

However it cannot match RAN1 feature list very well.

Suggested LPP TP:

DL-AoD

...,

[[

maxDL-PRS-FirstPathRSRP-MeasPerTRP-r17		ENUMERATED { n2, n4, n8, n16, n24 }		OPTIONAL, --27-2-1, FFS per UE or Per band

supportOfDL-PRS-FirstPathRSRP-Meas-r17		ENUMERATED { supported }				OPTIONAL,--27-2-1, FFS per UE or Per band

dl-PRS-FirstPathRSRP-MeasAboveEightPerTRP-r17		ENUMERATED { n16, n24 }		OPTIONAL, --27-2-1, per UE

]]

DL-TDOA

...,

[[

supportOfDL-PRS-AdditionalPathRSRP-MeasAbove2-r17		ENUMERATED { n4, n6, n8 }				OPTIONAL,--27-13

supportOfDL-PRS-AdditionalPathRSRP-Meas-r17		ENUMERATED { supported }				OPTIONAL,--27-13

supportOfDL-PRS-FirstPathRSRP-Meas-r17		ENUMERATED { supported }				OPTIONAL --27-13a, FFS per UE or Per band

]]

Multi-RTT

...,

[[

supportOfDL-PRS-AdditionalPathRSRP-MeasAbove2-r17		ENUMERATED { n4, n6, n8 }				OPTIONAL,--27-14

supportOfDL-PRS-AdditionalPathRSRP-Meas-r17		ENUMERATED { supported }				OPTIONAL,--27-14

supportOfDL-PRS-FirstPathRSRP-Meas-r17		ENUMERATED { supported }				OPTIONAL--27-14a, FFS per UE or Per band

]]

Discussion point 3.3.2-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | See the comments | For DL-AoD: 1. we do not need to have two fields, with one describing support or not while the other describing the number. Keeping only the first field seems sufficient. maxDL-PRS-FirstPathRSRP-MeasPerTRP-r17 supportOfDL-PRS-FirstPathRSRP-Meas-r17 2. dl-PRS-FirstPathRSRP-MeasAboveEightPerTRP-r17 should be dl-PRS-RSRPMeasAboveeightPerTRP-r16, i.e. no “first path” for this field For DL-TDOA and Multi-RTT 1. We only need two field corresponding to two rows supportOfDL-PRS-FirstPathRSRP-Meas-r17 ENUMERATED { supported } supportOfDL-PRS-AdditionalPathMeasAbove2-r17 ENUMERATED { n4, n6, n8 } We do not need a separate capability entry denoting support of additional path RSRPP because it is just a component within a FG. |

3.3.3 27-3/27-6 PPW

| 27. NR_pos_enh | 27-3-1 | M-sample measurements | The capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL PRS resource set | 13-1 | No |  |  | per band | n/a | n/a | n/a | The candidate values are {1 [FFS others]} If the UE does not provide the capability, the UE is assumed to support M=4 only. Need for location server to know if the feature is supported Note: The sample number M=1 does not account for the potential AGC sample Note: this feature is supported for both UE-assisted and UE based positioning | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG and in a PRS processing window - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected (FFS FR2) - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window [The DL signals/channels from all DL CCs (per UE) are affected (FFS FR2)] Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP | 13-1 | Yes |  |  | per band | n/a | n/a | n/a | Component 1 candidate values: [One or more of] {Type 1A, Type 1B, Type 2} Need for location server to know if the feature is supported Note: A UE that supports FG 27-3-2 also needs to support FG 27-3-2a | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-2a | Support of priority handing of PRS when PRS measurement is outside MG | Support of priority handing options of PRS: Option1, Option2 or Option3 - Option 1: UE may indicates support of two priority states. - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 2: UE may indicate support of three priority states - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS - Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: PRS is lower priority than all PDCCH/PDSCH/CSI-RS - Option 3: UE may indicate support of single priority state - State 1: PRS is higher priority than all PDCCH/PDSCH/CSI-RS | [27-3-3] | Yes |  |  | Per band | No | No | No | Candidate values: {option1, option2, option3} Note: A UE that supports FG 27-3-2a also needs to support FG 27-3-2 Note: if the FFS in FG 27-2a gets resolved as “per band’, FG 27-2a will be deleted and becomes a component of FG 27-3-2 | Optional with capability signaling |
| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering [2. Maximum duration of DL PRS symbols N in units of ms a UE can process in the first part of a PRS processing window assuming maximum DL PRS bandwidth in MHz, such that the UE is capable of reporting the measurements T-N ms after the last PRS symbol] 3. Max number of DL PRS resources that UE can process in a slot under it | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} [Candidate 2 component values: a) N: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T: {N+4, N+5, N+6, N+8} ms] Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: A UE may declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2” capabilities in case it supports multiple types in a band | Optional with capability signaling |
| 27. NR_pos_enh | 27-6 | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot under it |  |  |  |  |  |  |  |  | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state [, but instead LMF may set the response time assuming a specific RRC state during the PRS measurement and inform the gNB on the assumed RRC state, while the actual RRC state is still determined by UE/gNB that take the response time requirement and assumed RRC state into account.] | Optional with capability signaling |

In LPP running CR R2-2201723, it was captured for DL AoD, DL TDOA and Multiple RTT separately and as per UE capability

supportedDL-PRS-ProcessingSamplesFR1-r17				BIT STRING {

m1 (0)

}	(SIZE(1..8))			OPTIONAL,

supportedDL-PRS-ProcessingSamplesFR2-r17				BIT STRING {

m1 (0)

However it should be per band capability, and some of features are not captured. Therefore it should be captured as

Suggested LPP TP:

DL AoD, DL TDOA/Multi-RTT, we may introduce common IE, and then add the common IE per positioning method.

nr-DL-PRS-ProcessingCapabilityBandList-r17	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

NR-DL-PRS-ProcessingCapabilityPerBand-r17,

NR-DL-PRS-ProcessingCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

supportedDL-PRS-ProcessingSamples-r17				BIT STRING {

m1 (0)

}	(SIZE(1..8))			OPTIONAL,	--27-3-1

prs-ProcessingWindowType1A-r17		ENUMERATED { supported }			OPTIONAL,	-- 27-3-2

prs-ProcessingWindowType1B-r17		ENUMERATED { supported }			OPTIONAL,	-- 27-3-2

prs-ProcessingWindowType2-r17		ENUMERATED { supported }			OPTIONAL,	-- 27-3-2

supportedPrioHandlingOutOfPPW-r17	ENUMERATED { option1, option2, option3 }			OPTIONAL,	-- 27-3-2a

prs-BufferingCapability-r17			ENUMERATED { type1, type2 }			OPTIONAL,	-- 27-3-3 FFS on component 2

maxDL-PRS-ResourcesProcessInSlot-r17	ENUMERATED { n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }			OPTIONAL	-- 27-3-3 Do not see why FR1/FR2 is needed for per band capability

prs-ProcessingWindowTypeRRC-Inactive-r17		ENUMERATED { type1, type2 }			OPTIONAL,	-- 27-6 FFS on component 2

maxDL-PRS-ResourcesProcessInSlotRRC-Inactive-r17	ENUMERATED { n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }			OPTIONAL	-- 27-6 Do not see why FR1/FR2 is needed for per band capability

}

Suggested TS38.331 TP:

BandNR ::=                          SEQUENCE {

--Skip unrelated parts;--

[[

enhancedSkipUplinkTxConfigured-v1660      ENUMERATED {supported}                       OPTIONAL,

enhancedSkipUplinkTxDynamic-v1660         ENUMERATED {supported}                       OPTIONAL

]],

[[

maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16    ENUMERATED {n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}   OPTIONAL,

txDiversity-r16                           ENUMERATED {supported}                       OPTIONAL

]],

[[

prs-ProcessingWindowType1A-r17            ENUMERATED { supported }                      OPTIONAL, -- 27-3-2

prs-ProcessingWindowType1B-r17            ENUMERATED { supported }                      OPTIONAL, -- 27-3-2

prs-ProcessingWindowType2-r17             ENUMERATED { supported }                      OPTIONAL, -- 27-3-2

supportedPrioHandlingOutOfPPW-r17         ENUMERATED { option1, option2, option3 }      OPTIONAL, -- 27-3-2a

]]

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

Suggested TS38.306 TP :

| powerBoosting-pi2BPSK Indicates whether UE supports power boosting for pi/2 BPSK, when applicable as defined in 6.2 of TS 38.101-1 [2]. This capability is not applicable to IAB-MT. | Band | No | TDD only | FR1 only |
| --- | --- | --- | --- | --- |
| prs-ProcessingWindowType1A-r17 Indicates the UE supports the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR. | Band | No | N/A | N/A |
| prs-ProcessingWindowType1B-r17 Indicates the UE supports the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected (FFS FR2). | Band | No | N/A | N/A |
| prs-ProcessingWindowType2-r17 Indicates the UE supports the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window [The DL signals/channels from all DL CCs (per UE) are affected (FFS FR2)]. | Band | No | N/A | N/A |

| supportCodeWordSoftCombining-r16 Indicates whether UE supports codeword soft combining for FDMSchemeB. UE indicates support of this feature depends on whether the supportFDM-SchemeB-r16 is also supported. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| supportedPrioHandlingOutOfPPW-r17 Indicates the support of priority handing options of PRS when PRS measurement is outside MG. | Band | No | N/A | N/A |

Discussion point 3.3.3-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | See comments | According to the Note 27-3-2a Note: if the FFS in FG 27-2a gets resolved as “per band’, FG 27-2a will be deleted and becomes a component of FG 27-3-2 27-3-2 and 27-3-2a should be combined into a single FG. Then we should have a single field added to BandNR with its subfields being mandatory. In addition, we do not need separate fields for the support of different processing window types. The suggested change is BandNR ::= SEQUENCE { --Skip unrelated parts;-- [[ enhancedSkipUplinkTxConfigured-v1660 ENUMERATED {supported} OPTIONAL, enhancedSkipUplinkTxDynamic-v1660 ENUMERATED {supported} OPTIONAL ]], [[ maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16 ENUMERATED {n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100} OPTIONAL, txDiversity-r16 ENUMERATED {supported} OPTIONAL ]], [[ supportOfPPW-r17 PRS-ProcessingWindow OPTIONAL, -- 27-3-2/2a ]] } PRS-ProcessingWindow ::= SEQUENCE { supportedProcessingType ENUMERATED {Type1A, Type1B, Type2,..}, supportedPrioHandlingOutOfPPW-r17 ENUMERATED { option1, option2, option3,.. } } |

3.3.4 27-4, 27-12

| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | 1. Support reporting LoS/NLoS indicator type to LMF 2. LOS/NLOS indicator granularity | one of 13-5,13-6, or 13-11 | No |  |  | Per UE | n/a | n/a | n/a | [Component 1 candidate values: {hard value, soft value[, both]}] Component 2 candidate values: {trpSpecific, resourceSpecific[, both]} [Note: a single value is reported when both multi-RTT and DL-TDOA are supported] FFS: signalling per method Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity |  | No |  |  | Per UE | No | No | No | [Component 1 candidate values: {softValue, hardValue, both}] Component 2 candidate values: {resourceSpecific, trpSpecific[, both]} Need for location server to know if the feature is supported. | Optional with capability signaling. |

In LPP running CR R2-2201723, it was captured for DL AoD, DL TDOA and Multiple RTT separately and as per UE capability

nr-los-nlos-IndicatorSupport-r17		BIT STRING {	case1 (0),

case2 (1)

}	(SIZE(1..8))				OPTIONAL,

However some of features are not captured. Therefore it should be captured as

Suggested LPP TP:

DL AoD, DL TDOA/Multi-RTT.

nr-LOS-NLOS-IndicatorUE-Based-r17	ENUMERATED { hardvalue, softvalue, both }			OPTIONAL,	-- 27-4-1

nr-LOS-NLOS-IndicatorGranularityUE-Based-r17	ENUMERATED { trpspecific, resourcespecific, both}			OPTIONAL,	-- 27-4-1

nr-LOS-NLOS-IndicatorUE-Assisted-r17	ENUMERATED { hardvalue, softvalue, both }			OPTIONAL,	-- 27-4-1

nr-LOS-NLOS-IndicatorGranularityUE-Assisted-r17	ENUMERATED { trpspecific, resourcespecific, both}			OPTIONAL	-- 27-4-1

Discussion point 3.3.4-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | We understand that RAN1 is still discussing whether “both” should be supported. |

3.3.5 27-7

| 27. NR_pos_enh | 27-7 | Multiple measurement instances which can be included in a single measurement report | Support of mutiple measurement instances which can be included in a single measurement report FFS: 2. Maximum number of measurement instances which can be included in a single measurement report |  |  |  |  |  |  |  |  | FFS: Component 2 candidate values | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Suggested LPP TP:

DL AoD, DL TDOA/Multi-RTT.

multiMeasInSameMeasReport-r17	ENUMERATED { supported }			OPTIONAL,	-- 27-7, FFS on component 2

Discussion point 3.3.5-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSIlicon | Yes | OK to keep it as it is for now. but when the maximum number is determined, same issues as above that we don’t need two fields, one indicating the support another indicating the maximum number |

3.3.6 27-8

| 27. NR_pos_enh | 27-8 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning | 13-1 | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Suggested LPP TP:

DL TDOA

prs-TEG-AssociationUE-BasedSupport-r17	ENUMERATED { supported }			OPTIONAL,	-- 27-8

Discussion point 3.3.6-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSIlicon | Yes |  |

3.3.7 27-9

| 27. NR_pos_enh | 27-9 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band | n/a | n/a FR2 only | n/a | Component 2 candidate values: FFS Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Suggested LPP TP:

nr-DL-PRS-ProcessingCapabilityBandList-r17	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

NR-DL-PRS-ProcessingCapabilityPerBand-r17,

NR-DL-PRS-ProcessingCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

lowerRxBeamSweepingThan8-FR2-r17	ENUMERATED { supported }			OPTIONAL,	-- 27-9

numberOfRxBeamSweepingFactor-r17	ENUMERATED { ffs }			OPTIONAL	-- 27-9, FFS on value

}

Discussion point 3.3.7-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | No | Single field is sufficient. We do not need separate fields each for a component, which is against the common understanding on the relationship between FG and component. |

3.3.8 27-10, 27-10a, 27-11

| 27. NR_pos_enh | 27-10 | Support of UL MAC CE based MG activation request for PRS measurements | 1. Support of using UL MAC CE to request measurement gap for PRS measurements: The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG 2. Support of preconfiguration of MGs in RRC signaling for PRS measurements: Each MG in the preconfiguration is associated with an ID | 27-11 | Yes |  | Using UL MAC CE to indicate measurement gap for PRS measurements to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-10a | Low latency MG activation request for PRS measurements | support of low latency MG activation request for PRS measurements | [27-10] | No |  | Low latency MG activation request for PRS measurements is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-11 | Support of DL MAC CE based MG activation request for PRS measurements | 1. Support of preconfiguration of MGs in RRC signaling for PRS measurements: Each MG in the preconfiguration is associated with an ID 2. Support of using DL MAC CE to activate the MG for PRS measurements: The DL MAC CE for MG activation indicates the ID associated with the preconfigured MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG for PRS measurements is not supported | Per UE | No | No | No |  | Optional with capability signaling. |

Suggested LPP TP:

DL AOD, DL TDOA/Multi-RTT

mg-ActivationRequest-r17			ENUMERATED { supported }			OPTIONAL	-- 27-10a

Suggested TS38.331 TP :

MAC-ParametersCommon ::=    SEQUENCE {

lcp-Restriction                         ENUMERATED {supported}      OPTIONAL,

dummy                                   ENUMERATED {supported}      OPTIONAL,

lch-ToSCellRestriction                  ENUMERATED {supported}      OPTIONAL,

...,

[[

recommendedBitRate                      ENUMERATED {supported}      OPTIONAL,

recommendedBitRateQuery                 ENUMERATED {supported}      OPTIONAL

]],

[[

recommendedBitRateMultiplier-r16         ENUMERATED {supported}     OPTIONAL,

preEmptiveBSR-r16                        ENUMERATED {supported}     OPTIONAL,

autonomousTransmission-r16               ENUMERATED {supported}     OPTIONAL,

lch-PriorityBasedPrioritization-r16      ENUMERATED {supported}     OPTIONAL,

lch-ToConfiguredGrantMapping-r16         ENUMERATED {supported}     OPTIONAL,

lch-ToGrantPriorityRestriction-r16       ENUMERATED {supported}     OPTIONAL,

singlePHR-P-r16                          ENUMERATED {supported}     OPTIONAL,

ul-LBT-FailureDetectionRecovery-r16      ENUMERATED {supported}     OPTIONAL,

-- R4 8-1: MPE

tdd-MPE-P-MPR-Reporting-r16              ENUMERATED {supported}     OPTIONAL,

lcid-ExtensionIAB-r16                    ENUMERATED {supported}     OPTIONAL

]],

[[

spCell-BFR-CBRA-r16                      ENUMERATED {supported}     OPTIONAL

]],

[[

srs-ResourceId-Ext-r16                   ENUMERATED {supported}     OPTIONAL

]],

[[

mg-ActivationRequest-r17                 ENUMERATED {supported}      OPTIONAL, --27-10

mg-Activation-r17                        ENUMERATED {supported}      OPTIONAL, --27-11

]]

}

Suggested TS38.306 TP

| longDRX-Cycle Indicates whether UE supports long DRX cycle as specified in TS 38.321 [8]. | UE | Yes | Yes | No |
| --- | --- | --- | --- | --- |
| mg-Activation-r17 Indicates the support of using DL MAC CE to activate the preconfigured MG for PRS measurements: . | UE | No | Yes | No |
| mg-ActivationRequest-r17 Indicates the support of using UL MAC CE to request the activation of the preconfigured MG for PRS measurements: . | UE | No | Yes | No |

Discussion point 3.3.8-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Yes in general, but | We suggest the rename the field to highlight the MG activation request and command corresponds to PRS measurement. |

3.3.9 27-20, 27-21, 27-22

| 27. NR_pos_enh | 27-20 | PRS subset association for UE assisted DL-AoD | 1. Support of assistance data enhancement to indicate a subset of PRS resources for each PRS resource for the purpose of prioritization of DL-AoD reporting. [2. Supported resource set relationship for the target PRS resource and the associated subset [3. Support associated subset measurement reporting] |  | No |  | PRS subset association for DL-AoD is not supported by the UE. | Per UE | n/a | n/a | n/a | [Component 2 candidate values: {sameSet, DifferentSet, sameOrDifferentSet}] [Component 3 candidate values: {associated subset only, the target PRS resource and the associated subset}] Need for location server to know | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-21 | PRS boresight direction for UE-assisted DL-AoD | Support of assistance data enhancement to indicate the boresight direction of a PRS resource for UE-assisted DL-AoD. |  | No |  | UE-assisted DL-AoD with boresight direction of each DL-PRS is not supported. | Per UE | n/a | n/a | n/a | Need for location server to know | Optional with capability signaling. |
| 27. NR_pos_enh | 27-22 | PRS beam pattern for UE-based DL-AoD | Support of PRS beam pattern for DL-AoD |  | No |  | UE-based DL-AoD with PRS beam pattern is not supported. | Per UE | n/a | n/a | n/a | Need for location server to know | Optional with capability signaling. |

In LPP running CR R2-2201723, it was captured as

nr-DL-PRS-BeamInfoSup-r17				ENUMERATED { supported }					OPTIONAL, -- 27-22

nr-DL-PRS-ResourcePriorityListSup-r17	ENUMERATED { supported }					OPTIONAL, --27-20 FFS on component 2 and 3

Seems 27-21 is missing.

Suggested LPP TP:

DL AOD

nr-DL-PRS-BoresightInfoSup-r17				ENUMERATED { supported }					OPTIONAL, -- 27-21

nr-DL-PRS-BeamInfoSup-r17				ENUMERATED { supported }					OPTIONAL, -- 27-22

nr-DL-PRS-ResourcePriorityListSup-r17	ENUMERATED { supported }					OPTIONAL, --27-20 FFS on component 2 and 3

Discussion point 3.3.9-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

3.3.10 27-15---27-19

| 27. NR_pos_enh | 27-15 | Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP | 1. Max number of SRS Resource Sets for positioning supported by UE 2. Max number of [P/SP]SRS Resources for positioning 3. Max number of [P/SP]SRS Resources for positioning per slot 4. Max number of periodic SRS Resources for positioning 5. Max number of periodic SRS Resources for positioning per slot Note: OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. No dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 12, 16} Component 2 candidate values: {1,2,4,8,16,32,64} Component 3 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Component 4 candidate values: {1,2,4,8,16,32,64} Component 5 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} [Need for location server to know if the feature is supported] FFS: outside initial BWP | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-15a | Support of positioning SRS transmission in RRC_INACTIVE state for initial BWP with semi-persistent SRS | 1. Max number of semi-persistent SRS Resources for positioning 2. Max number of semi-persistent SRS Resources for positioning per slot | 27-15 | Yes |  |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {1,2,4,8,16,32,64} Component 2 candidate values: {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} [Need for location server to know if the feature is supported] FFS: outside initial BWP | Optional with capability signaling |
| 27. NR_pos_enh | 27-16 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-17 | Support of positioning in RRC_INACTIVE state | Support of PRS processing in RRC_INACTIVE | [13-1, 13-2, 13-3, 13-4] | FFS |  |  | FFS | FFS | FFS | FFS | [Need for location server to know if the feature is supported.] FFS: separate UE capability for location information reporting in RRC_INACTIVE state using SDT Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference measurement | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA |  | FFS |  |  | FFS | FFS | FFS | FFS | [Need for location server to know if the feature is supported.] Note: Applicable for both UE-assisted and UE-based DL-TDOA Note: PRS capabilities for DL-TDOA measurement and reporting described in FGs in 13-3, 13-3a, 13-3b, 13-6, 13-13 are the same for RRC Inactive. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD |  | FFS |  |  | FFS | FFS | FFS | FFS | [Need for location server to know if the feature is supported.] Note: Applicable for both UE-assisted and UE-based DL-AoD Note: PRS capabilities for DL-AOD measurement and reporting described in FGs 13-2, 13-2a, 13-2b, 13-5, 13-13 are the same for RRC Inactive. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-18c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT [2. Support of positioning SRS transmission in RRC_INACTIVE state] |  | FFS |  |  | FFS | FFS | FFS | FFS | [Need for location server to know if the feature is supported.] Note: PRS capabilities for Multi-RTT measurement and reporting described in FGs in 13-4, 13-4a, 13-4b, 13-11, 13-11a, 13-14 are the same for RRC Inactive | Optional with capability signaling. |
| 27. NR_pos_enh | 27-19 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |

The content of 27-15, 27-15a, 27-16, 27-19 is same as Rel-16 RRC capability. But in Rel-16, 27-15, 27-15a was per FSPC instead of per band.

Suggested LPP TP:

SRS-CapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16			FreqBandIndicatorNR-r16,

olpc-SRS-Pos-r16				OLPC-SRS-Pos-r16									OPTIONAL,

spatialRelationsSRS-Pos-r16		SpatialRelationsSRS-Pos-r16							OPTIONAL,

...,

[[

olpc-SRS-PosRRC-Inactive-r17	OLPC-SRS-Pos-r16									OPTIONAL, --27-16

spatialRelationsSRS-Pos-r16		SpatialRelationsSRS-Pos-r16							OPTIONAL  --27-19

]]

}

DL AOD, DL TDOA, Multi-RTT

nr-DL-PRS-MeasRRC-Inactive-r17			ENUMERATED { supported }					OPTIONAL, -- 27-18a, 27-18b, 27-18c,

nr-DL-PRS-ProcessingRRC-Inactive-r17			ENUMERATED { supported }			OPTIONAL, -- 27-17

Suggested TS38.331 TP:

BandNR ::=                          SEQUENCE {

--Skip unrelated parts;--

[[

enhancedSkipUplinkTxConfigured-v1660      ENUMERATED {supported}                       OPTIONAL,

enhancedSkipUplinkTxDynamic-v1660         ENUMERATED {supported}                       OPTIONAL

]],

[[

maxUplinkDutyCycle-PC1dot5-MPE-FR1-r16    ENUMERATED {n10, n15, n20, n25, n30, n40, n50, n60, n70, n80, n90, n100}   OPTIONAL,

txDiversity-r16                           ENUMERATED {supported}                       OPTIONAL

]],

[[

srs-AllPosResourcesRRC-Inactive-r17       SRS-AllPosResources-r16                       OPTIONAL, -- 27-15, 27-15a, FFS on LPP capability

olpc-SRS-PosRRC-Inactive-r16              OLPC-SRS-Pos-r16                        OPTIONAL, -- 27-16

spatialRelationsSRS-PosRRC-Inactive-r177  SpatialRelationsSRS-Pos-r16             OPTIONAL --27-19

]]

}

-- TAG-RF-PARAMETERS-STOP

-- ASN1STOP

Suggested TS38.306 TP :

| spatialRelationsSRS-Pos-r16 Indicates whether the UE supports spatial relations for SRS for positioning. The capability signalling comprises the following parameters. - spatialRelation-SRS-PosBasedOnSSB-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the serving cell in the same band. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnPRS-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports any of DL PRS Resources for DL AoD, DL PRS Resources for DL-TDOA or DL PRS Resources for Multi-RTT defined in TS37.355 [22], or srs-PosResources-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnSRS-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SRS in the same band. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnSSB-Neigh-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnPRS-Neigh-r16 indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnPRS-Serving-r16. Otherwise, the UE does not include this field; NOTE: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. | Band | No | N/A | FR2 only |
| --- | --- | --- | --- | --- |
| spatialRelationsSRS-PosRRC-Inactive-r17 Indicates whether the UE supports spatial relations for SRS for positioning in RRC_INACTIVE. The capability signalling comprises the following parameters. - spatialRelation-SRS-PosBasedOnSSB-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the serving cell in the same band. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnPRS-Serving-r16 indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports any of DL PRS Resources for DL AoD, DL PRS Resources for DL-TDOA or DL PRS Resources for Multi-RTT defined in TS37.355 [22], or srs-PosResources-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnSRS-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SRS in the same band. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnSSB-Neigh-r16 indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving-r16. Otherwise, the UE does not include this field; - spatialRelation-SRS-PosBasedOnPRS-Neigh-r16 indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnPRS-Serving-r16. Otherwise, the UE does not include this field; NOTE: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. | Band | No | N/A | FR2 only |

| srs-AssocCSI-RS Parameters for the calculation of the precoder for SRS transmission based on channel measurements using associated NZP CSI-RS resource (srs-AssocCSI-RS) as described in clause 6.1.1.2 of TS 38.214 [12]. UE supporting this feature shall also indicate support of non-codebook based PUSCH transmission. This capability signalling includes list of the following parameters: - maxNumberTxPortsPerResource indicates the maximum number of Tx ports in a resource; - maxNumberResourcesPerBand indicates the maximum number of resources across all CCs within a band simultaneously; - totalNumberTxPortsPerBand indicates the total number of Tx ports across all CCs within a band simultaneously. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| srs-PosResources-r16 Indicates support of SRS for positioning in RRC_INACTIVE. UE supporting this feature should also support open loop power control for positioning SRS based on SSB from the serving cell. The capability signalling comprises the following parameters: - maxNumberSRS-PosResourceSetPerBWP-r16 Indicates the max number of SRS Resource Sets for positioning supported by UE per BWP; - maxNumberSRS-PosResourcesPerBWP-r16 indicates the max number of SRS resources for positioning supported by UE per BWP, including periodic, semi-persistent, and aperiodic SRS; - maxNumberSRS-ResourcesPerBWP-PerSlot-r16 indicates the max number of SRS resources configured by SRS-Resource and SRS-PosResource-r16 supported by UE per BWP, including periodic, semi-persistent, and aperiodic SRS; - maxNumberPeriodicSRS-PosResourcesPerBWP-r16 indicates the max number of periodic SRS resources for positioning supported by UE per BWP; - maxNumberPeriodicSRS-PosResourcesPerBWP-PerSlot-r16 indicates the max number of periodic SRS resources for positioning supported by UE per BWP per slot. | Band | No | N/A | N/A |
| srs-PosResourceAP-r16 Indicates support of aperiodic SRS for positioning in RRC_INACTIVE. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maxNumberAP-SRS-PosResourcesPerBWP-r16 indicates the max number of aperiodic SRS resources for positioning supported by UE per BWP; - maxNumberAP-SRS-PosResourcesPerBWP-PerSlot-r16 indicates the max number of aperiodic SRS resources for positioning supported by UE per BWP per slot. | Band | No | N/A | N/A |
| srs-PosResourceSP-r16 Indicates support of semi-persistent SRS for positioning in RRC_INACTIVE. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field. The capability signalling comprises the following parameters: - maxNumberSP-SRS-PosResourcesPerBWP-r16 indicates the max number of semi-persistent SRS resources for positioning supported by UE per BWP; - maxNumberSP-SRS-PosResourcesPerBWP-PerSlot-r16 indicates the max number of semi-persistent SRS resources for positioning supported by UE per BWP per slot | Band | No | N/A | N/A |

| olpc-SRS-Pos-r16 Indicates whether the UE supports OLPC for SRS for positioning. The capability signalling comprises the following parameters. - olpc-SRS-PosBasedOnPRS-Serving-r16 indicates whether the UE supports OLPC for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports NR-DL-PRS-ProcessingCapability-r16 defined in TS 37.355 [22], and srs-PosResources-r16. Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnSSB-Neigh-r16 indicates whether the UE supports OLPC for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnPRS-Neigh-r16 indicates whether the UE supports OLPC for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports olpc-SRS-PosBasedOnPRS-Serving-r16. Otherwise, the UE does not include this field; NOTE: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. - maxNumberPathLossEstimatePerServing-r16 indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissios. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving-r16, olpc-SRS-PosBasedOnSSB-Neigh-r16 and olpc-SRS-PosBasedOnPRS-Neigh-r16. Otherwise, the UE does not include this field. | Band | No | N/A | N/A |
| --- | --- | --- | --- | --- |
| olpc-SRS-PosRRC-Inactive-r17 Indicates whether the UE supports OLPC for SRS for positioning in RRC_INACTIVE. The capability signalling comprises the following parameters. - olpc-SRS-PosBasedOnPRS-Serving-r16 indicates whether the UE supports OLPC for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports NR-DL-PRS-ProcessingCapability-r16 defined in TS 37.355 [22], and srs-PosResources-r16. Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnSSB-Neigh-r16 indicates whether the UE supports OLPC for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports srs-PosResources-r16. Otherwise, the UE does not include this field; - olpc-SRS-PosBasedOnPRS-Neigh-r16 indicates whether the UE supports OLPC for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports olpc-SRS-PosBasedOnPRS-Serving-r16. Otherwise, the UE does not include this field; NOTE: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. - maxNumberPathLossEstimatePerServing-r16 indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissios. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving-r16, olpc-SRS-PosBasedOnSSB-Neigh-r16 and olpc-SRS-PosBasedOnPRS-Neigh-r16. Otherwise, the UE does not include this field. | Band | No | N/A | N/A |

Discussion point 3.3.10-1: do companies agree the Suggested TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Yes in general, but | We do not think there should be the capability signaling to indicate support of AP-SRS in RRC_INACTIVE state. Also no need to capture 27-17 in LPP. |

3.4 RAN4 feature lists

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14. NR_pos_enh | 14-1 | per-FR MG for PRS measurement | Capability of supporting per-FR MG for PRS measurement | Rel-15 per-FR gap (independentGapConfig) | yes | no |  | Per UE | No | No | N/A |  | Optional with capability signalling |

TS38.331 TP:

MeasAndMobParametersCommon ::=          SEQUENCE {

supportedGapPattern                     BIT STRING (SIZE (22))                  OPTIONAL,

ssb-RLM                                 ENUMERATED {supported}                  OPTIONAL,

ssb-AndCSI-RS-RLM                       ENUMERATED {supported}                  OPTIONAL,

...,

[[

eventB-MeasAndReport                    ENUMERATED {supported}                  OPTIONAL,

handoverFDD-TDD                         ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting                     ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting                        ENUMERATED {supported}                  OPTIONAL

]],

[[

independentGapConfig                    ENUMERATED {supported}                  OPTIONAL,

periodicEUTRA-MeasAndReport             ENUMERATED {supported}                  OPTIONAL,

handoverFR1-FR2                         ENUMERATED {supported}                  OPTIONAL,

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

]],

[[

nr-CGI-Reporting-ENDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

eutra-CGI-Reporting-NEDC                ENUMERATED {supported}                  OPTIONAL,

eutra-CGI-Reporting-NRDC                ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NEDC                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NRDC                   ENUMERATED {supported}                  OPTIONAL

]],

[[

reportAddNeighMeasForPeriodic-r16       ENUMERATED {supported}                  OPTIONAL,

condHandoverParametersCommon-r16        SEQUENCE {

condHandoverFDD-TDD-r16                  ENUMERATED {supported}              OPTIONAL,

condHandoverFR1-FR2-r16                  ENUMERATED {supported}              OPTIONAL

}                                                                               OPTIONAL,

nr-NeedForGap-Reporting-r16             ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-NRonly-r16          BIT STRING (SIZE (10))                  OPTIONAL,

supportedGapPattern-NRonly-NEDC-r16     ENUMERATED {supported}                  OPTIONAL,

maxNumberCLI-RSSI-r16                   ENUMERATED {n8, n16, n32, n64}          OPTIONAL,

maxNumberCLI-SRS-RSRP-r16               ENUMERATED {n4, n8, n16, n32}           OPTIONAL,

maxNumberPerSlotCLI-SRS-RSRP-r16        ENUMERATED {n2, n4, n8}                 OPTIONAL,

mfbi-IAB-r16                            ENUMERATED {supported}                  OPTIONAL,

dummy                                   ENUMERATED {supported}                  OPTIONAL,

nr-CGI-Reporting-NPN-r16                ENUMERATED {supported}                  OPTIONAL,

idleInactiveEUTRA-MeasReport-r16        ENUMERATED {supported}                  OPTIONAL,

idleInactive-ValidityArea-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-r16                ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NEDC-r16           ENUMERATED {supported}                  OPTIONAL,

eutra-AutonomousGaps-NRDC-r16           ENUMERATED {supported}                  OPTIONAL,

pcellT312-r16                           ENUMERATED {supported}                  OPTIONAL,

supportedGapPattern-r16                 BIT STRING (SIZE (2))                   OPTIONAL

]],

[[

independentGapConfigPRS-r17             ENUMERATED {supported}                  OPTIONAL

]]

}

TS38.306 TP:

| independentGapConfig This field indicates whether the UE supports two independent measurement gap configurations for FR1 and FR2 specified in clause 9.1.2 of TS 38.133 [5]. The field also indicates whether the UE supports the FR2 inter-RAT measurement without gaps when (NG)EN-DC is not configured. | UE | No | No | No |
| --- | --- | --- | --- | --- |
| independentGapConfigPRS-r17 This field indicates for PRS measurement whether the UE supports two independent measurement gap configurations for FR1 and FR2 specified in clause 9.1.2 of TS 38.133 [5]. | UE | No | No | No |

Discussion point 3.4-1: For 14-1, do companies agree the TPs shown as above?

| Company’s name | Yes/No | Comments, if any |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

Summary:

- Summary report and proposals

Easy agreements:

Online discussion:

Postpone:

- Open issues list for Positioning UE capabilities (R2-2202005)

Table 3.7: open issue lists for UE positioning capability

| Topic | Open issues Note: Open Issues should be defined for aspects that need to be closed, important to make already agreed functionality work in a reasonable way. Not yet agreed optimizations that may not be needed shall not be listed as Open Issues. | Related to the completion of WI? The topic has to be removed from Rel-17 scope if the corresponding open issues cannot be resolved. | Remark RAN1 provided updated UE feature list in R1-2200767; some are still open. | Status |
| --- | --- | --- | --- | --- |
| Latency reduction | Scheduled location time UE capability |  | Status: draft in LPP running CR, check the status of LPP email discussion 116bis-628 RAN2#116bis: - Include the capability to support scheduled location in each method-ProvideCapabilities message, where 'method' can be any of the LPP positioning methods. The capability should indicate the time base(s) supported for scheduling location measurements. - Pre117-e607 - Question2: Do comapies agree that it is necessary for the UE capability reporting for positioning methods that support multiple positioning modes to differentiate its UE capability of time based for different positioning modes? |  |
|  | Preconfigured AD UE capability/configuration limitation FFS the maximum number of preconfigured assistance data instances; | Yes | Status: check the status of LPP email discussion 116bis-628 RAN2#116bis: Proposal 3.2.1.3-1 (modified): [Easy agreements] [10/10] Include the capability to support validity area in each method ProvideCapabilities message, where “method” can be any of the LPP positioning methods that rely on DL-PRS. FFS on other validity criteria. Pre117-e607 Question6: Do companies agree that multiple AD instances can already be supported by the current LPP spec? |  |
|  | UE capabilities for MG enhancements | Yes | Status: check the status of RAN1 feature list RAN2 also needs to discuss how to capture UE capability based on RAN1 feature list R1-2111810R1-2200767 RRC:27-10, 27-11 LPP:27-10a, | Pre117-e612 based on RAN1 feature list |
|  | UE capabilities for PPW enhancements | Yes | Status: check the status of RAN1 feature list RAN2 also needs to discuss how to capture UE capability based on RAN1 feature list R1-2111810R1-2200767 RRC: 27-3-2, LPP: 27-3-3 | Pre117-e612 based on RAN1 feature list |
| On-Demand PRS | UE capability on On-Demand PRS FFS on per positioning method | Yes | Pre117-e608 Q16 Do companies agree that the LMF may request UE-initiated on-demand PRS capability per positioning method, while the UE may similarly respond on its UE-initiated on-demand PRS capability per positioning method? Q17 Companies are invited to provide their views on the following UE behaviour related to the reception of the on-demand PRS configuration index and whether it has an impact on the UE-initiated on-demand PRS capability: - The UE may store a number of pre-defined on-demand PRS configurations until it is overridden by a new index of on-demand PRS configurations. - The number of pre-defined on-demand PRS configurations that a UE may store has an impact on the UE’s capability. |  |
| RRC_INACTIVE | UE capabilities on positioning in RRC_INACTIVE in RAN1 feature lists 27-6 DL PRS processing capabilities in RRC inactive state 27-15 Support of positioning SRS transmission in RRC_INACTIVE state [for initial BWP] 27-16 OLPC for positioning SRS in RRC_INACTIVE state 27-17 Support of [PRS measurement in RRC_INACTIVE] 27-18a Support of PRS measurement in RRC_INACTIVE state for DL-TDOA 27-18b Support of PRS measurement in RRC_INACTIVE state for DL-AoD 27-18c Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 27-19 Spatial relation for positioning SRS in RRC_INACTIVE state | Yes | Status: check the status of RAN1 feature list and the discussion in R2-2201767; Follow RAN2 agreements “RRC state is transparent to LMF and no different handling on PRS for different RRC state”, RAN2 should avoid to optimize these aspects even if RAN1 agrees to introduce RRC_INACTIVE specific LPP capabilities (27-6, 27-16, 27-17, 27-18a, 27-18b, 27-18c, 27-19). RAN1 feature lists in R1-2200767; FFS on LPP: 27-17, 27-18a, 27-18b, 27-18c FFS on RRC: 27-17, 27-18a, 27-18b, 27-18c LPP: 27-6 Note from RAN1 on 27-6: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state [, but instead LMF may set the response time assuming a specific RRC state during the PRS measurement and inform the gNB on the assumed RRC state, while the actual RRC state is still determined by UE/gNB that take the response time requirement and assumed RRC state into account.] | Pre117-e612 based on RAN1 feature list |
|  | UL capability Wait for RAN1 decision on whether UL related RRC_INACTIVE specific capabilities (27-15, 27-16, 27-19) should be captured in RRC or LPP. | Yes | Status: check the status of RAN1 feature list and the discussion in R2-2201767; RAN1 feature lists in R1-2200767; RAN1 has agreed: RRC: 27-15, 27-15a, FFS on LPP: 27-15, 27-15a, | Pre117-e612 based on RAN1 feature list |
| GNSS Integrity | GNSS Integrity capability | Yes | Status: see the discussion in R2-2201767 Companies would like to wait for the outcome from GNSS integrity discussion. | Pre117-e612 Discussion point 3.2.2-1: For GNSS integrity capability, do you agree capabilities captured in the running LPP CR R2-2201723? |
| RAN1 Led Item-Accuracy | Accuracy improvements-PRU | Yes | Status RAN2#116bis RAN2 will not discuss PRUs further without further guidance from RAN1 (LS or feature list). RAN1 did not provide capability on this in RAN1 feature list R1-2200767 |  |
|  | UE capability for Enhancements of information reporting from UE and gNB for multipath/NLOS mitigation | Yes | Status: check the status of LPP email discussion 116bis-628, check the status of RAN1 feature list. Check RAN1 feature list R1-2200767; | Pre117-e612 based on RAN1 feature list |
|  | UE capability for Accuracy improvements by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays | Yes | Status: Discussion see R2-2201768. check the status of LPP email discussion 116bis-628, check the status of RRC email discussion 116bis-631 Check RAN1 feature list R1-2200767; RRC: 27-1-2 | Pre117-e612 based on RAN1 feature list |
|  | UE capability for Accuracy improvements for DL-AoD positioning solutions | Yes | Status: Discussion see R2-2201768. check the status of LPP email discussion 116bis-628; Check RAN1 feature list R1-2200767; | Pre117-e612 based on RAN1 feature list |
| RAN4 feature list (Not listed in R2-2202005) | 14-1 per-FR MG for PRS measurement Capability of supporting per-FR MG for PRS measurement |  | Need to be captured; | Pre117-e612 based on RAN4 feature list |

- Open issues list for Positioning UE capabilities (R2-2201722)

| R2-A1 | UE capabilities | Capabilities may need corrections based on RAN1/RAN4 input. | ProvideCapabilities | Rapporteur |
| --- | --- | --- | --- | --- |

| R2-B4 | Capability for scheduled location request | Differentiation between UE-based and UE-assisted support and indication of time bases supported. | OTDOA-ProvideCapabilities-->scheduledLocationRequest-r17 A-GNSS-ProvideCapabilities-->scheduledLocationRequest-r17 ECID-ProvideCapabilities-->scheduledLocationRequest-r17 TBS-ProvideCapabilities-r13-->scheduledLocationRequest-r17 Sensor-ProvideCapabilities-r13-->scheduledLocationRequest-r17 WLAN-ProvideCapabilities-r13-->scheduledLocationRequest-r17 BT-ProvideCapabilities-r13-->scheduledLocationRequest-r17 NR-ECID-ProvideCapabilities-r16-->scheduledLocationRequest-r17 NR-DL-TDOA-ProvideCapabilities-r16-->scheduledLocationRequest-r17 NR-DL-AoD-ProvideCapabilities-r16-->scheduledLocationRequest-r17 NR-Multi-RTT-ProvideCapabilities-r16-->scheduledLocationRequest-r17 | Huawei, vivo, Nokia Pre117-e607 - Question2: Do comapies agree that it is necessary for the UE capability reporting for positioning methods that support multiple positioning modes to differentiate its UE capability of time based for different positioning modes? |  |
| --- | --- | --- | --- | --- | --- |

| R1-7 | Capability for 10ms Response Time | Do we need a capability for all methods? | ResponseTime --> unit-r15 --> ten-milli-seconds-r17 | Huawei(110) vivo(110) | Pre117-e612 Discussion point 3.2.1-1: For the finer granularity, which option do you prefer? Option 1 Finer granularity is only applied for NR RAT dependent positioning methods; Option 2 Finer granularity is only applied for NR RAT dependent positioning methods and RAT independent positioning methods; Option 3 Finer granularity is applied for LTE and NR RAT dependent positioning methods and RAT independent positioning methods; |
| --- | --- | --- | --- | --- | --- |

- Reference

- R1-2200780 Rel17 RAN1 UE feature List

- R4-2202400 (R4 feature list)

- R2-2202005 Summary of postmeeting 634

- R2-2201722_([Post116bis-e][628][POS]37.355)_summary

- R2-2201723 LPP running CR